| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m OFDMA Parameter – SDD Text Proposal |  |
| Date Submitted | 2008-11-03 |  |
| Source(s) | Yih-Guang Jan, Yang-Han Lee, Hsien-Wei Tseng, Ming-Hsueh Chuang Tamkang University (TKU) Youn-Tai Lee, Chun-Yen Hsu, Jiun-Je Jian Institute for Information Industry (III) Shiann-Tsong Sheu National Central University (NCU) Whai-En Chen National Ilan University (NIU) Shih-Lin Wu Chang Gung University (CGU) | yihjan@yahoo.com lyt@nmi.iii.org.tw stsheu@ce.ncu.edu.tw wechen@niu.edu.tw slwu@mail.cgu.edu.tw |
| Re: | 02.16m-08/040, “Call for Contributions and Comments on Project 802.16m System Description Document (SDD)” - TGm SDD: Other |  |
| Abstract | IEEE 802.16m OFDMA Parameter – SDD Text Proposal |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m OFDMA Parameter – SDD Text Proposal

Yih-Guang Jan, Yang-Han Lee, Hsien-Wei Tseng, Ming-Hsueh Chuang

Tamkang University (TKU)

- Youn-Tai Lee, Chun-Yen Hsu, Jiun-Je Jian

- Institute for Information Industry (III)

Shiann-Tsong Sheu

- National Central University (NCU)

- Whai-En Chen

- National Ilan University (NIU)

- Shih-Lin Wu

Chang Gung University (CGU)

- Introduction

This contribution provides the modification and insertion of SDD text proposal to OFDMA parameters. The added parameter description is referenced from standard IEEE Draft P802.16Rev2 and substitutes the words in the OFDMA parameters table with symbols. And remove the contents in the columns of 7 and 8.75 MHz from the table due to it is not presented or described in the SDD description.

- Proposed SDD Text Modification

[In 80216m-08_003r5, Section 11.3, insert the section 11.3.1 “Primitive parameter definitions” and the section 11.3.2 “Derived parameter definitions” and modify the current content of the “OFDMA Parameter” as in the following text]

===================Start of Proposed Text====================

11.3 OFDMA Parameters

    - Primitive parameter definitions

Five primitive parameters characterize the OFDM symbol:

- BW: This is the nominal channel bandwidth.

- n: Over sampling factor.

- NFFT: Number of FFT size.

- G: This is the ratio of Cyclic Prefix (CP) time to useful symbol time.

- NOFDM: Number of OFDM symbols per frame.

    - Derived parameter definitions

The following parameters are defined in terms of the primitive parameters of 11.3.1.

- Fs: Sampling frequency = BW × n

- Δf: Subcarrier spacing = Fs / NFFT

- Tu: Useful symbol time = 1/Δf

- Tg: Cyclic Prefix (CP) time = G × Tu  (G = 1/8, 1/16)

- Ts: OFDM symbol time = Tu + Tg

- Ti: Idle time per frame= 5(ms) – NOFDM × Ts

The OFDMA parameters for the IEEE 802.16m are specified as follows:

| Nominal Channel Bandwidth BW (MHz) |  | 5 | 7 | 8.75 | 10 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| Over-sampling Factor n |  | 28/25 | 8/7 | 8/7 | 28/25 | 28/25 |
| Sampling Frequency Fs (MHz) |  | 5.6 | 8 | 10 | 11.2 | 22.4 |
| FFT Size NFFT |  | 512 | 1024 | 1024 | 1024 | 2048 |
| Sub-Carrier Spacing Δf (kHz) |  | 10.937500 | 7.812500 | 9.765625 | 10.937500 | 10.937500 |
| Useful Symbol Time Tu (µs) |  | 91.429 | 128 | 102.4 | 91.429 | 91.429 |
| Cyclic Prefix Tg=1/8Tu G=1/8 | Symbol Time Ts (µs) Ts (µs) | 102.857 | 144 | 115.2 | 102.857 | 102.857 |
|  | Number of OFDM symbols per frame NOFDM | 48 | 34 | 43 | 48 | 48 |
|  | Ideal time (us) Ti (µs) | 62.86 | 104 | 46.40 | 62.86 | 62.86 |
| Cyclic Prefix Tg=1/16Tu G=1/16 | Symbol Time Ts (µs) Ts (µs) | 97.143 |  |  | 97.143 | 97.143 |
|  | Number of OFDM symbols per frame NOFDM | 51 |  |  | 51 | 51 |
|  | Ideal time (us) Ti (µs) | 45.71 |  |  | 45.71 | 45.71 |

Table 1 OFDMA parameters for IEEE 802.16m

A CP size longer than 1/8 is used in channels with long delay spread.

====================End of Proposed Text====================