| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m Open-loop SU-MIMO – SDD Text Proposal |  |
| Date Submitted | 2008-11-03 |  |
| Source(s) | Yih-Guang Jan, Yang-Han Lee, Hsien-Wei Tseng, Ming-Hsueh Chuang Tamkang University (TKU) Youn-Tai Lee, Chun-Yen Hsu Institute for Information Industry (III) Shiann-Tsong Sheu National Central University (NCU) Whai-En Chen National Ilan University (NIU) Shih-Lin Wu Chang Gung University (CGU) | yihjan@yahoo.com lyt@nmi.iii.org.tw stsheu@ce.ncu.edu.tw wechen@niu.edu.tw slwu@mail.cgu.edu.tw |
| Re: | 02.16m-08/040, “Call for Contributions and Comments on Project 802.16m System Description Document (SDD)” - TGm SDD: Other |  |
| Abstract | IEEE 802.16m Open-loop SU-MIMO – SDD Text Proposal |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m Open-loop SU-MIMO – SDD Text Proposal

Yih-Guang Jan, Yang-Han Lee, Hsien-Wei Tseng, Ming-Hsueh Chuang

Tamkang University (TKU)

- Youn-Tai Lee, Chun-Yen Hsu

- Institute for Information Industry (III)

Shiann-Tsong Sheu

- National Central University (NCU)

- Whai-En Chen

- National Ilan University (NIU)

- Shih-Lin Wu

Chang Gung University (CGU)

- Introduction

This contribution provides modification and insertion of the SDD text proposal to Open-loop SU-MIMO. The contents in the original SDD description are modified and some missing matrices are added.

- Proposed SDD Text Modification

[In 80216m-08_003r5, Section 11.8.2.1.1, modify and insertion the following text]

===================Start of Proposed Text====================

11.8.2.1.1 Open-loop SU-MIMO

After line 20, page 79:

On a given subcarrier k, the precoding matrix P can be defined using the following equation:

P(k) = D(k) W(k), equation 4

The procedure is composed of two matrices. The first matrix W (k) is anNT×NS matrix, whereNTis the number of transmit antennas andNSis the number of streams. The matrix W (k) is selected from a predefined unitary codebook, and changes every u subcarriers, and/or ν OFDM symbols. A codebook is a unitary codebook if each of its matrices consists of columns of a unitary matrix. [The detailed unitary codebook, and the parameters u and ν are FFS.]

The second matrix D(k) is an NT x NT diagonal matrix defined as follows:

D(k)=ejθ0k0.00ejθ1k000ejθ(NT−1)k, equation 5

where k denotes frequency resource index andθi, i = 0, 1, 2, …NT-1 denotes the phase shift for the i -th transmit antenna across two adjacent frequency resources. [The value of θi is FFS.]

For OL SU-MIMO, the following schemes are FFS: 4Tx rate-1 SFBC + Antenna hopping, 4 Tx rate-2 Double SFBC + Antenna hopping, 4Tx rate-2 SM + Antenna hopping, 4Tx rate -3 SM + Antenna hopping, 4 Tx rate-3 hybrid SM + SFBC + Antenna hopping.

11.8.2.1.1.1 Transmit Diversity

The following transmit diversity modes are supported for open- loop single-user MIMO:

- 2 Tx rate 1: STBC/SFBC, and rank -1 precoder

- 4 Tx rate-1: STBC/SFBC with precoder, and rank-1 precoder

- 8 Tx rate -1 STBC/SFBC with precoder, and rank -1 precoder

For the transmit diversity modes with M=1, the input to MIMO encoder isx=s1, and the output of the MIMO encoder is a scalarz=x.

- The output of the rank-1 precoder for NT=2,4 and 8Tx antennas is a NT×1 matrixy=W×zwhere W may be frequency and /or time dependence as described in section 11.8.2.1.1.

- For the transmit diversity mode with M = 2, the input to the MIMO encoder is represented a 2×1vector. The MIMO encoder generates 2Tx SFBC, and then multiplied byNT×2 matrix as described in section 11.8.2.1.1

For the transmit diversity modes, the input to the MIMO encoder is represented as a 2×1 vector

x =s1s2, Equation 5

The output of the MIMO encoder is a 2×2 matrix

z =s1−s2s2s1, Equation 6

For the 2 Tx rate -1 modes, the output of the precoder is a 2×2 matrix

y = z, Equation 7

For the 4 Tx rate -1 with precoder, the output of the precoder is a 4×2 matrix

y = Wz, Equation 8

where W is a 4×2precoder. Note that W may be frequency and/or time dependent as described in section 11.8.2.1.1.

For the 8 Tx rate -1 with precoder, the output of the precoder is a 8×2 matrix

y = Wz, Equation 9

where W is a8×2 precoder. Note that W may be frequency and/or time dependent as described in section 11.8.2.1

11.8.2.1.1.2 Spatial Multiplexing

The following spatial multiplexing modes are supported for open-loop single-used MIMO:

- Rate -2 spatial multiplexing modes:

- . 2 Tx rate-2: rate 2 SM

- . 4 Tx rate-2: rate 2 SM with precoding

- . 8 Tx rate-2: rate 2 SM with precoding

- Rate -3 spatial multiplexing modes:

- . 4 Tx rate-3: rate 3 SM with precoding

- . 8 Tx rate-3: rate 3 SM with precoding

- Rate -4 spatial multiplexing modes:

- . 4 Tx rate-4: rate 3 SM

- . 8 Tx rate-4: rate 3 SM with precoding

For the rate -2 spatial multiplexing modes, the input to the MIMO encoder is represented as a2×1vector

x =s1s2, Equation 10

The output of the MIMO encoder is a2×1vector

z = x, Equation 11

For the 2Tx rate -2 modes, the output of the precoder is a 2×1 vector

y = z, Equation 12

For the 4Tx rate -2 mode with precoding, the output of the precoder is a 4×1 vector

y = Wz, Equation 13

where W is a 4 2 precoder. Note that W may be frequency and/or time dependent as described in section 11.8.2.1.1.

For the 8Tx rate -2 mode with precoding , the output of the precoder is a 8×1 vector

y = Wz, Equation 14

where W is a 82 precoder. Note that W may be frequency and/or time dependent as described in section 11.8.2.1.1.

For the rate -3 spatial multiplexing modes, the input to the MIMO encoder is represented as a3×1 vector

x =s1s2s3, Equation 15

The output of the MIMO encoder is a3×1 vector

z = x, Equation 16

For the 4Tx rate -3 mode with precoding, the output of the precoder is a 4×1 vector

y = W z, Equation 17

where W is a 4 3 precoder. Note that W may be frequency and/or time dependent as described in section 11.8.2.1.1.

For the 8Tx rate -3mode with precoding , the output of the precoder is a8×1 vector

y = W z, Equation 18

where W is a 8 3 precoder. Note that W may be frequency and/or time dependent as described in section 11.8.2.1.1.

For the rate -4 spatial multiplexing modes, the input to the MIMO encoder is represented as a4×1 vector

x =s1s2s3s4, Equation 19

The output of the MIMO encoder is a4×1 vector

z = x, Equation 20

For the 4Tx rate -4 mode, the output of the precoder is a 4×1vector

y = z, Equation 21

For the 8Tx rate -4 mode with precoding , the output of the precoder is a8×1 vector

y = W z, Equation 22

where W is a 8 4 precoder. Note that W may be frequency and/or time dependent as described in section 11.8.2.1.1.

====================End of Proposed Text====================