| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Avoiding Control Channel Femto-Macro Interference. |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Guang Han, Stavros Tzavidas Motorola 1501 W. Shure Dr. Arlington Heights IL 60004 | E-mail: guang.han@motorola.com stavros.tzavidas@motorola.com |
| Re: | IEEE 802.16m-08/040 “Call for Comments and Contributions on Project 802.16m System Description Document (SDD)” on the topic of “Femtocells” |  |
| Abstract | This document proposes an approach to avoid femtocell interference with macro base station control channels |  |
| Purpose | Review and adopt proposed text changes in 802.16m system description document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Avoiding Control Channel Femto-Macro Interference.

Guang Han, Stavros Tzavidas

Motorola

# Introduction

Femtocells are small base stations deployed indoors to improve network coverage and capacity. In typical deployments, femtocells will be purchased and owned by consumers and will be “closed access”, i.e. consumers will configure their femto cells such that they provide service only to a small number of subscribers (e.g. family members).

Closed access creates unique challenges for users who are close to a femto cell but are not allowed to access it and who instead attempt to communicate with a macro cell on the same frequency as the femto cell. The received signal strength from the femtocell will be much larger than that of the macro base station. Consequently, these users will experience what we term “femto-macro interference”: high received signal strength from a femto cell that a user is not permitted to access while attempting to instead connect to a macro base station at the same frequency. This interference may eventually lead the user to completely lose the connection and we therefore need specific mechanisms to allow such users to avoid femto-macro cell interference.

Macro Cell (MC)

Mobile Station (MS)

Femto Cell (FC)

MS attempts to communicate with MC, while not allowed to access FC (closed access FC)

Referring to the figure above, an MS that is close to a femto cell (but is not allowed to access it) will instead attempt to connect to the nearest macro cell. However, if the femto and macro cells are in the same frequency and if they both transmit their control signaling (e.g. BCH) using the (static and pre-determined) same resource blocks, then the femto cell transmissions will “mask” the control channel transmissions from the macro BS and will not allow the MS to decode BCH from the macro cell. Consequently,  the interference created by the femto cell may lead to a drop in the connection between the MS and the macro cell, or potentially may not even allow the MS to detect the macro BS.

A solution to this problem is to allow the femto cell to transmit its control signaling (e.g. BCH) using different resource block configurations. For example the femto cell can transmit its BCH at a later time than the macro BS, thus avoiding interference. Note that this interference problem is much less severe for the SCH, due to the orthogonality provided in this channel. We can therefore use the SCH to signal to the MS the existence of a femto cell that is transmitting its control signaling using different resource blocks.

# Proposed Text Changes

[add the following subsection in Section 17 “Support for Femtocell” on page 104 of 802.16m-08/003r4]

17.x Interference mitigation

17.x.1 Interference mitigation for control channels

Femtocells can transmit their control signaling (e.g. BCH) using different resource block arrangements than those used by a macro cell in the same frequency, in order to avoid creating interference for MSs communicating with the macro cell. The MS can distinguish different resource block arrangements for control signaling based on synchronization channel (SCH) that differentiates between macrocells and Femtocells.