| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed SDD Text for STC HARQ |  |
| Date Submitted | 2008-11-04 |  |
| Source(s) | Robert Novak, Sophie Vrzic, Dongsheng Yu, Hosein Nikopourdeilami, Mo-Han Fong, Jun Yuan, Kathiravetpillai Sivanesan Nortel Networks Tao Wu, Yinwei Zhao, Qi Li, Hongjie Si, Chongli Liu Huawei Wataru Matsumoto**,Toshiyuki Kuze** Philip V. Orlik++, Andreas F. Molisch++, Ramesh Annavajjala++ **Mitsubishi Electric Corporation ++Mitsubishi Electric Research Lab. Yakun Sun, Jungwon Lee Marvell Semiconductor Inc. Xiaolu Dong, Ying Du CATR | E-mail: rnovak@nortel.com mhfong@nortel.com E-mail: liuchongli@huawei.com E-mail: Matsumoto.Wataru@aj.MitsubishiElectric.co.jp {porlik, molisch, annavajjala}@merl.com E-mail: {yakunsun, junglee}@marvell.com E-mail: dongxiaolu@mail.ritt.com.cn duying@mail.ritt.com.cn |
| Re: | TGm SDD : Other In response to IEEE 802.16m-08/040 “Call for Contributions and Comments on Project 802.16m System Description Document (SDD)” for Session 58 |  |
| Abstract | This contribution proposes SDD text to support STC HARQ |  |
| Purpose | Discuss and adopt the text proposed in this document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed SDD Text for STC HARQ Subpacket Combining

Robert Novak, Sophie Vrzic, Dongsheng Yu, Hosein Nikopourdeilami, Mo-Han Fong, Jun Yuan, Kathiravetpillai Sivanesan

Nortel Networks

Tao Wu, Yinwei Zhao, Qi Li,

Hongjie Si, Chongli Liu

Huawei

Wataru Matsumoto**,Toshiyuki Kuze** Philip V. Orlik++, Andreas F. Molisch++, Ramesh Annavajjala++                   **Mitsubishi Electric Corporation

++Mitsubishi Electric Research Lab.

Yakun Sun, Jungwon Lee

Marvell Semiconductor Inc.

Xiaolu Dong, Ying Du

CATR

- Introduction

STC HARQ subpacket combining is an enhancement for STC schemes and supported in IEEE 802.16e. Contributions IEEE C802.16m-HARQ-08/41r2 and IEEE C802.16m-08-1194r1 propose enhancements to the existing method for use in IEEE 802.16m systems. This contribution proposes SDD text to support STC subpacket combining as well as the proposed enhancements.

- Text Proposal

------------------------------------------------Start of suggested text -----------------------------------------------------------

## 11.x.1.2 HARQ

### 11.x.1.2.5 MIMO HARQ

For HARQ subpacket retransmission, the mapping of bits or modulated symbols to spatial streams may be

applied to exploit spatial diversity with given mapping pattern, depending on the type of IR. In this case, the

predefined set of mapping patterns should be known to both transmitter and receiver. The specific mechanism is

FFS and it should be determined with the consideration of MIMO architecture and data processing for IEEE

802.16m.

STC subpacket combining should be supported for HARQ. For the case of 2 transmit antennas, STC subpacket combining scheme specified in IEEE 802.16e shall be reused by IEEE 802.16m. For the cases of 3 transmit antennas and 4 transmit antennas, the current design in IEEE 802.16e shall be enhanced by introducing interference cancellation to spatial diversity and spatial multiplexing scenarios, or alternatively, introducing STC code mappings for subsequent HARQ transmissions to fully exploit the spatial diversity for all bits or modulated symbols. The support STC subpacket combining is FFS

------------------------------------------------End of suggested text ------------------------------------------------------------