| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment for Interference Mitigation using Fractional Frequency Reuse |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Zhifeng (Jeff) Tao, Weihuang Fu, Andreas F. Molisch, Philip V. Orlik, Jinyun Zhang Mitsubishi Electric Research Lab Toshiyuki Kuze Mitsubishi Electric Corp | Voice: 617-621-{7558, 7557, 7570, 7595} Fax: 617-621-7550 {molisch, tao, porlik, jzhang}@merl.com Voice: +81-467-41-2885 Fax: +81-467-41-2486 Kuze.Toshiyuki@ah.MitsubishiElectric.co.jp |
| Re: | Response to the Call for Comments and Contributions on Project 802.16m System Description Document (SDD): IEEE 802.16m-08/040 |  |
| Abstract | This contribution proposes further clarification and enhancement for FFR section in the current 802.16m system description document (SDD). |  |
| Purpose | To adopt the changes proposed herein into IEEE 802.16m system description document (SDD). |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment for Interference Mitigation Using Fractional Frequency Reuse

Zhifeng (Jeff) Tao, Weihuang Fu, Andrea F. Molisch, Philip V. Orlik, Jinyun Zhang

Mitsubishi Electric Research Lab

Toshiyuki Kuze

Mitsubishi Electric Corp

# Introduction

Fractional Frequency Reuse (FFR) has been proposed in section 20.1 of the Draft IEEE 802.16m System Description Document [1].  For DL FFR, the example to integrate FFR with DL power control is shown in Fig. 1.  However, such allocation principle is not efficient for IEEE802.16m network with non-uniform traffic loads.

Figure 1: Example to integrate FFR and DL power control [1]

The method adjusting all corresponding frequency partitions simultaneously in an aligned manner to adapt to traffic loads cannot effectively utilize the spectrum. We will illustrate this problem and propose a simple solution in Section 2.

# Proposed Scheme

In our proposed scheme, we adjust the frequency allocation in partitions in conjunction with DL power level. The principles are shown by the example illustrated in Fig. 2.  Although the results are shown for cells, it is equally applicable to sectorized deployment. Suppose totally 12 sub-channels (indexed by hexadecimal) are allocated for 12 cells. Fig. 2(a) shows the problems that cluster A adjusts its cluster without coordinating with other clusters. While cluster A extends sub-channel 4 for the edge sub-band of cell A1, sub-channel 4 will interfere with the usage of this channel at the edge zones of cells C2 and D2. If cluster A extends sub-channel 9 for the center sub-bands in the cluster, it will interfere with the same sub-channel used by B3 and C3. The interfered sub-channels at the cell are denoted by the shaded boxes.

Fig. 2(b) shows an allocation solution yielded by our proposed scheme. Sub-channel 1, 2, 3, and 4 are allocated for A1 edge zone. The sub-channels of other cells are adjusted to mitigate interference while ensure the number of needed sub-channels at each cell. Cells A2 and A3 are able to utilize sub-channels 9 and a, respectively, for the center zone throughput and adjust the DL power according to the sub-channel usage of adjacent cell edge zones. Such frequency resource rearrangement and DL power adjustment enable each cell to have different amount of resource for partitions according to its requirement and employ the residual capacity of the sub-channels.

Figure 2: (a) Allocation problem and (b) solution provided by the proposed scheme

# Proposed Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

20.1 Interference Mitigation using Fractional Frequency Reuse (FFR)

[Modify the paragraph as follows]

Resource allocation in an FFR system takes several factors into consideration such as reuse factor in partition, power at partition, traffic load in partition, available multi-antenna technologies, as well as interference-based measurements taken at MS.

[Modify the paragraph as follows]

The Figure 46 shows an example to integrate FFR with DL power control. This allows the system to adaptively designate different DL power boosting over different PRUs in each frequency partition. The power allocation of each PRU may be higher or lower than normal level, it should be well coordinated from system-wide consideration.  With proper DL power control and sharing of frequency resource between sector center and sector edge, the boundary of corresponding frequency partition in different sectors need not be aligned.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# Reference

[1] Shkumbin Hamiti, “The Draft IEEE 802.16m System Description Document”, IEEE 802.16m-008/003r5

Weihuang Fu is affiliated with University of Cincinnati, Cincinnati, OH.