| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Performance Metrics for Relay Evaluation Methodology |  |
| Date Submitted | 2008-11-13 |  |
| Source(s) | Jerry Sydir Alexander Maltsev Andrey Pudeyev Andrey Chervyakov Vadim Sergeyev Alexey Khoryaev Alexei Davydov Intel Corporation Jianfei Tong Huawei | jerry.sydir@intel.com tongjianfei@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | TGmEVAL Change Request for 16m EMD |  |
| Abstract | In this document we provide a definition of relay specific metrics to flesh out the metrics section of the relay chapter of the EMD. |  |
| Purpose | For consideration and adoption into the 16m EMD document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Performance Metrics for Relay Evaluation Methodology

Jerry Sydir, Alexander Maltsev, Andrey Pudeyev, 
Andrey Chervyakov, Vadim Sergeyev, Alexey Khoryaev, Alexei Davydov

Intel Corporation

# Introduction

The relay evaluation methodology section which was included in the 16m EMD in session 57, indicates that a metric which measures the coverage/capacity tradeoff is of particular importance for relay. The definition of this metric for the case of relay was left for further study. The current EMD defines such a metric as does the 16j EVM. This contribution draws on the definitions from those two documents to specify the definition that should be used for relay in 16m.

# Text Proposal

[Modify the text in section 14.8 of the 16m EMD as indicated below:]

## 14.8 Performance metrics

Most of the performance metrics proposed in section 13 can be applied to the relay case without any additions, since they are derived from the user data throughput (Section 13.2.1.1). The users are scheduled on a per-frame basis; the resources of each frame are distributed between access (BS-MS, RS-MS) and relay (BS-RS) links. The relay link overhead is automatically taken into account in a user data throughput metric since part of frame resources are given to a BS-RS link by the scheduler. Therefore, definition of the basic performance metrics, such as user throughput and sector throughput are the same as in section 13.

### 14.8.1 System performance metrics

#### 14.8.1.1 Spectral efficiency and aggregate sector throughput

Spectral efficiency metrics and cell/sector throughput metrics should be considered in the sense of aggregate sector throughput going through the BS (including useful traffic from all RSs in the sector).

Coverage/capacity tradeoff and latency metrics are specifically important for the relay performance analysis. Both metrics are covered by Section 13, but specific definition is required for relays. The 16j EVM [83] specifies a Combined Coverage and Capacity metric (section 4.2.1). This metric may be used as the basis for a coverage/capacity tradeoff metric for 16m relay. Details of how to apply this metric to 16m relay are left for further study.

#### 14.8.1.2 Combined Coverage and capacity index

The Combined Coverage and Capacity index (cc)  is the number of simultaneous users per sector that can be supported achieving a target information throughput Rmin with specified coverage reliability (x%, 95% by default). The definition of this metric is based on the “Combined Coverate and Capacity Index Metric, Method One” as specified in the 802.16j EVM in section 4.2.2.

The metric is calculated individually within each zone of the frame structure and the individual values are summed as shown in the formula below. Assume that N MSs are dropped uniformly in the service area. For the MSs assigned to each zone, the simulator calculates the achievable data rate ri, based on the assigned MCS and available time-frequency resources within the zone. All MSs are sorted in descending order and only the top x% of the MSs are considered for further calculations. If k is the number of MSs that were selected for consideration, then Combined Coverage and Capacity index is:

cc=zones11ki=1kRminri

If min(ri) < Rmin, than cc = 0 and service with Rmin throughput cannot be provided with the required coverage, regardless the number of users.

For large N, coverage and capacity index cc approaches the expected value of the number of users that can be supported by the system for service requiring Rmin throughput with the given x% coverage.

### 14.8.2 Relay specific performance metrics

The described relay evaluation methodology requires introducing several associated performance metrics in addition those defined for non-relay case.

#### 14.8.2.1 Relay link overhead percentage

The Relay overhead is defined as the average number of additional slots required to transmit data from BS to RS for further distribution to MS associated with the RS relative to the total number of slots in a frame. Since the overhead is defined with respect to the time and frequency slots (not the amount of data), it reflects the efficiency of BS to RS links and takes into account possible spatial multiplexing of BS to different RS stations that may be exploited.

#### 14.8.2.2 Relay link average SE

This metric characterizes the BS-RS link quality in terms of spectral efficiency. This is an important characteristic of the relay system and should be used for relay link overhead calculation, along with Relay link PER. Relay link average SE is measured in the same way as access link average SE as described in Section 13.

#### 14.8.2.3 Relay link PER

Packet error rate for the BS-RS and RS-RS links defined as the ratio of the successfully received relay link packets to the total number of transmitted relay links packets. Relay link PER is measured in the same way as access link PER, as described in Section 13.