| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MS Association Procedure Proposal for Relay Evaluation Methodology |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Jerry Sydir Alexander Maltsev Andrey Pudeyev Andrey Chervyakov Vadim Sergeyev Alexey Khoryaev Alexei Davydov Intel Corporation Jianfei Tong Huawei | jerry.sydir@intel.com tongjianfei@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | TGmEVAL Change Request for 16m EMD |  |
| Abstract | In this document we provide a definition for the procedure by which the RS or BS to which an MS should be associated is determined in relay simulations. In the current version of the 16m EMD, an example for how this association can be performed is included, but the definition of the recommended procedure is left for further study. |  |
| Purpose | For consideration and adoption into the 16m EMD document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MS Association Procedure Proposal for Relay Evaluation Methodology

Jerry Sydir, Alexander Maltsev, Andrey Pudeyev, 
Andrey Chervyakov, Vadim Sergeyev, Alexey Khoryaev, Alexei Davydov

Intel Corporation

Jianfei Tong

Huawei

## Introduction

The relay evaluation methodology section which was included in the 16m EMD in session 57, indicates that the procedure by which the RS/BS to which a given MS should associate needs to be specified. The definition of this procedure is left for further study. This contribution proposes a definition for this procedure.b

# Text Proposal

[Modify the text in section 14.6 of the 16m EMD as indicated below:]

## MS Association

MS association is the procedure of selecting the BS or RS each MS within the desired sectors will associate with. A default MS association procedure should be is specified here. Other MS association procedures may be used to investigate advanced relay modes for which the default procedure is not appropriate. Proponents must describe the MS association procedure that was used in their simulation. The specifics of such a procedure are left for further study. One potential method is to assign MSs according to the following procedure:

- Estimate of the received signal strength (SNR) for each MS in the deployment delivered from each access station (BS and RS) of the deployment

- Associate the MS with the sector (BS or RS) delivering the maximum received signal strength.

- Final association to the BS or RSs may be performed on the base of effective throughput/spectral efficiency metrics, taking into account the relay link overhead for transmission via relay. This final association models the process by which the BS and RSs perform load-balancing within the sector. The definition of effective throughput is for further study.

The default MS Association procedure uses a one or two step association process. In the first step SNR is used to determine an initial association. SNR is estimated between each MS and each of the BS sectors and RSs. Each MS is associated with the BS or RS to which it has highest SNR. The second step is optional. In the second step effective capacity is calculated between each MS and the BS and RSs within the sector to which the MS was assigned. The effective capacity calculation may take into accountaccount SINR at the MS, relay link overhead and the possibility of resource reuse by BS and RSs. The MS is assigned to the BS or RS with which it has the highest value of effective capacity. MS Association and the details of the procedure need to be described by the proponent in sufficient detail in order to allow other members to duplicate the simulation in order to verify the results.

CBS−MSeffCBS−MS

CBS−MSeff=FBS-MS∗CBS−MS

FBS-MSFBS-MS

CRS−MSeff=Freuse∗CRS−MS1+(Freuse−1)∗CRS−MSCBS−RSeff

FreuseN=FreuseFreuse

CBS−RSeffCBS−MSeff

CBS−RSeff=FBS-RS∗CBS−RS

FBS-RS