| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Relay Link Modeling Proposal for Relay Evaluation Methodology |  |
| Date Submitted | 2008-11-13 |  |
| Source(s) | Jerry Sydir Alexander Maltsev Andrey Pudeyev Andrey Chervyakov Vadim Sergeyev Alexey Khoryaev Alexei Davydov Intel Corporation | jerry.sydir@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | TGmEVAL Change Request for 16m EMD |  |
| Abstract | In this document we define the procedure for modeling the relay link. |  |
| Purpose | For consideration and adoption into the 16m EMD document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Relay Link Modeling Proposal for Relay Evaluation Methodology

Jerry Sydir, Alexander Maltsev, Andrey Pudeyev, 
Andrey Chervyakov, Vadim Sergeyev, Alexey Khoryaev, Alexei Davydov

Intel Corporation

# Introduction

The relay evaluation methodology section which was included in the 16m EMD in session 57, left for further study the procedure for modeling the relay link. This contribution proposes a definition for this procedure.

# Text Proposal

[Modify the text in section 14.5 as indicated below:]

## 14.5 Simulation Procedure and Flow

- Scheduling

  - The generic proportional fair scheduler specified in appendix F, is used for allocation of the resources.

  - The access link is simulated explicitly for all test scenarios. It may or may not be necessary to simulate the relay link explicitly as the relay link is assumed to be fairly static in the ART test scenario. More study is required to specify the simulation procedure for the relay link.

  - The resources used for relay link transmissions are determined as part of the scheduling of each RS access link transmission. The relay link resources that can be used within each frame are limited by the size of the zone in which relay transmissions are performed. If the relay zone resources are used up, no additional RS access link transmissions can be scheduled in that frame. Proponents should specify the average relay link spectral efficiency that was assumed for a given simulation.

    - For test scenarios in which a static relay link is assumed, the average relay link spectral efficiency shall be used to calculate the relay link resources for all transmissions.

    - For test scenarios in which a dynamic relay link is assumed the spectral efficiency of the relay link is calculated explicitly for each transmission by determining the appropriate MCS and corresponding packet error rate. This spectral efficiency is used to calculate the amount of allocated for relay link resources for the given transmission. The relay link spectral efficiency is calculated to take into account the PER of the relay link:

SE = SEMCS(1-PER),

where SEMCS  spectral efficiency of selected modulation coding scheme and PER the packet error rate.

For evaluation of HARQ protocols the HARQ process is simulated explicitly on each hop. The procedure as well as details of the HARQ simulation model (queuing, buffer size, etc) need to be specified by proponent in sufficient detail in order to allow other members to recreate the simulation in order to verify the results.

[Insert the following text into section 14.1.1 on page 132 after line 14]

In ART scenarios, the relay link is modeled as a static link. See section 14.5 for the modeling procedure for static links.

[Insert the following text into section 14.1.2]

In BRT scenarios, the relay link is modeled as a dynamic link. See section 14.5 for the modeling procedure for dynamic links.