| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Synchronization Channel Structure for the IEEE 802.16m |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Sungho Moon LG Electronics | Voice : +82-31-450-1852 E-mail: msungho@lge.com |
| Re: | Re: TGm SDD: Other; IEEE 802.16m-08/040, Call for Detailed Comments on the IEEE 802.16m-08/003r5 |  |
| Abstract | The contribution proposes a candidate for the IEEE 802.16m SCH structure based on the document C802.16m-08/003r5. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Synchronization Channel Structure for the IEEE 802.16m

Sungho Moon

LG Electronics

# Introduction

In the previous meeting, 16m members agreed to use a hierarchical structure, but the location and periodicity of synchronization symbols was not decided. Thus, for further evaluations on the performance of the IEEE 802.16m synchronization channel, we should decide a high-level synchronization channel (SCH) structure through this meeting.

# Proposed SCH Structure

- Since P-SCH is used for the superframe synchronization, P-SCH should be located in the SFH with a span of one OFDM symbol. Three S-SCH symbols should be located every 5ms after one P-SCH. Each S-SCH consists of one OFDM symbol.

# Text Proposal for the 802.16m SDD

============================== Start of Proposed Text =================================

[Adopt the following modifications:]

11.7.2.1.2.1.4 Location of synchronization symbols

In mixed deployments, the presence of the 16e preamble in the first symbol of the 16e frame is implicit. The location of the SCH symbol(s) is fixed within the superframe.

The 16m SCH is transmitted in the first subframe of every 16m frame as The location of SCH is shown in Figure X-1, and detailed location of SCH symbols within a subframe is FFS.

Figure X-1 Location of P-SCH and S-SCH

============================== End of Proposed Text =================================