| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modulation and Coding Set designing for 16m system |  |
| Date Submitted | 2008-10-31 |  |
| Source(s) | Hua Zhou, Sengbao Liang, Yuanrong Lan; Jie Zhang, Yuantao Zhang, Jinyin Xue, Jian Wang, Jun Tian Fujitsu R&D Center Ltd. | { zhouhua; sengbaol; lanyr; jie.zhang; zyuantao; xuejy;jianwang; tianjun}@cn.fujitsu.com |
| Re: | In response to the TGm Call for Contributions and Comments IEEE 802.16m-08/040 on 802.16m-08/003r5 |  |
| Abstract | Comment on Channel coding scheme in 802.16m System Description Document |  |
| Purpose | Proposal for consideration in the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Modulation and Coding Scheme designing for 16m system

FUJITSU

# Introduction

The existing 16e MCS for data transmission is based on punctured CTC and QAM, which have the following performance as showing blow.

Figure 1 BLER performance of MCS sets in 16E standard

Here, we find that if the target BLER is 10%, the SINR threshold for different MCS is 1.5dB, 4.5dB, 7.0dB, 10.5dB, 11.5dB, 15dB, 16dB, and 17.5dB. The SINR distance between two adjacent MCS levels is 3dB, 2.5dB, 3.5dB, 1.0dB, 3.5dB, 1dB, and 1.5dB, which have big variance. In some cases, this set of MCS levels will decrease system throughput, for example, most users in one system can have limited SINR range, such as, from 0dB to 10dB, but in this range, the MCS satisfied for target BLER for user can be only selected from 3 MCS levels (QPSK ½, QPSK ¾, 16QAM 1/2,). On the other hand, if the SINR distribution range can be roughly estimated, appropriate MCS levels will be assigned to optimize the system throughput.

Figure 2 shows the PDF of instantaneous SINR for 16e baseline system. From this figure, we can see that most of SINR value are between -10dB and 15dB. It means that a higher system throughput can be explored if the users are optimized with SINR values between -10dB and 15dB. One straightforward solution is to assign more MCS levels in certain SINR area, i.e., we design more MCS level for SNR with high probability, and less MCS level for SNR with less probability. It can be found that the MCS levels in 16e system are not reasonably designed from the BLER performance in figure 1, i.e., too many MCS levels greater than 15dB, less MCS levels between -10dB and 15dB. We recommend that more MCS levels should be added from -10dB to 15dB.

Figure 2 Distribution of Instantaneous SINR for 16e baseline system

# MCS design

For illustrating MCS set design by example, figure 3 shows a modified MCS sets with more MCS level and its corresponding BLER performance over AWGN channel.

Figure 3 Performance of Modified MCS set in AWGN channel

The following table shows the detail MCS set with coding rate and modulation scheme, and the corresponding SNR value for each MCS.

| MCS Index | Target Code Rate | Mod Order | SNR | Spectral Efficiency |
| --- | --- | --- | --- | --- |
| 0 | 0.1333(2/15) | 2 | -4.97dB | 0.2667 |
| 1 | 0.2174(5/23) | 2 | -2.85dB | 0.4348 |
| 2 | 0.3448(10/29) | 2 | -0.84dB | 0.6897 |
| 3 | 0.4762(10/21) | 2 | 0.91dB | 0.9524 |
| 4 | 0.6000(11/18) | 2 | 2.52dB | 1.2000 |
| 5 | 0.3157(6/19) | 4 | 3.98dB | 1.2632 |
| 6 | 0.4167(5/12) | 4 | 5.40dB | 1.6667 |
| 7 | 0.4848(16/33) | 4 | 6.56dB | 1.9394 |
| 8 | 0.5556(5/9) | 4 | 7.52dB | 2.2222 |
| 9 | 0.6250(5/8) | 4 | 8.61dB | 2.5000 |
| 10 | 0.7142(5/7) | 4 | 9.94dB | 2.8571 |
| 11 | 0.5000(1/2) | 6 | 11.56dB | 3.0000 |
| 12 | 0.6111(11/18) | 6 | 13.19dB | 3.6667 |
| 13 | 0.7037(19/27) | 6 | 14.94dB | 4.2222 |
| 14 | 0.8000(4/5) | 6 | 16.64dB | 4.8000 |
| 15 | 0.9048(19/21) | 6 | 18.94dB | 5.4286 |

In link level simulation, we can get single user throughput as showing below. It’s shown from this figure that new MCS has better throughput, especially in the SNR value about 10dB.

# System level simulation results

The following two figures are the CDF of user throughput with two different MCS set in system level simulation, from which, we can get the sector throughput and user coverage in the table.

Figure 4 User throughput for SISO system

Figure 5 user throughput for MIMO (2x2) system

|  | Sector Throughput (bps/Hz) |  | User Coverage 5%(bps/Hz) |  |
| --- | --- | --- | --- | --- |
|  | SISO | MIMO (2x2) | SISO | MIMO (2x2) |
| 16e MCS table | 1.06 | 1.41 | 0.020 | 0.027 |
| New MCS table | 1.13 | 1.57 | 0.022 | 0.032 |
| Performance Gain | 6.6% | 11.3% | 10% | 18% |

# Text proposal

Insert the following text into SDD Sections indicated below

---------------------------text start-----------------------------

### 11.13 Channel coding and HARQ

### 11.13.1.6 Modulation and Coding Set

| MCS Index | Target Code Rate | Modulation Order |
| --- | --- | --- |
| 0 | 0.1333(2/15) | 2 |
| 1 | 0.2174(5/23) | 2 |
| 2 | 0.3448(10/29) | 2 |
| 3 | 0.4762(10/21) | 2 |
| 4 | 0.6000(11/18) | 2 |
| 5 | 0.3157(6/19) | 4 |
| 6 | 0.4167(5/12) | 4 |
| 7 | 0.4848(16/33) | 4 |
| 8 | 0.5556(5/9) | 4 |
| 9 | 0.6250(5/8) | 4 |
| 10 | 0.7142(5/7) | 4 |
| 11 | 0.5000(1/2) | 6 |
| 12 | 0.6111(11/18) | 6 |
| 13 | 0.7037(19/27) | 6 |
| 14 | 0.8000(4/5) | 6 |
| 15 | 0.9048(19/21) | 6 |

------------------------------text end---------------------------

# Simulation parameters

| Parameters | Values |
| --- | --- |
| Bandwidth | 10 MHz |
| FFT size | 1024 |
| Carrier frequency | 2.5 GHz |
| Subframe structure | According to the latest revision of IEEE 80216m-08/003r5 |
| Channel Model | Modified ITU PedB/VA with spatial correlation |
| Mobile speed | 3 km/h |
| Antenna configuration | 1x1, 2x2 |
| Receiver algorithm | MMSE |
| Channel coding & modulation | 16e 11 MCS & new MCS |
| Channel estimation | Perfect channel estimation |
| HARQ scheme (async. vs. sync, IR vs. Chase Combining, adaptive vs. non-adaptive, retransmission delay) | Synchronous HARQ with Chase Combining, Retransmission delay = 8 TTI with maximum 3 retransmissions and initial target PER of 10% |
| CQI feedback period and delay | Every 8 TTI |
| Scheduling method | PF |
| Cell to cell distance | 500m |
| BTS power | 46dbm |
| Shadow fading | 8dB |
| UE noise figure | 9dB |
| Link adaptation method | RBIR for MMSE receiver and SISO |

# References

[1] IEEE 802.16m-07/002r4, “802.16m System Requirements”

[2] IEEE 802.16m-08/003r5,”802.16m system description document”