| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Pilot design for 8 Tx antennas in 16m system |  |
| Date Submitted | 2008-11.10 |  |
| Source(s) | Jie Zhang, Hua Zhou, Jun Tian, Jinyin Xue, Yuantao Zhang, Jian Wang Fujitsu Research & Development Center Co., Ltd. | Voice: +86-10-5969-1525 Fax: +86-10-5969-1504 {jie.zhang, zhouhua, tianjun, xuejy, zyuantao, jianwang}@cn.fujitsu.com |
| Re: | SDD Session 57 Cleanup; in response to the TGm Call for Contributions and Comments 802.16m-08/003r5 for Session 58 |  |
| Abstract | Comment on 802.16m System Description Document |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Pilot design for 8 Tx antennas in 16m sytem

Jie Zhang, Hua Zhou, Jun Tian, Jinyin Xue, Yuantao Zhang, Jian Wang
Fujitsu Research & Development Center Co., Ltd.

Introduction

As specified of antenna configuration in current SDD (IEEE 802.16m-08/003r5), the maximum number of Tx/Rx antenna settings is (8, 8). Moreover, for open-loop spatial multiplexing and closed-loop SU-MIMO, 8 streams with 8 Tx/8 Rx is FFS. This document suggests 8-stream transmission and proposes a pilot pattern designed for 8 Tx antennas configuration.

Proposed Changes:

In SDD Section 11.8.1.2 of IEEE 802.16m-08/003r5, modify line 19~20 of page 77 as following:

For open-loop spatial multiplexing and closed-loop SU-MIMO, the number of streams, NS≤minNT,NR, where NS is no more than 4 (8 streams are FFS). For open-loop transmit diversity modes.........

In line 8 of page 60, Insert the following figure

Fig. xx Pilot Pattern C for 8 stream pilots, pilot k denotes pilot for transmit antenna k.

Appendix

Simulations Parameter Settings

- Channel Model

  - ITU 6-path channel

  - Type: PB 3km/h, VA 30km/h, VA 60km/h

- Antenna Configuration

  - 8 Tx x 8Rx , no correlation on both Tx and Rx sides

- Channel Estimation

  - 2-D MMSE

- Pilot power boosting: 9 dB

- Performance Evaluation

  - Mean square error in channel estimation

Fig. 1. MSE over PB 3Km/H channels

Fig. 2. MSE over VA 30Km/H channels

Fig. 3. MSE over VA 60Km/H channels