| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed amendment text on 802.16m Downlink Physical Structure |  |
| Date Submitted | 2008-11-03 |  |
| Source(s) | Guan Yanfeng, Liu Xiangyu, Liu Ying, Lu Zhaohua, Zhu Dengkui, Fang Huiying ZTE Corporation | Voice: 086-0755-26770659 E-mail:guan.yanfeng@zte.com.cn |
| Re: | IEEE 802.16m-08/042 Call for Contributions on Project 802.16m Draft Amendment Content Topic: Downlink Physical Structure |  |
| Abstract | This contribution presents the outer permutation process for the DL resource mapping |  |
| Purpose | To be discussed and incorporated into the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed amendment text on 802.16m Downlink Physical Structure

Guan Yanfeng, Liu Xiangyu, Liu Ying, Zhu Dengkui, Lu Zhaohua, Fang Huiying

ZTE Corporation

# Introduction

In the current IEEE 802.16m SDD, section 11.5.2.2 has described the whole framework of the downlink resource mapping, however, many details in DL subcarrier to resource unit mapping are still TBD, such as the outer permutation process. In this contribution, we propose a reasonable solution to the outer permutation process.

# Proposed Scheme

BS shall implement the whole resource unit mapping according to the related resource configuration.

- First-level or outer permutation is applied to the PRUs in the units of N1 and N2 PRUs, where N1=4 (TBD) and N2 =1 or 2 depending on system bandwidth (TBD). All the PRUs are separated in the unit of N1 PRUs and reordered by the outer permutation (row-column permutation), and then reserve the PRUs in the unit of N1 PRUs for frequency partitions. Then the remained PRUs are separated in the unit of N2 PRUs and reordered by the permutation (row-column permutation). Direct mapping of outer permutation can be supported only for LLRU.

- Distributing the reordered PRUs into frequency partitions by reserving the PRUs in the unit of N1 PRUs and the PRUs in the unit of N2 PRUs for each frequency partition.

- The frequency partition is divided into localized(LLRU) and/or distributed(LDRU) resources for each resource group. Sector specific permutation can be supported and direct mapping of the resources can be supported for localized resources. The sizes of the distributed/localized resources are flexibly configured per sector (TBD). Adjacent sectors do not need to have same configuration of localized and distributed resources;

- The localized and distributed groups are further mapped into LRUs (by direct mapping of LLRU and by “Subcarrier permutation” on DRUs) as shown in the following figure.

A typical implementation for the 16m system with 5MHz bandwidth is shown in Fig.1 24 PRUs are separated into 6 parts (0, 1, 2, 3, 4, 5) in the unit of N1(4) PRUs and reordered by the row-column permutation into the (0, 3, 1, 4, 2, 5), and part 0 and 3 will be reserved for frequency partition 2. The remained PRUs are separated in the unit of N2 (1) PRU and reordered by the row-column permutation. According to the frequency partition configuration, the PRUs are reserved for each frequency partition and it will be separated further into the localized resources and/or distributed resources.

Another way is that part 1 and 4 will be directly reserved for frequency partition 2 after permutation in the unit of N1, and then other parts is permuted in the unit of N2.

In addition, the PRUs formed by the guard subcarriers can be mapped into the last frequency partition which includes the localized resources in the multi-carrier mode.

Fig. 1 – Proposed outer permutation for DL subcarrier to resource unit mapping

# Analysis

An important function of the outer permutation is to eliminate the correlation of the reordered physical resource units, especially for the distributed resource units. Therefore, we mainly evaluate the statistical property of channel coefficients of the permuted PRUs. Define the following complex channel coefficient vector and statistics.

The complex channel coefficient vector before the outer permutation is and the complex channel coefficient vector after the outer permutation is , where N is the number of the used subcarriers.

The mean vector of non-overlapping subsequence of Y:

Z shows the uniformity extents of the channel coefficients and will fluctuates slightly around the mean of Y for a good outer permutation.

we compare the proposed permutation with random permutation in ITU channel. Actually, the random permutation is a kind of ideal permutation. The evaluation criterions are shown in table 1. Obviously, the random extent generated by the proposed permutation very approach to the random extent generated by the random permutation.

Table 1: Evaluation criterions between the proposed permutation and random permutation

| Evaluation criterion | Proposed permutation |  |  |  | Random Permutation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ITU-PA | ITU-VA | ITU-PB | ITU-VB | ITU-PA | ITU-VA | ITU-PB | ITU-VB |
|  | 0.056 | 0.34 | 0.23 | 0.40 | 0.054 | 0.33 | 0.18 | 0.378 |

# Conclusions

In this contribution, the proposed outer permutation for DL subcarrier to resource unit mapping is discussed. This scheme is able to resolve the remained issues in the current DL resource unit mapping and meet the requirements in the current SDD. Therefore, we suggest to adopt the outer permutation procedure mentioned above for 16m DL subcarrier to resource unit mapping.

# References

[1] IEEE P802.16 Rev2 / D7, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[2] IEEE 802.16m-07/002r6, “802.16m System Requirements”.

[3] IEEE 802.16m-08/003r5, “The Draft IEEE 802.16m System Description Document”.

[4] IEEE C802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”.

Text proposal for the 802.16m amendment

---------------------------------------------------  Text Start  ---------------------------------------------------

15.3.5 Downlink physical structure

15.3.5.2 Downlink subcarrier to resource unit mapping

- First-level or outer permutation is applied to the PRUs in the units of N1 and N2 PRUs, where N1=4 (TBD) and N2 =1 or 2 depending on system bandwidth (TBD). All the PRUs are separated in the unit of N1 PRUs and reordered by the outer permutation (row-column permutation), and then reserve the PRUs in the unit of N1 PRUs for frequency partitions. Then the remained PRUs are separated in the unit of N2 PRUs and reordered by the permutation (row-column permutation). Direct mapping of outer permutation can be supported only for LLRU.

- Distributing the reordered PRUs into frequency partitions by reserving the PRUs in the unit of N1 PRUs and the PRUs in the unit of N2 PRUs for each frequency partition.

- The frequency partition is divided into localized (LLRU) and/or distributed(LDRU) resources for each resource group. Sector specific permutation can be supported and direct mapping of the resources can be supported for localized resources. The sizes of the distributed/localized resources are flexibly configured per sector (TBD). Adjacent sectors do not need to have same configuration of localized and distributed resources;

- The localized and distributed groups are further mapped into LRUs (by direct mapping of LLRU and by “Subcarrier permutation” on DRUs) as shown in the following figure.

---------------------------------------------------  Text End  ---------------------------------------------------