| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Physical and Logical Resource unit for the IEEE 802.16m Amendment |  |
| Date Submitted | 2008-11-03 |  |
| Source(s) | Yang-Han Lee, Yih-Guang Jan, Hsien-Wei Tseng, Ming-Hsueh Chuang Tamkang University (TKU) Youn-Tai Lee, Chun-Yen Hsu Institute for Information Industry (III) Shiann-Tsong Sheu National Central University (NCU) Whai-En Chen National Ilan University (NIU) Shih-Lin Wu Chang Gung University (CGU) | yhleepp@yahoo.com.tw lyt@nmi.iii.org.tw stsheu@ce.ncu.edu.tw wechen@niu.edu.tw slwu@mail.cgu.edu.tw |
| Re: | “802.16m amendment text”: IEEE 802.16m-08/042, “Call for Contributions on Project 802.16m Draft Amendment Content”. Target topic: “11.5 Downlink Physical Structure”. |  |
| Abstract | Proposed Text of Common pilot structure for the IEEE 802.16m Amendment |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Physical and Logical Resource unit for the

IEEE 802.16m Amendment

Yang-Han Lee, Yih-Guang Jan, Hsien-Wei Tseng, Ming-Hsueh Chuang

Tamkang University (TKU)

- Youn-Tai Lee, Chun-Yen Hsu

- Institute for Information Industry (III)

Shiann-Tsong Sheu

- National Central University (NCU)

- Whai-En Chen

- National Ilan University (NIU)

- Shih-Lin Wu

Chang Gung University (CGU)

- Introduction

This contribution provides the SDD text proposal to Physical and Logical Resource Unit. The Physical and Logical Resource unit is described and added in the 802.16m amendment text.

- Text proposal for inclusion in the 802.16m amendment

[In Section 11.5.1 Insert the following text]

===================Start of Proposed Text====================

11.5.1 Physical and logical Resource Unit

A physical resource unit (PRU) is the basic physical unit for resource allocation that comprises Psc consecutive subcarriers by Nsym consecutive symbols. Psc is 18 subcarriers and Nsym is 6 OFDMA symbols for type -1 subframes and Nsm is 7 OFDM symbols for type -2 subframes. A logical resource unit (LRU) is the basic logical unit for distributed and localized resource allocations. A LRU is 18*6 subcarriers for type-1 subframes and 18*7 subcarriers for type-2 subframes. Note that the LRU includes in its numerology the number of pilots that are used in a PRU, and may include control information. So, the effective number of data subcarriers in an LRU depends on the number of allocated pilots and control channel presence.

====================End of Proposed Text====================