| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Downlink Pilot Structure for the IEEE802.16m Amendment |  |
| Date Submitted | 2008-11-3 |  |
| Source(s) | Takaaki KISHIGAMI, Isamu YOSHII Panasonic Corporation Lei Huang Panasonic Singapore Laboratories | Voice: + 81-46-840-5123 E-mail: kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | IEEE802.16m-08/042(“Call for Contributions on Project 802.16m Draft Amendment Content”) |  |
| Abstract | We propose the text for the DL pilot structure for the subframe consisting of 5 and 7 OFDMA symbols for the P802.16m Amendment. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Downlink Pilot Structure
 for the IEEE802.16m Amendment

Takaaki KISHIGAMI, Isamu YOSHII

Panasonic Corporation

Lei Huang

Panasonic Singapore Laboratories

# Overview

In Section 11.5.3 of SDD [1], DL Pilot Pattern A is shown for 1/2 pilot streams for the subframe consisting of 6 OFDMA symbols. For the subframe consisting of 5 or 7 OFDMA symbols, one of OFDMA symbols is deleted or added. However, interlaced pilot patterns based on the Pilot pattern A in those subframe types are not clearly defined.

In this contribution, we clarify the Pilot Pattern A for the subframe consisting of 5 or 7 OFDMA symbols including interlaced pilot structure. And we also propose to introduce sector- or BS- specific orthogonal sequences (or CDM pilot) to the interlaced pilot in order to reduce inter-BS or inter-sector interference in the pilot symbols in the case that the collision between inter-sector pilots happens. As for the sector- or BS- specific orthogonal sequences, we have already shown the superior performance by link level simulation in the reference of [2].

# Modifications to the SDD Text

For Pilot Pattern A for the subframe consisting of 5 OFDMA symbols, the sixth symbol is deleted from the the subframe consisting 6 OFDMA symbols and the two pilot symbols are added in the fifth symbol to keep the number of pilot symbols as same as the 6 OFDMA symbol subframe and to ensure the channel estimation accuracy. Since the current SDD text includes the constraint that there is equal number of pilots for each PRU of a data burst assigned to one MS, our proposal could easily support that the data burst assigned to one MS include PRUs with different sizes.

For Pilot Pattern A for the subframe consisting of 7 OFDMA symbols, the seventh symbol not including pilot symbols is added in the end of the 6 OFDMA symbol subframe.

For interlaced pilot, we propose to introduce sector- or BS- specific orthogonal sequences (or CDM pilot) to the interlaced pilot in order to reduce inter-BS or inter-sector interference in the pilot symbols in the case that the collision between inter-sector pilots happens.

# Proposed Text

We propose the following the text for the P802.16m Amendment in Section x.5. Since most of part is maintained as Section 11.5.3 of SDD, the modifications to SDD are indicated by the yellow highlighted text.

-------------------------------  Start text proposal  ---------------------------------------------------

## x Advanced Air Interface

- x.5 Downlink Physical Structure

- x.5.y Pilot Structure

The transmission of pilot subcarriers in the downlink is necessary for enabling channel estimation, measurements of channel quality indicators such as the SINR, frequency offset estimation, etc. To optimize the system performance in different propagation environments and applications, IEEE 802.16m supports both common and dedicated pilot structures. The categorization in common and dedicated pilots is done with respect to their usage. The common pilots can be used by all MSs. Dedicated pilots can be used with both localized and distributed allocations.  Pilot subcarrriers that can be used only by a group of MSs is a special case of common pilots and are termed shared pilots. The dedicated pilots are associated with a specific resource allocation, can be only used by the MSs allocated to said specific resource allocation, and therefore can be precoded or beamformed in the same way as the data subcarriers of the resource allocation. The pilot structure is defined for up to four transmission (Tx) streams and there is a unified pilot pattern design for common and dedicated pilots. There is equal pilot density per Tx stream, while there is not necessarily equal pilot density per OFDMA symbol of the downlink subframe. Further, there is equal number of pilots for each PRU of a data burst assigned to one MS.

The pilots can be used for channel estimation, measurements (CQI and interference mitigation/cancellation), frequency offset estimation and time offset estimation. Pilot patterns are proposed for efficiency and performance. Pattern A is used for 1 and 2 DL data streams dedicated and common pilot scenarios.

Figure 1 Pattern A for 1/2 pilot streams.

For the subframe consisting of 5 or 7 OFDMA symbols, one of OFDMA symbols is deleted or added.

Figure 2 shows the Pattern A for the subframe consisting of 5 and 7 OFDMA symbols.

(a) subframe consisting of 5 OFDMA symbols    (b) subframe consisting of 7 OFDMA symbols

Figure 2 Pattern A for 1/2 pilot streams.

The interlaced pilot patterns can be generated by cyclic shifting the base pilot pattern. The interlaced pilot patterns are shown in Figure 3 and can be optionally used by different BSs or sectors.

Figure 4 and 5 show the interlaced pilot patterns for the subframe consisting of 5 and 7 OFDMA symbols. The different orthogonal sequence can be used in the different BSs or sectors to reduce inter-BS or inter-sector interference in the pilot symbols in the case that the collision between inter-sector pilots happens such as Figure 4. As for the BS or sector specific orthogonal sequence, frequency domain spreading is used by adding phase rotation. The BS- or sector- specific phase rotation (φ) of 0, 2π/3, and 4π/3 is added every adjacent pilot subcarrier to the BS or sector #1, #2, and #3, respectively.

The use of interlaced pilot pattern is FFS supported.

Figure 1 Interlaced Pattern A for 1/2 pilot streams.

Figure 4 Interlaced Pattern A for 1/2 pilot streams for the subframe consisting of 5 OFDMA symbols.

Figure 5 Interlaced Pattern A for 1/2 pilot streams for the subframe consisting of 7 OFDMA symbols

Pattern B is used for 4 data streams DL dedicated and common pilot pattern.

Figure 6 Pilot Pattern B for 4 stream pilots, pilot k denotes pilot for transmit antenna stream k

For the subframe consisting of 5 OFDMA symbols, the third or fourth OFDMA symbol is deleted. For the subframe consisting of 7 OFDMA symbols, the third or fourth OFDMA symbol is added in the end of the subframe.

-------------------------------  End text proposal  ---------------------------------------------------

# References

[1]IEEE 802.16m-08/003r5, “The Draft IEEE 802.16m System Description Document”

[2]IEEE C80216m-08/716, Panasonic, “Sector-Specific Common Pilot Structure in IEEE802.16m Downlink”