| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Downlink Pilot Structure for 8 Tx antennas for the IEEE802.16m Amendment |  |
| Date Submitted | 2008-11-3 |  |
| Source(s) | Takaaki KISHIGAMI, Isamu YOSHII Panasonic Corporation Lei Huang Panasonic Singapore Laboratories | Voice: + 81-46-840-5123 E-mail: kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | IEEE802.16m-08/042(“Call for Contributions on Project 802.16m Draft Amendment Content”) |  |
| Abstract | We propose the DL pilot structure for 8Tx antenna including the BS or sector-specific pilots for the P802.16m Amendment. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Downlink Pilot Structure for 8 Tx antennas
 for the IEEE802.16m Amendment

Takaaki KISHIGAMI, Isamu YOSHII

Panasonic Corporation

Lei Huang

Panasonic Singapore Laboratories

# Overview

In DL MIMO, the 8 Tx antenna configurations such as (NT, NR) = (8, 2), (8, 4), (8, 8) are included as described in Section 11.8.1.1 of SDD[1]. However, DL pilot structure for 8Tx is still open issue.

In this contribution, we propose the 8Tx pilot structure based on the 4 Tx pilot pattern in order to enable to re-use for 4 Tx pilot receiving hardware. And we also propose to introduce sector- or BS- specific orthogonal sequences (or CDM pilot) in stead of the interlaced pilot, since we have already shown the superior performance of the CDM pilot compared with the interlaced pilot by link level simulation in the reference of [2].

# Proposed Text

We propose the following text for the P802.16m Amendment in Section x.5

-------------------------------  Start text proposal  ---------------------------------------------------

## x Advanced Air Interface

- x.5 Downlink Phisical Structure

x.5.y Pilot Pattern for 8 Antennas

For 8 antenna pilot pattern, (k+4)th antenna pilots are allocated with adding m subcarrier offset and/or n OFDMA symbol offset to k-th antenna pilot allocation of the 4 Tx antenna pilot pattern A/B to avoid the collision between other antenna pilots. Here, k=5, 6, 7 or 8.

Figure 1 shows the example of the pilot pattern for 8 Tx antenna pilot with based Pilot Pattern B with the offset of (n,m)=(1,0).

The different orthogonal sequence can be used in the different BSs or sectors to reduce inter-BS or inter-sector interference. As for the BS or sector specific orthogonal sequence, frequency domain spreading is used by adding phase rotation. The BS or sector specific phase rotation(φ) of 0, 2π/3, and 4π/3 is added every adjacent pilot subcarrier to the BS or the sector #1, #2, and #3, respectively.

Figure 1 Pilot Pattern B for 8 stream pilots, pilot k denotes pilot for transmit stream k.

-------------------------------  End text proposal  ---------------------------------------------------

# References

[1]IEEE 802.16m-08/003r5, “The Draft IEEE 802.16m System Description Document”

[2]IEEE C80216m-08/716, Panasonic, “Sector-Specific Common Pilot Structure in IEEE802.16m Downlink”