| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Amendment Text on Downlink Physical Structure |  |
| Date Submitted | 2008-11-03 |  |
| Source(s) | HanGyu Cho, Jinsoo Choi, Seung-Hyun Kang, Jin Sam Kwak LG Electronic Inc. | Voice: +82-31-450-1857 E-mail: {hgcho, emptylie, sh_kang, samji}@lge.com |
| Re: | C80216m-08_042: Call for Contributions on Project 802.16m Draft Amendment Content Target Topic: Downlink Physical Structure |  |
| Abstract | Proposal of amendment text on Downlink Physical Structure |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m amendment text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Amendment Text on Downlink Physical Structure

HanGyu Cho, Jinsoo Choi, Seung-Hyun Kang, Jin Sam Kwak

LG Electronics

# 1 Introduction

In this contribution, the amendment text is proposed on Downlink Physical Structure in accordance with the IEEE 802.16m System Requirement Document [1] and System Description Document [2]. Any proposed text here is developed such that it can be readily combined with IEEE P802.16Rev2/D7 [3] and other active IEEE 802.16 amendment projects, as appropriate. In addition, this contribution follows the tentative outlines, style and format guidelines contained in [4].

# 2 Motivation

The section “Downlink physical structure” addresses DL subcarrier to resource unit mapping and pilot structure. Key concepts described in the current SDD text on DL physical structure are:

• 18 by Nsym physical resource unit (PRU) as a basic physical unit for resource allocation

• FDM-multiplexing of distributed and localized resources

• Frequency partitions for FFR and/or MBS

• Outer permutation with different permutation granularities N1 and N2

• Inner permutation in the unit of a pair of subcarriers for distributed allocation

• Unified pilot pattern design for common and dedicated pilots for 1, 2, and 4 streams considering all the sub-

frame types

What this contribution does is to clarify the SDD text, to correct editorial mistakes, and to provide technical details fit for amendment text as much as possible.

# 3 Key proposals in Subsection 15.3.5

The main technical proposals to be adopted in the amendment text are as follows:

• Overall procedure of DL subcarrier to resource unit mapping supporting outer permutation with N1 and N2,                                second-level permutation, and inner permutation for distributed resource

• Outer and second-level permutation rule and related sequence generation method

• Text change from “the minimum unit for forming the LDRU is equal to one subcarrier” to “the minimum unit for forming the LDRU is equal to a pair of subcarriers”

• Text change from “LDRU” to “DRU” and from “LLRU” to “CRU” to avoid uncertainties in the current SDD text

• Inner permutation rule and related sequence generation method in the unit of a pair of subcarriers for distributed subchannelization

• Description of pilot subcarrier mapping into the physical subcarrier indices

• Details on permutation sequence generation methods can be found in [5]

# 4 References

[1] IEEE 802.16m-07/002r4, “802.16m System Requirements”

[2] IEEE 802.16m-08/003r5, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D7, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-08/1468, “Details on Permutations for DL and UL PHY structures”

# 5 Proposed Amendment Text on 15.3.5 Downlink Physical Structure

-----------------------------------------------------------Start of the Text---------------------------------------------------------

## 15 Advanced Air Interface

## 15.3 Physical Layer

## Downlink Physical Structure

Each downlink subframe is divided into a number of frequency partitions, where each partition consists of a set of physical resource units across the total number of OFDMA symbols available in the subframe. Each frequency partition includes contiguous (localized) and/or non-contiguous (distributed) physical resource units. Each frequency partition is used for different purposes such as fractional frequency reuse (FFR) or multicast and broadcast services (MBS). Figure 15.3.5.1 illustrates the downlink physical structure in the example of two frequency partitions with frequency partition 2 including both localized and distributed resource allocations.

Figure 15.3.5.1 Hierarchical representation of the downlink physical structure

### 15.3.5.1 Physical and Logical Resource Unit

A physical resource unit (PRU) is the basic physical unit for resource allocation that comprises Psc consecutive subcarriers by Nsym consecutive OFDMA symbols. Psc is 18 subcarriers and Nsym is 6 OFDMA symbols for type-1 subframes, and Nsym is 7 OFDM symbols for type-2 sub frames. A logical resource unit (LRU) is the basic logical unit for distributed and localized resource allocations. A LRU is Psc∙Nsym subcarriers for type-1 subframes and type-2 subframes. The LRU includes the pilots in 15.3.5.3 that are used in a PRU. The effective number of subcarriers in an LRU depends on the number of allocated pilots.

### 15.3.5.1.1 Distributed resource unit

The distributed resource unit (DRU) contains a group of subcarriers which are spread across the distributed resource allocations within a frequency partition. The size of the DRU equals the size of PRU, i.e., Psc subcarriers by Nsym OFDMA symbols. The minimum unit for forming the DRU is equal to a pair of subcarriers as defined in 15.3.5.2.3.1.

### 15.3.5.1.2 Localized resource unit

The localized resource unit, also known as contiguous resource unit (CRU) contains a group of subcarriers which are contiguous across the localized resource allocations. The size of the CRU equals the size of the PRU, i.e., Psc subcarriers by Nsym OFDMA symbols.

### 15.3.5.2 Subchannelization and Resource mapping

### 15.3.5.2.1 Basic Symbol Structure

The subcarriers of an OFDMA symbol are partitioned into Ng,left left guard subcarriers, Ng,right right guard subcarriers, and Nused used subcarriers. The DC subcarrier is not loaded. The Nused subcarriers are divided into PRUs. Each PRU contains pilot and data subcarriers. The number of used pilot and data subcarriers depends on MIMO mode, rank and number of multiplexed MS as well as the type of the subframe, i.e., type-1 or type-2.

Table 15.3.5.1 summarizes the parameters of the basic symbol structure. The number of available PRUs depends on the channel bandwidth and the usage of guard band. The PRU-Subband denotes the N1 contiguous PRUs, where N1 = 4 is used for the outer permutation in 15.3.5.2.2.

Table 15.3.5.1 Parameters for basic symbol structure

| The nominal channel bandwidth, |  | 20MHz | 10MHz | 5MHz | 7MHz | 8.75MHz |
| --- | --- | --- | --- | --- | --- | --- |
| FFT size |  | 2048 | 1024 | 512 | 1024 | 1024 |
| Number of used subcarriers Nused (excluding DC subcarrier) |  | 1728 | 864 | 432 | [TBD] | [TBD] |
| Guard subcarriers | Left (Ng,left) | 160 | 80 | 40 | [TBD] | [TBD] |
|  | Right (Ng,right) | 159 | 79 | 39 | [TBD] | [TBD] |
| Number of PRUs NPRU (excluding guard carriers) |  | 96 | 48 | 24 | [TBD] | [TBD] |
| Number of PRU-Subbands NP-Sub |  | 24 | 12 | 6 | [TBD] | [TBD] |

### 15.3.5.2.2 Downlink subcarrier to resource unit mapping

The DL subcarrier to resource unit mapping process is defined in the Figure 15.3.5.2 and illustrated in the Figure 15.3.5.3 as follows:

1. The outer permutation is applied to the PRUs in the units of N1 and N2 contiguous PRUs, where N1=4 and N2 = 1.

2. Distribute the outer-permuted PRUs into frequency partitions.

3. Each frequency partition is divided into localized (CRU) and/or distributed (DRU) resources. Sector specific second-level permutation can be supported and direct mapping of the resources for localized resources can be also supported. The sizes of the distributed/localized resources are flexibly configured per sector.

4. The localized and distributed resources are further mapped into localized LRUs and distributed LRUs by direct mapping of CRU and by inner permutation on DRUs, respectively.

Figure 15.3.5.2 High-level illustration of the downlink subcarrier to resource unit mapping

Figure 15.3.5.3 illustrates an example of downlink subcarrier to resource unit mapping into 4 frequency partitions with NPRU = 48, N1 = 4, and N2 = 1.

Figure 15.3.5.3 Example of DL subcarrier to resource unit mapping

### 15.3.5.2.2.1 Outer permutation

The goal of the outer permutation is

• To provide two different physical allocation units - N1 and N2 contiguous PRUs such that localized and distributed resources can be flexibly assigned from two different allocation units and

• To provide cell-common permutation such that frequency partitions are common among cells.

The size of PRU-subband N1 is equal to four, and the size of N2 is equal to one. In order to divide the available PRUs into the frequency partitions consisting of N1 and/or N2 PRUs, the outer permutation has two successive permutations in the units of N1 and N2 PRUs, respectively, as follows:

For given NPRU PRUs depending on the system bandwidth in Table 15.3.5.1, the outer permutation P1 permutes NP-Sub (= NPRU/N1) PRU-subbands to allocate the NP-Sub PRU-subbands into frequency partitions. The cell-common permutation P1 is given by

P1[i] = {NPSub_Dist · i + SPSub_Offset + floor(i / WPSub_Length)} mod NP-Sub,  i = 0,1,…, NP-Sub-1

where

NPSub_Dist : Distance between two adjacent PRU-subband indices; NPSub_Dist  is given by NPSub_Dist = floor(NP-Sub / NN1) to distribute NN1 PRU-subbands evenly across the available frequencies, where NN1 is the number of PRU-subbands to be assigned in terms of N1 allocation unit.

SPSub_Offset : Initial PRU-Subband index offset for the outer permutation P1; SPSub_Offset  in the unit of PRU-subband is a pre-defined value and set to be the same among cells for cell-common frequency partitions.

WPSub_Length = NP-Sub / GCD(NP-Sub, NPSub_Dist), where GCD(x, y) is the greatest common divisor of x and y

After performing outer permutation P1, the permuted NN1 PRU-subbands numbered from P1[0] to P1[NN1 -1] are reserved for allocation with N1 granularity. According to the system configuration information, NN1 is the sum of the number of PRU-subbands required in each frequency partition, i.e., NN1 = NN1,1 + NN1,2 + …+ NN1,Npart , where Npart denotes the number of frequency partitions. Then, the permuted PRU-subband indices are allocated into each frequency partition sequentially such as P1[0] ~ P1[NN1,1-1] for the first frequency partition and P1[NN1 - NN1,Npart] ~ P1[NN1 -1] for the last frequency partition. If the m-th frequency partition does not require PRU-subbands, then, NN1,m is set to zero. In Figure 15.3.5.3, for example, NN1,1 = NN1 = 6 and NN1,2 = NN1,3 = NN1,4 = 0.

After reserving NN1 PRU-subbands for allocation with N1 granularity, the remaining PRU-subbands numbered from P1[NN1] to P1[NP-Sub -1] are renumbered in the unit of PRU from 0 to NN2 -1, where NN2 denotes the number of PRUs obtained from the remaining (NP-Sub- NN1) PRU-subbands. The outer permutation P2 allocates the renumbered NN2 PRUs into frequency partitions. The cell-common permutation P2 is given by

P2[i] = {NPRU_Dist ·i + SPRU_Offset + floor(i / WPRU_Length)} mod NN2,  i = 0,1,…, NN2 -1

where

NPRU_Dist : Distance between two adjacent PRU indices; NPRU_Dist  is given by NPRU_Dist = Npart for each frequency partition to have distributed PRUs evenly across the available frequencies.

SPRU_Offset : Initial PRU index offset for the outer permutation P2; SPRU_Offset  in the unit of PRU is a pre-defined value and set to be the same among cells for cell-common frequency partitions and varies with frame.

WPRU_Length = NN2 / GCD(NN2, NPRU_Dist), where GCD(x, y) is the greatest common divisor of x and y

According to the system configuration information, the number of PRUs NN2 is the sum of the number of PRUs required in each frequency partition, i.e., NN2 = NN2,1 + NN2,2 + …+ NN2,Npart. Then, the permuted PRU indices are allocated into each frequency partition sequentially such as P2[0] ~ P1[NN2,1-1] for the first frequency partition and P2[NN2 - NN2,Npart] ~ P2[NN2 -1] for the last frequency partition. If the m-th frequency partition consists of only PRU-subbands without N2 granularity allocation, then, NN2,m is set to zero. In Figure 15.3.5.3, for example, NN2,1 = 0 and NN2,2 = NN2,3 = NN2,4 = 8.

### 15.3.5.2.2.2 Second-level permutation

The second-level permutation is performed within each frequency partition in the unit of PRU for permuting the PRUs to be used as the distributed subchannelization.

Frequency partition m includes both NN1,m PRU-subbands and NN2,m PRUs. First, the outer-permuted PRUs are allocated into the DRUs and CRUs. The CRUs in the localized resource group are numbered in order of N1  and N2 allocation units. For the distributed resource group consisting of NDRU PRUs, the PRUs are numbered from 0 to NDRU -1 in order of N1 and N2 allocation units. The second-level permutation P3 is used for renumbering NDRU PRUs as follows:

P3[i] = {NDRU_Dist ·i + SDRU_Offset + floor(i / WDRU_Length)} mod NDRU,  i = 0,1,…, NDRU-1

where

NDRU_Dist : Distance between two adjacent indices; NDRU_Dist is a pre-defined cell-specific information

SDRU_Offset : Initial PRU index offset for second-level permutation P3; SDRU_Offset in the unit of PRU is a predefined cell-specific information.

WDRU_Length = NDRU / GCD(NDRU, NDRU_Dist), where GCD(x, y) is the greatest common divisor of x and y. If

NDRU_Dist is greater than NDRU, then, NDRU_Dist = NDRU_Dist mod NDRU

### 15.3.5.2.3 Subchannelization for DL distributed resource

### 15.3.5.2.3.1 Inner permutation for DL distributed resource

The inner permutation for the DL distributed subchannelization within a frequency partition is used to spread the subcarrriers of the DRU across the whole distributed resource group. The granularity of the inner permutation is equal to a pair of subcarriers. Let NDRU be the number of PRUs within the distributed resource and Npair,PRU denote the number of pairs in each OFDMA symbol within a PRU, i.e., Npair,PRU = (Psc - nl)/2, where nl denotes the number of pilot tones in the l-th OFDMA symbol within a PRU. The procedure of the inner permutation P4 for DL distributed subchannelization is as follows:

For each l-th OFDMA symbol in the subframe,

1. Allocate the nl pilots within each PRU as described in 15.3.5.3.

2. Renumber the remaining NDRU ∙ (Psc - nl) data subcarriers in order, from 0 to NDRU ∙ (Psc - nl) -1. The contiguous logically renumbered subcarriers are grouped into NDRU ∙ Npair,PRU pairs and renumbered from 0 to Npair -1, where Npair denotes the total number of pairs through the NDRU PRUs.

00000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000he inner permutation  uses elizationrequency partitions consisting of 00000000000000000000000000000000000000000000000000000000003. Apply the inner permutation P4 to form a total of NDRU distributed LRUs. The inner permutation P4    is  0000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000used to permute and allocate Npair = NDRU  ∙ Npair,PRU pairs in each OFDMA symbol into the NDRU distributed LRUs. The k-th pair of s-th distributed LRU (subchannel) in l-th symbol is mapped into

npair (s, k, l) = nGR,pair·NDRU + PInner[s ; nGR,pair+l], l = 0, 1, …, Nsym,TTI

where npair (s, k, l) denotes pair index of the k-th pair of s-th distributed LRU (subchannel) in l-th symbol, nGR,pair= (5k+7s) mod Npair,PRU, Nsym,TTI is the number of OFDMA symbols during the TTI, Pinner[i ; j] denotes the i-th element of the series obtained by rotating the base permutation sequence Pinner[i ; 0] cyclically to the left j times, where

PInner[i, 0] = {NInnerDist ·i + SInnerOffset + floor(i / WInnerLength)} mod NDRU,  i = 0,1,…, NDRU -1

where

NInner_Dist : Distance between two adjacent pair indices; NinnerDist is a pre-defined cell-specific

information.

SInnerOffset : Initial index offset for inner permutation Pinner; SinnerOffset is a pre-defined cell-specific

information.

WInnerLength = NDRU / GCD(NDRU, NInnerDist), where GCD(x, y) is the greatest common divisor of x and y.

If Ninner_Dist  is greater than NDRU, then, Ninner_Dist = Ninner_Dist mod NDRU

### 15.3.5.2.4 Subchannelization for DL localized resource

The direct mapping on the CRUs into the localized LRUs is applied without inner permutation.

### 15.3.5.3 Pilot Structure

The transmission of pilot subcarriers in the downlink is necessary for enabling channel estimation, measurements of channel quality indicators such as the SINR, frequency offset estimation, etc. To optimize the system performance in different propagation environments and applications, the Advanced Air Interface (AAIF) supports both common and dedicated pilot structures. The categorization in common and dedicated pilots is done with respect to their usage. The common pilots can be used by all MSs. Dedicated pilots can be used with both localized and distributed allocations. Pilot subcarrriers that can be used only by a group of MSs are a special case of common pilots and are termed shared pilots. The dedicated pilots are associated with a specific resource allocation, can be only used by the MSs allocated to said specific resource allocation, and therefore can be precoded or beamformed in the same way as the data subcarriers of the resource allocation. The pilot structure is defined for up to four transmission (Tx) streams and there is a unified pilot pattern design for common and dedicated pilots. There is equal pilot density per Tx stream, while there is not necessarily equal pilot density per OFDMA symbol of the downlink subframe. Further, there is equal number of pilots for each PRU of a data burst assigned to one MS.

The pilots can be used for channel estimation, measurements (CQI and interference mitigation/cancellation), frequency offset estimation and time offset estimation. Pilot patterns are proposed for efficiency and performance. Pattern A is used for 1 and 2 DL data streams dedicated and common pilot scenarios.

Figure 15.3.5.4 shows the pilot subcarriers for 1 and 2 DL data streams. The pilot subcarrier allocated in the i–th antenna, the l-th OFDMA symbol, and k-th PRU is defined as follows:

Piloti(k,l) = 18k +16∙(l mod 3) mod 24 + i mod 2

For the subframe consisting of 5 OFDMA symbols, the last OFDMA symbol is deleted. For the subframe consisting of 7 OFDMA symbols, the second or fifth OFDMA symbol is added in the end of the subframe.

Figure 15.3.5.4 DL Pilot pattern for 1 and 2 data streams

The pilot structure shown in Figure 15.3.5.5 is used for 4 data streams. When l0{0,1,4,5}, where l0 = l mod 6, the pilot subcarrier allocated in the i–th antenna, the l-th OFDMA symbol, and k-th PRU is defined as follows:

Piloti(k,l) = 18k +12∙{( l0 +floor(i/2)) mod 2}+5∙{(i + floor(l0/4)) mod 2}

For the subframe consisting of 5 OFDMA symbols, the third or fourth OFDMA symbol is deleted. For the subframe consisting of 7 OFDMA symbols, the third or fourth OFDMA symbol is added in the end of the subframe.

Figure 15.3.5.5 DL Pilot pattern 4 data streams

------------------------------------------------------------End of the Text---------------------------------------------------------