| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Amendment Text on Uplink Physical Structure |  |
| Date Submitted | 2008-11-03 |  |
| Source(s) | HanGyu Cho, Jinsoo Choi, Seung-Hyun Kang, Jin Sam Kwak LG Electronic Inc. | Voice: +82-31-450-1857 E-mail: {hgcho, emptylie, sh_kang, samji}@lge.com |
| Re: | C80216m-08_042: Call for Contributions on Project 802.16m Draft Amendment Content Target Topic: Uplink Physical Structure |  |
| Abstract | Proposal of amendment text on Uplink Physical Structure |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m amendment text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Amendment Text on Uplink Physical Structure

HanGyu Cho, Jinsoo Choi, Seung-Hyun Kang, Jin Sam Kwak

LG Electronics

# 1 Introduction

In this contribution, the amendment text on Uplink Physical Structure is proposed in accordance with the IEEE 802.16m System Requirement Document [1] and System Description Document [2]. Any proposed text here is developed such that it can be readily combined with IEEE P802.16Rev2/D7 [3] and other active IEEE 802.16 amendment projects, as appropriate. In addition, this contribution follows the tentative outlines, the style and format guidelines contained in [4].

# 2 Motivation

The section “Uplink physical structure” addresses UL subcarrier to resource unit mapping, pilot structure, and uplink physical structure for legacy support. Key concepts described in the current SDD text on UL physical structure are as follows:

• 18 by Nsym physical resource unit (PRU) as a basic physical unit for resource allocation

• FDM-multiplexing of distributed and localized resources

• Frequency partitions for supporting FFR

• Outer permutation with different permutation granularities N1 and N2

• Inner permutation in the unit of tile for distributed allocation

• DL pilot patterns designed for PRU are used for UL MIMO support up to 4 TX streams.

What this contribution does is to clarify the SDD text, to correct editorial mistakes, and to provide technical details fit for amendment text as much as possible.

# 3 Key proposals

Main technical proposals to be adopted in the amendment text are as follows:

• Overall procedure of UL subcarrier to resource unit mapping supporting outer permutation with N1 and N2,

second-level permutation, and inner permutation for distributed resource

• Inner permutation rule and related sequence generation methods in the unit of tile for distributed resource

• Text change from “LDRU” to “DRU” and from “LLRU” to “CRU” to avoid uncertainties in the current SDD text

• Description of pilot subcarrier mapping into the physical subcarrier indices

• Details on permutation sequence generation methods can be found in [5]

# 4 References

[1] IEEE 802.16m-07/002r4, “802.16m System Requirements”

[2] IEEE 802.16m-08/003r5, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D7, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Oct. 2008.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-08/1468, “Details on Permutations for DL and UL PHY structures”

# 5 Proposed Amendment Text on 15.3.6 Uplink Physical Structure

-----------------------------------------------------------Start of the Text---------------------------------------------------------

## 15 Advanced Air Interface.

## 15.3 Physical Layer

## 15.3.6 Uplink Physical Structure

Each UL subframe is divided into a number of frequency partitions, where each partition consists of a set of physical resource units across the total number of OFDMA symbols available in the subframe. Each frequency partition can include contiguous (localized) and/or non-contiguous (distributed) physical resource units. Each frequency partition can be used for different purposes such as fractional frequency reuse (FFR). Figure 15.3.6.1 illustrates the uplink physical structure in the example of two FFR groups with FFR group 2 including both localized and distributed resource allocations.

Figure 15.3.6.1 Hierarchical representation of the uplink physical structure

### 15.3.6.1 Physical and Logical Resource Unit

A physical resource unit (PRU) is the basic physical unit for resource allocation that comprises Psc consecutive subcarriers by Nsym consecutive OFDMA symbols. Psc is 18 subcarriers and Nsym is 6 OFDMA symbols for type-1 subframes, and Nsym is 7 OFDMA symbols for type-2 sub frames. A logical resource unit (LRU) is the basic logical unit for distributed and localized resource allocations and its size is Psc∙Nsym subcarriers for data transmission. For control channel/message transmission, the size of LRU should be the same as that of data transmission and multiple users are allowed to share one control LRU. The LRU includes the number of pilots in a PRU. The effective number of data subcarriers in an LRU depends on the number of allocated pilots.

### 15.3.6.1.1 Distributed resource unit

The distributed resource unit (DRU) contains a group of subcarriers which are spread across the distributed resource allocations. The size of the DRU equals the size of the LRU for distributed allocations, i.e., Psc subcarriers by Nsym OFDMA symbols. The minimum unit for forming the DRU is a tile. The UL tile size is 6 consecutive subcarriers by Nsym consecutive OFDMA symbols.

### 15.3.6.1.2 Localized resource unit

The localized resource unit, also known as contiguous resource unit (CRU) contains a group of subcarriers which are contiguous across the localized resource allocations. The size of the CRU equals the size of the LRU for localized allocations, i.e., Psc subcarriers by Nsym OFDMA symbols.

### 15.3.6.2 Subchannelization and Resource mapping

### 15.3.6.2.1 Basic Symbol Structure

The subcarriers of an OFDMA symbol are partitioned into Ng,left left guard subcarriers, Ng,right right guard subcarriers, and Nused used subcarriers. The DC subcarrier is not loaded. The Nused subcarriers are divided into PRUs. Each PRU contains pilot and data subcarriers. The number of used pilot and data subcarriers depends on MIMO mode, rank and number of multiplexed MS as well as the type of the subframe, i.e., type-1 or type-2.

Table 15.3.6.1 summarizes the parameters of the basic symbol structure. The number of available PRUs depends on the channel bandwidth and the usage of guard band. The PRU-Subband denotes the N1 contiguous PRUs, where N1 = 4 is used for the outer permutation in 15.3.6.2.2.

Table 15.3.6.1 Parameters for basic symbol structure

| The nominal channel bandwidth, |  | 20MHz | 10MHz | 5MHz | 7MHz | 8.75MHz |
| --- | --- | --- | --- | --- | --- | --- |
| FFT size |  | 2048 | 1024 | 512 | 1024 | 1024 |
| Number of used subcarriers Nused (Excluding DC subcarrier) |  | 1728 | 864 | 432 | [TBD] | [TBD] |
| Guard subcarriers | Left (Ng,left) | 160 | 80 | 40 | [TBD] | [TBD] |
|  | Right (Ng,right) | 159 | 79 | 39 | [TBD] | [TBD] |
| Number of PRUs NPRU (excluding guard carriers) |  | 96 | 48 | 24 | [TBD] | [TBD] |
| Number of PRU-Subbands NP-Sub |  | 24 | 12 | 6 | [TBD] | [TBD] |

### 15.3.6.2.2 Uplink subcarrier to resource unit mapping

The main features of resource mapping include:

1. Support of localized resource unit (LLRU) and distributed resource unit (LDRU) in an FDM manner.

2. DRUs comprise multiple tiles which are spread across the distributed resource allocations.

3. FFR can be applied in UL.

Based on the main design concepts above, the UL subcarrier to resource unit mapping process is defined in the Figure 15.3.6.2:

1. The outer permutation is applied to the NPRU PRUs in the units of N1 and N2 contiguous PRUs, where N1 = 4 and N2 = 1.

2. Distribute the outer-permuted PRUs into frequency partitions.

3. Each frequency partition is divided into localized (CRU) and/or distributed (DRU) resources. Sector specific second-level permutation can be supported and direct mapping of the resources can be supported for localized resource. The sizes of the distributed/localized resources are flexibly configured per sector.

4. The localized and distributed resources are further mapped into localized LRUs. For the CRU resources, the mapping is direct. For the DRU resources, a tile permutation or hopping permutation is carried out for permuting or hopping the tiles of the distributed groups.

Figure 15.3.6.2 High-level illustration of the uplink subcarrier to resource unit mapping

### 15.3.6.2.2.1 Outer permutation

The goal of the outer permutation is

• To provide two different physical allocation units - N1 and N2 contiguous PRUs such that localized and

distributed resources can be flexibly assigned from two different allocation units and

• To provide cell-common permutation such that frequency partitions can be aligned among cells.

The size of PRU-subband N1 is equal to four, and the size of N2 is equal to one. In order to divide the available PRUs into the frequency partitions consisting of N1 and/or N2 PRUs, the outer permutation has two successive permutations in the units of N1 and N2 PRUs, respectively, as follows:

For given NPRU PRUs depending on the system bandwidth in Table 15.3.6.1, the outer permutation P1 permutes NP-Sub (= NPRU/N1) PRU-subbands to allocate the NP-Sub PRU-subbands into frequency partitions. The cell-common permutation P1 is given by

P1[i] = {NPSub_Dist · i + SPSub_Offset + floor(i / WPSub_Length)} mod NP-Sub,  i = 0,1,…, NP-Sub-1

where

NPSub_Dist : Distance between two adjacent PRU-subband indices; NPSub_Dist  is given by NPSub_Dist = floor(NP-Sub / NN1) to distribute NN1 PRU-subbands evenly across the available frequencies, where NN1 is the number of PRU-subbands to be assigned in terms of N1 allocation unit.

SPSub_Offset : Initial PRU-Subband index offset for the outer permutation P1; SPSub_Offset  in the unit of PRU-subband is a pre-defined value and set to be the same among cells for cell-common frequency partitions.

WPSub_Length = NP-Sub / GCD(NP-Sub, NPSub_Dist), where GCD(x, y) is the greatest common divisor of x and y

After performing outer permutation P1, the permuted NN1 PRU-subbands numbered from P1[0] to P1[NN1 -1] are reserved for allocation with N1 granularity. According to the system configuration information, NN1 is the sum of the number of PRU-subbands required in each frequency partition, i.e., NN1 = NN1,1 + NN1,2 + …+ NN1,Npart , where Npart denotes the number of frequency partitions. Then, the permuted PRU-subband indices are allocated into each frequency partition sequentially such as P1[0] ~ P1[NN1,1-1] for the first frequency partition and P1[NN1 - NN1,Npart] ~ P1[NN1 -1] for the last frequency partition. If the m-th frequency partition does not require PRU-subbands, then, NN1,m is set to zero.

After reserving NN1 PRU-subbands for allocation with N1 granularity, the remaining PRU subbands numbered from P1[NN1] to P1[NP-Sub -1] are renumbered in the unit of a PRU from 0 to NN2 -1, where NN2 denotes the number of PRUs obtained from the remaining (NP-Sub- NN1) PRU-subbands. The outer permutation P2 allocates the renumbered NN2 PRUs into frequency partitions. The cell-common permutation P2 is given by

P2[i] = {NPRU_Dist ·i + SPRU_Offset + floor(i / WPRU_Length)} mod NN2,  i = 0,1,…, NN2 -1

where

NPRU_Dist : Distance between two adjacent PRU indices; NPRU_Dist  is given by NPRU_Dist = Npart for each frequency partition to have evenly distributed PRUs across the available frequencies.

SPRU_Offset : Initial PRU index offset for outer permutation P2; SPRU_Offset  in the unit of PRU is a pre-defined value and set to be the same among the cells for cell-common frequency partitions and varies with frame.

WPRU_Length = NN2 / GCD(NN2, NPRU_Dist), where GCD(x, y) is the greatest common divisor of x and y

According to the system configuration information, the number of PRUs NN2 is the sum of the number of PRUs required in each frequency partition, i.e., NN2 = NN2,1 + NN2,2 + …+ NN2,Npart. Then, the permuted PRU indices are allocated into each frequency partition sequentially such as P2[0] ~ P1[NN2,1-1] for the first frequency partition and P2[NN2 - NN2,Npart] ~ P2[NN2 -1] for the last frequency partition. If the m-th frequency partition consists of only PRU-subbands without N2 granularity allocation, then, NN2,m is set to zero.

### 15.3.6.2.2.2 Second-level permutation

The second-level permutation is performed within each frequency partition in the unit of PRU for permuting the PRUs to be used for distributed subchannelization. Frequency partition m includes both NN1,m PRU-subbands and NN2,m PRUs. First, the outer-permuted PRUs are allocated into the DRUs and CRUs. The CRUs in the localized resource group is numbered in order of N1 and N2 allocation units. For the distributed resource group consisting of NDRU PRUs, the PRUs are numbered from 0 to NDRU -1 in order of N1 and N2 allocation units. The second-level permutation P3 is used for renumbering NDRU PRUs as follows:

P3[i] = {NDRU_Dist ·i + SDRU_Offset + floor(i / WDRU_Length)} mod NDRU,  i = 0,1,…, NDRU-1

where

NDRU_Dist : Distance between two adjacent indices; NDRU_Dist is a pre-defined cell-specific information

SDRU_Offset : Initial PRU index offset for second-level permutation P3; SDRU_Offset in the unit of PRU is a predefined cell-specific information.

WDRU_Length = NDRU / GCD(NDRU, NDRU_Dist), where GCD(x, y) is the greatest common divisor of x and y. If

NDRU_Dist  is greater than NDRU, then, NDRU_Dist = NDRU_Dist mod NDRU

### 15.3.6.2.3 Subchannelization for UL distributed resource

### 15.3.6.2.3.1 Inner permutation for UL distributed resource

The inner permutation for the UL distributed subchannelization within a frequency partition is used to spread the tiles of the DRU across the whole distributed resource group. The granularity of the inner permutation is equal to a tile. Using the UL tile size of 6 consecutive subcarriers by Nsym consecutive OFDMA symbols, the number of tiles in a PRU is three, i.e., NPRU,tile = 3. Let NDRU be the number of PRUs within the distributed resource. The inner permutation procedure for UL distributed subchannelization is as follows:

- Renumber the total number of tiles Ntiles = NDRU ∙ NPRU,tile = 3 NDRU in order, from 0 to Ntiles -1 tiles.

- Apply the inner permutation  0000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000     reseries obtained by rotating tion is equal to a tile.00000000000000000000000000000000000000P4 to form a total of NDRU distributed LRUs. The inner permutation P4 is  0000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000used to permute and allocate Ntiles =NDRU ∙ NPRU,tile = 3∙NDRU pairs in each subframe into the NDRU distributed LRUs. The k-th tile of s-th distributed LRU (subchannel) in l-th subframe is mapped into

ntile (s, k, l) = nGR,tile·NDRU + Pinner[s; nGR,pair+l], l = 0, 1, …, NSF,TTI -1

where ntile (s, k, l) denotes tile index of the k-th tile of s-th distributed LRU (subchannel) in l-th subframe, nGR,tile = (5k+7s) mod NPRU,tile = (5k+7s) mod 3, NSF,TTI is the number of subframes during the TTI, and Pinner[i; j] denotes the i-th element of the series obtained by rotating the basic permutation sequence Pinner[i, 0] cyclically to the left j times, where

Pinner[i, 0] = {NinnerDist ·i + SinnerOffset + floor(i / WinnerLength)} mod NDRU,  i = 0,1,…, NDRU-1

where

NInner_Dist : Distance between two adjacent pair indices; NinnerDist is a pre-defined cell-specific information.

SInnerOffset : Initial index offset for inner permutation Pinner; SinnerOffset is a pre-defined cell-specific

information.

WInnerLength = NDRU / GCD(NDRU, NInnerDist), where GCD(x, y) is the greatest common divisor of x and y.

If Ninner_Dist  is greater than NDRU, then, Ninner_Dist = Ninner_Dist mod NDRU

### 15.3.6.2.4 Subchannelization for UL localized resource

The localized subchannels contain subcarriers which are contiguous in frequency. There is no inner permutation defined for the UL localized resource allocations. The CRU is directly mapped to localized LRU within each frequency partition. Precoding and/or boosting applied to the data subcarriers will also be applied to the pilot subcarriers.

### 15.3.6.3 Pilot Structure

The transmission of pilot subcarriers in the uplink is necessary for enabling channel estimation, measurement of channel quality indicators such as SINR, frequency offset and timing offset estimation, etc. The uplink pilots are dedicated to localized and distributed resource units and are precoded using the same precoding as the data subcarriers of the resource allocation. The pilot structure is defined for up to 4 Tx streams with orthogonal patterns.

The pilot pattern may support variable pilot boosting. When pilots are boosted, each data subcarrier should have the same Tx power across all OFDM symbols in a resource block. The boosting values are TBD.

The UL pilot pattern for LLRU is used up to 4 data streams.

Figure 15.3.6.3 shows the pilot subcarriers for 1 and 2 UL data streams. The pilot subcarrier allocated in the i–th antenna, the l-th OFDMA symbol, and k-th PRU is defined as follows:

Piloti(k,l) = 18k +16∙(l mod 3) mod 24 + i mod 2

For the subframe consisting of 5 OFDMA symbols, the last OFDMA symbol is deleted. For the subframe consisting of 7 OFDMA symbols, the second or fifth OFDMA symbol is added in the end of the subframe.

Figure 15.3.6.3 UL Pilot pattern for 1 and 2 data streams

The pilot structure shown in Figure 15.3.6.4 is used for 4 data streams. When l0{0,1,4,5}, where l0 = l mod 6, the pilot subcarrier allocated in the i–th antenna, the l-th OFDMA symbol, and k-th PRU is defined as follows:

Piloti(k,l) = 18k +12∙{( l0 +floor(i/2)) mod 2}+5∙{(i + floor(l0/4)) mod 2}

For the subframe consisting of 5 OFDMA symbols, the third or fourth OFDMA symbol is deleted. For the subframe consisting of 7 OFDMA symbols, the third or fourth OFDMA symbol is added in the end of the subframe.

Figure 15.3.6.4 DL Pilot pattern 4 data streams

For UL LDRU, the pilot structure is used up to 2 data stream. It is shown in Figure 15.3.6.5. When l0{0,1,4,5}, where l0 = l mod 6, the pilot subcarrier allocated in the i–th antenna, the l-th OFDMA symbol, and s-th tile is defined as follows:

Piloti(k,l) = 6s +4∙( l0 mod 2)+ i mod 2

Figure 15.3.6.5 UL pilot pattern for 6x6 tile

For the subframe consisting of 5 OFDMA symbols, the third or fourth OFDMA symbol is deleted. For the subframe consisting of 7 OFDMA symbols, the third or fourth OFDMA symbol is added in the end of the subframe.

### 15.3.6.4 Uplink Physical Structure for WirelessMAN-OFDMA Support

The AAIF uplink physical structure supports the WirelessMAN-OFDMA system in the manner of both FDM and TDM. Depending on the subchannelization of WirelessMAN-OFDMA, the multiplexing types and rules in UL are defined as

- FDM or TDM when the WirelessMAN-OFDMA system operates in AMC mode

When the WirelessMAN-OFDMA system operates in AMC mode, the UL symbol structure, UL subchannelization rule, and UL pilot structure in 15.3.6.1, 15.3.6.2, and 15.3.6.3, respectively, are used for FDM/TDM-based WirelessMAN-OFDMA support. For FDM-based WirelessMAN-OFDMA support, the WirelessMAN-OFDMA and AAIF are multiplexed in UL based on the UL PRU size in 15.3.6.1.

- FDM or TDM when the WirelessMAN-OFDMA system operates in PUSC mode

When the WirelessMAN-OFDMA system operates in PUSC mode, the UL symbol structure, UL subchannelization rule, and UL pilot structure in 15.3.6.1, 15.3.6.2, and 15.3.6.3, respectively, are used for TDM-based WirelessMAN-OFDMA support. For FDM-based WirelessMAN-OFDMA support, the UL symbol structure and subchannelization follow the AAIF-PUSC described in 15.3.6.4.1 and 15.3.6.4.2.

### 15.3.6.4.1 Distributed resource unit for AAIF-PUSC

The AAIF-PUSC tile for consisting of a LDRU in the AAIF-PUSC is two consecutive tiles in WirelessMAN-OFDMA UL PUSC. A LDRU contains six AAIF-PUSC tiles, in which each tile size is 4 subcarriers by Nsym, where Nsym depends on the subframe type. Figure 15.3.6.6 shows the AAIF-PUSC tile structure when a subframe has 6 symbols.

Figure 15.3.6.6 Tile structure in AAIF-PUSC

### 15.3.6.4.2 Subchannelization for Advanced Air Interface PUSC

A subchannelization for AAIF-PUSC is identical to WirelessMAN-OFDMA uplink PUSC. For a given system bandwidth, total usable subcarriers are allocated to form tiles (four contiguous subcarriers) and every tiles are permuted according to permutation defined in WirelessMAN-OFDMA uplink PUSC. Once subchannelization is done, every subchannel is assigned to either the WirelessMAN-OFDMA system or the AAIF system.

------------------------------------------------------------End of the Text---------------------------------------------------------