| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Pilot structure of tile for UL legacy PUSC support |  |
| Date Submitted | 2008-11-09 |  |
| Source(s) | Dongsheng Yu, Mo-Han Fong, Jun Yuan, Sophie Vrzic, Robert Novak, Hosein Nikopourdeilami, Kathiravetpillai Sivanesan Nortel | Email: dongshengyu@nortel.com mhfong@nortel.com |
| Re: | TGm SDD : Other(UL PHY) In response to IEEE 802.16m-08/040 “Call for Contributions and Comments on Project 802.16m System Description Document (SDD)” for Session 58 |  |
| Abstract | This contribution proposes SDD text for UL pilot structure for FDM PUSC legacy support |  |
| Purpose | Discuss and adopt the text proposed in this document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed SDD Text for UL Pilot for legacy PUSC support

# Dongsheng Yu, Mo-Han Fong, Jun Yuan, Sophie Vrzic, Robert Novak, Hosein Nikopourdeilami, Kathiravetpillai Sivanesan

# Nortel Networks

# 1. Introduction

This contribution is to propose pilot structure of UL tile for legacy PUSC support. Pilot structure is proposed for up to 1, 2 and 4 streams.

# Text Proposal modification to SDD

------------------------------------ Start of text proposal------------------------------------

[Add the following figure at the end of section 11.6.4.1]

Figure xx Tile and pilot structure for 1, 2 and 4 streams

------------------------------------End of text proposal-------------------------------------