| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Remedy for SDD Comment #177 |  |
| Date Submitted | 2008-11-09 |  |
| Source(s) | Sungho Moon, Jin Sam Kwak LG Electronics Lei Wang NextWave Wireless Inc. | Voice : +82-31-450-1852 E-mail : msungho@lge.com E-mail : lwang@nextwave.com |
| Re: | Re: Reply for comment #177 |  |
| Abstract | The contribution proposes a remedy harmonized with the commenter regarding SDD comment #177. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Remedy for SDD Comment #177

Sungho Moon, Jin Sam Kwak

LG Electronics,

Lei Wang

NextWave Wireless Inc.

============================== Start of Proposed Text =================================

[Adopt the following modifications in section 11.4.1.2.]

#### 11.4.1.2 Frame Structure for CP=1/16 Tu

For nominal channel bandwidths of 5, 10, and 20 MHz, an IEEE 802.16m frame for a CP of 1/16 Tu shall have five type-1 subframes and three type-2 subframes.

Figure 1 illustrates an example of TDD and FDD frame structure with a CP of 1/16 Tu. Assuming OFDMA symbol duration of 97.143 µs and a CP length of 1/16 Tu, the length of type-1 and type-2 subframes are 0.583 ms and 0.680 ms, respectively. Other numerologies may result in different number of subframes per frame and symbols within the subframes.

Figure 1 TDD and FDD Frame Structure with a CP of 1/16 Tu (DL to UL ratio of 5:3)

============================== End of Proposed Text =================================