| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on Draft of UMAC RG Harmonized SDD Text Proposal for ARQ Topics |  |
| Date Submitted | 2008-11-12 |  |
| Source(s) | Youngbin Chang, Anil Agiwal, Baowei Ji Samsung Electronics Xiangying Yang, Yuan Zhu, Hujun Yin, Jie Hui Intel Corporation Yih-Shen Chen, I-Kang Fu, Kelvin Chou, Paul Cheng MediaTek Inc. Sean McBeath, JueJun Liu Huawei Technologies Co.,Ltd. Ming-Hung Tao, Mamadou Kone ITRI | yb.chang@samsung.com, Xiangying.yang@intel.com yihshen.chen@mediatek.com smcbeath@huawei.com mhtao@itri.org.tw |
| Re: | Call for comments on Draft of UMAC RG Harmonized SDD Text (C80216m-08_1409r2) |  |
| Abstract | Comments on Draft of UMAC RG Harmonized SDD Text |  |
| Purpose | For review and discussion in the Project 802.16m UMAC Rapporteur Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on Draft of UMAC RG Harmonized SDD Text Proposal

for ARQ Topics

Youngbin Chang, Anil Agiwal, Baowei Ji

Samsung Electronics

Xiangying Yang, Yuan Zhu, Hujun Yin, Jie Hui

Intel Corporation

Yih-Shen Chen, I-Kang Fu, Kelvin Chou, Paul Cheng

MediaTek Inc.

Sean McBeath, JueJun Liu

Huawei Technologies Co.,Ltd.

Ming-Hung Tao, Mamadou Kone

ITRI

# Motivation of this proposal:

Companies interested in ARQ-HARQ interworking uses different assumption of 16m ARQ and HARQ operation. Therefore, it is premature to discuss ARQ and HARQ interworking. We would prefer to leave all of ARQ-HARQ interworking items in SDD ‘For Further study’

# Text Proposal

================= Start of Proposed Text ========================

# 10. Medium Access Control Sub-Layer

# 10.2.3 HARQ and ARQ Interactions

HARQ and ARQ interaction mechanism is FFS.

When both ARQ and HARQ are applied for a flow, HARQ and ARQ interactions described here is applied to the corresponding flow.

If the HARQ entity in the transmitter determines that the HARQ process was terminated with an unsuccessful outcome, the HARQ entity in the transmitter informs the ARQ entity in the transmitter about the failure of the HARQ burst. The ARQ entity in the transmitter can then initiate retransmission and re-segmentation of the appropriate ARQ blocks.

If the HARQ entity in the receiver determines that the HARQ process was terminated with an unsuccessful outcome, the HARQ entity in the receiver informs the ARQ entity in the receiver about the failure of the HARQ process. The ARQ entity in the receiver can then undertake to inform the corresponding ARQ entity in the transmitter of the HARQ process failure. ARQ entity in the receiver can wait for a predetermined time before sending MAC signaling to the ARQ entity in the transmitter.  The ARQ entity in the transmitter can then initiate retransmission and re-segmentation of the appropriate ARQ blocks.

# 10.2.3.1 Local NACK between HARQ and ARQ

A local NACK is sent to the ARQ process in the event that the HARQ process was terminated with an unsuccessful outcome.

# 10.2.3.2 Local ACK between HARQ and ARQ

A local ACK is sent to the ARQ process in the event that the HARQ process was terminated with a successful outcome.

# 10.2.3.3 NACK to ACK Errors

To mitigate for NACK to ACK errors, the ARQ transmitter delays the purging of ARQ blocks acknowledged by the local ACK indication.  This allows the receiver time to detect the error and take the appropriate action to correct it.  The receiver, upon detection of a NACK-to-ACK error event, sends a MAC signalling message to the transmitter.  The MAC signalling message identifies the specific HARQ process which incurred the NACK-to-ACK error, e.g. frame/subframe number and HARQ process ID.  If the transmitter does not receive a MAC NACK-to-ACK error indication before a predetermined time, it purges the ARQ blocks acknowledged by the local ACK indication.

# 10.2.3.4 ACK to NACK Errors

Recovery from ACK to NACK errors is achieved via duplicate ARQ block detection.  Duplicated ARQ blocks are discarded.

# 10.4 ARQ

An ARQ block is generated from one or multiple MAC SDU(s) or MAC SDU fragment(s). ARQ blocks can be variable in size. ARQ blocks are sequentially numbered. The location of this sequence number in the MAC PDU is FFS.

Retransmission of a failed ARQ block can be performed with or without rearrangement.

As described in section 10.2.3, a NACK based ARQ protocol is used in conjunction with the local NACK/ACK indication from the HARQ entity to the ARQ entity

============================== End of Proposed Text ===============