| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Final UL PHY Control Rapporteur Group SDD Text Proposal |  |
| Date Submitted | 2008-11-13 |  |
| Source(s) | Xiaoyi Wang Ron Murias Roshni Srinivasan UL PHY Control Rapporteur Group Chairs | xiaoyi.wang@nsn.com ron@murias.ca roshni.m.srinivasan@intel.com |
| Re: | IEEE 802.16m-08/038, TGm Closing Report-Session #57 and IEEE 802.16m-08/041, Rapporteur Group Chair Guidance for the interval between Session #57 and Session #58, and at Session #58. |  |
| Abstract | Contribution to TGm SDD to replace current text in section 11.9 (except E-MBS feedback) on the UL Control Structure. |  |
| Purpose | Discussion and adoption of the text proposed in this document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Final UL PHY Control Rapporteur Group SDD Text Proposal

## UL Control Structure

Details of the UL control structure are described in the following sections.

### UL Control Information Classification

The UL control channels carry multiple types of control information to support air interface procedures. Information carried in the control channels is classified as follows.

<Editors’ Notes: Text included in this section depends on SDD text being developed by other Rapporteur Groups (MIMO, HARQ).>

#### Channel quality feedback

Channel quality feedback provides information about channel conditions as seen by the MS. This information is used by the BS for link adaptation, resource allocation, power control etc. Channel quality measurement includes narrowband and wideband measurements. CQI feedback overhead reduction is supported through differential feedback or other compression techniques. Examples of CQI include Physical CINR, Effective CINR, band selection, etc. Channel sounding can also be used to measure uplink channel quality.

#### MIMO feedback

MIMO feedback provides wideband and/or narrowband spatial characteristics of the channel that are required for MIMO operation. The MIMO mode, precoder matrix index, rank adaptation information, channel covariance matrix elements, power loading factor, eigenvectors and channel sounding are examples of MIMO feedback information.

#### HARQ feedback

HARQ feedback (ACK/NACK) is used to acknowledge DL transmissions. Multiple codewords in MIMO transmission can be acknowledged in a single ACK/NACK transmission.

#### Synchronization

Uplink synchronization signals are needed to acquire uplink synchronization during initial access or handover and also to periodically maintain synchronization. Reference signals for measuring and adjusting the uplink timing offset are used for these purposes.

#### Bandwidth request

Bandwidth requests are used to provide information about the needed uplink bandwidth to the BS. Bandwidth requests are transmitted through indicators or messages. A bandwidth request indicator notifies the BS of a UL grant request by the MS sending the indicator. Bandwidth request messages can include information about the status of queued traffic at the MS such as buffer size and quality of service, including QoS identifiers.

#### E-MBS feedback

<Editors’ Notes : This section is a placeholder for text to be developed based on SDD text that will be added to Section 15 of the SDD (Support for Enhanced Multicast Broadcast Service). >

E-MBS feedback provides information for DL MBS transmission to one or multiple cells. Details are TBD.

### UL Control Channels

<Editors’ Notes: Text included in this section depends on SDD text being developed by other Rapporteur Groups (MIMO, HARQ).>

The UL subframe size for transmission of control information is 6 symbols.

#### UL Fast Feedback Channel

The UL fast feedback channel carries channel quality feedback and MIMO feedback. Transmission of BW REQ indicators on the UL fast feedback channel is FFS.

The mapping of UL fast feedback information into physical channels is described in Section 11.9.2.1.2.

There are two types of UL fast feedback control channels: primary and secondary fast feedback channels. The UL primary fast feedback control channel provides wideband feedback information including channel quality and MIMO feedback. It is used to support robust feedback reports. The UL secondary fast feedback control channel carries narrowband CQI and MIMO feedback information. The secondary fast feedback channel can be used to support CQI reporting at higher code rate and thus more CQI information bits.

##### Multiplexing with other control channels and data channels

The UL fast feedback channel is FDM with other UL control and data channels.

The UL fast feedback channel starts at a pre-determined location, with the size defined in a DL broadcast control message. Fast feedback allocations to an MS can be periodic and the allocations are configurable. For periodic allocations, the specific type of feedback information carried on each fast feedback opportunity can be different.

The UL fast feedback channel carries one or more types of fast feedback information. The use of TDM/FDM or CDM to multiplex fast feedback channels from one or more users is FFS.

##### PHY structure

The secondary fast feedback channel can be allocated in a non-periodic manner based on traffic, channel conditions etc. The number of bits carried in the fast feedback channel can be adaptive.

A UL feedback mini-tile (FMT) is defined as 2 contiguous subcarriers by 6 OFDM symbols.  A UL FMT of 6 contiguous subcarriers by 2 OFDM symbols is FFS. The primary and secondary fast feedback channels are comprised of NFMT distributed FMTs, where NFMT is FFS. One LRU consists of 9 FMTs and can be shared by multiple fast feedback channels.

#### UL HARQ Feedback Channel

This channel is used to carry HARQ feedback information.

##### Multiplexing with other control channels and data channels

The UL HARQ feedback channel starts at a pre-determined offset with respect to the corresponding DL transmission.

The UL HARQ feedback channel is FDM with other control and data channels.

Orthogonal signaling is used to multiplex multiple HARQ feedback channels.

##### PHY structure

The UL HARQ feedback channel is comprised of three distributed UL feedback mini-tiles (FMT), where the UL FMT is defined as 2 contiguous subcarriers by 6 OFDM symbols.  A UL FMT of 6 contiguous subcarriers by 2 OFDM symbols is FFS. One LRU consists of 9 FMTs and can be shared by multiple HARQ feedback channels.

#### UL Sounding Channel

The UL sounding channel is used by an MS to send a sounding signal for MIMO feedback, channel quality feedback and acquiring UL channel information at the BS. The sounding channel occupies specific UL sub-bands or whole UL OFDMA symbol(s).

##### Multiplexing with other control information and data

The BS can configure an MS to transmit an UL sounding signal on specific UL sub-bands or across the whole UL band.  The sounding signal is transmitted over predefined subcarriers within the intended sub-bands.  The periodicity of the sounding signal for each MS is configurable.

The UL sounding channel is FDM and/or TDM with other control and data channels.

##### Multiplexing sounding feedback for multiple users

The BS can configure multiple MSs to transmit UL sounding signals on the corresponding UL sounding channels.  The UL sounding channels from multiple users or multiple antennas per user can be CDM, FDM, or TDM.

Strategies for combating inter-cell-interference may be utilized to improve the sounding performance.

###### Opportunistic UL sounding

Opportunistic UL sounding may be needed for sounding channel quality. The usage of opportunistic UL sounding and the details of the scheme used are FFS.

##### UL Sounding Channel Power Control

Power control for the UL sounding channel is supported to manage the sounding quality. Each MS’s transmit power for UL sounding channel may be controlled separately according to its sounding channel target CINR value. The details of power control scheme are FFS.

##### PHY structure

Sounding from single or multiple antennas and multiple users are supported to provide MIMO channel information for DL transmission. Power allocation, sounding sequence design and mapping to subcarriers is TBD.

#### Ranging Channel

The UL ranging channel is used for UL synchronization. The UL ranging channel can be further classified into ranging channel for non-synchronized mobile stations and synchronized mobiles stations. A random access procedure, which can be contention based or non-contention based is used for ranging. Contention-based random access is used for initial ranging, periodic ranging and handover. Non-contention based random access is used for periodic ranging and handover.

##### Ranging Channel for Non-Synchronized Mobile Stations

The ranging channel for non-synchronized MSs is used for initial access and handover.

          - Multiplexing with other control channels and data channels

The UL ranging channel for non-synchronized MSs starts at a configurable location with the configuration defined in a DL broadcast control message.

The UL ranging channel for non-synchronized MSs is FDM with other UL control channels and data channels.

###### PHY structure

The physical ranging channel for non-synchronized mobile stations consists of three parts: 1) ranging cyclic prefix (RCP), 2) ranging preamble (RP) and 3) guard time (GT). The length of RCP shall not be shorter than the sum of the maximum channel delay spread and round trip delay (RTD) of supported cell size. The length of GT shall not be also shorter than the RTD of supported cell size. The length of ranging preamble shall be equal to or longer than RCP length of ranging channel. The details on the length of each part and its configurations are FFS. To support large cell sizes, the ranging channel for non-synchronized MSs can span multiple concatenated subframes.

The physical resource of ranging channel for non-synchronized mobile stations is consecutive Nrsc ranging subcarriers (BWRCH-NS Hz corresponding to continuous Nrru CRUs) and Nrsym OFDMA symbols (TRCH-NS sec). As a default configuration, Nrsc and Nrsym are equal to [TBD] ranging subcarriers and Nsym OFDMA symbols, respectively, where Nsym depends on the subframe type as described in section 11.6.

Figure xxx shows the default ranging channel structure spanning one subframe. The ranging preamble is repeated as a single opportunity. A single preamble can be used by different non-synchronized MS for increasing ranging opportunities. When the preamble is repeated as a single opportunity, the second RCP can be omitted for coverage extension. The guard subcarriers shall be reserved at the edge of non-synchronized ranging channel(s) physical resource. CDM allows multiple MSs to share the same ranging channel. The details of the ranging structure within the localized resource are FFS. In the TDD mode, the GT can be omitted for extending the length of RCP.

Figure xxx: The default ranging structure for non-synchronized MSs

Support for multi-antenna transmission is FFS. When multi-antenna transmission is supported by MS, it can be used to increase the ranging opportunity by spatial orthogonality.

##### Ranging Channel for synchronized mobile stations

The ranging channel for synchronized MSs is used for periodic ranging. The use of the ranging channel for synchronized MSs for handover is FFS.

###### Multiplexing with other control channels and data channels

The UL ranging channel for synchronized MSs starts at a configurable location with the configuration defined in a DL broadcast control message.

The UL ranging channel for synchronized MSs is FDM with other UL control channels and data channels.

###### PHY structure

The ranging sequence design and mapping to subcarriers are TBD.

#### Bandwidth Request Channel

Contention based or non-contention based random access is used to transmit bandwidth request information on this control channel. Prioritized bandwidth requests are supported on the bandwidth request channel. The mechanism for such prioritization is TBD.

The random access based bandwidth request procedure is described in Figure yy. A 5-step regular procedure (step 1 to 5) or an optional 3-step quick access procedure (step 1,4 and 5) may be supported concurrently. Step 2 and 3 are used only in 5-step regular procedure. In step 1, MS sends a bandwidth request indicator for quick access that may indicate information such as MS addressing and/or request size (FFS) and/or uplink transmit power report (FFS), and/or QoS identifiers (FFS), and the BS may allocate uplink grant based on certain policy. The 5-step regular procedure is used independently or as a fallback mode for the 3-step bandwidth request quick access procedure. The MS may piggyback additional BW REQ information along with user data during uplink transmission (step 5). In step 2 and step 4, BS may send message to acknowledge the reception status.

Figure yyy: Bandwidth Request Procedure

##### Multiplexing with other control channels and data channels

The bandwidth request channel starts at a configurable location with the configuration defined in a DL broadcast control message. The bandwidth request channel is FDM with other UL control and data channels.

##### PHY structure

The bandwidth request (BW REQ) channel contains resources for the MS to send in BW REQ access sequence at the step-1 of the bandwidth request procedure shown in Figure yyFigure yyy.

A BW REQ tile is defined as 6 contiguous subcarriers by 6 OFDM symbols. Each BW REQ channel consists of 3 distributed BW-REQ tiles.

CDM allows multiple bandwidth request indicators to be transmitted on the same BW REQ channel. In addition, multiple BW REQ channels may be allocated per subframe using FDM. The ranging sequence design and mapping to subcarriers are TBD.

### UL Inband Control Signaling

Uplink control information can be multiplexed with data on the UL data channels as MAC headers or MAC management messages. Inband control signaling can contain information such as uplink bandwidth requests or bandwidth assignment updates.

### Mapping of UL control information to UL control channels

<Editors’ Notes: This table needs to be updated as the mapping of UL control information to UL control channels is developed.>

| Information | Channel |
| --- | --- |
| Channel quality feedback | UL Fast Feedback Channel |
|  | UL Sounding Channel |
| MIMO feedback | UL Fast Feedback Channel |
|  | UL Sounding Channel |
| HARQ feedback | UL HARQ Feedback Channel |
| Synchronization | UL Ranging Channel |
| Bandwidth request | Bandwidth Request Channel |
|  | UL Inband Control Signaling |
|  | UL Fast Feedback Channel*(FFS) |
| E-MBS feedback | FFS |

* Transmission of BW REQ indicators on the UL Fast Feedback Channel is FFS