| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Downlink Control Structure related to Hybrid-ARQ |  |
| Date Submitted | 2008-03-10 |  |
| Source(s) | Hyungho Park, Doo-Hyun Sung, Eunjong Lee, HanGyu Cho LG Electronics | Voice : +82-31-450-1934 E-mail: hyunghopark@lge.com, dh.sung@lge.com, mehappy3@lge.com, hgcho@lge.com |
| Re: | 802.16m-08/005 Call for contributions on project 802.16m system description document (SDD). Specific topic : Downlink control structures |  |
| Abstract | This contribution describes and proposes many consideration points and some candidates for IEEE 802.16m DL control channel design related to HARQ. |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Downlink Control Structure related to Hybrid-ARQ

Hyungho Park, Doo-hyun Sung, Eunjong Lee, HanGyu Cho

LG Electronics

# Introduction

This contribution focuses on downlink (DL) control structure related to Hybrid-ARQ (HARQ). Based on the comparison of pros. and cons. of candidate HARQ operations, we strongly propose synchronous non-adaptive HARQ as the UL HARQ operation, and also address the necessity of new ACK/NACK channel design. In addition, we propose HARQ timing diagram for synchronous non-adaptive retransmission in UL HARQ. Also proposed is the maximum number of HARQ processes which is one of the component fields required for HARQ operation and is needed for explicit or implicit retransmission timing mapping in various DL/UL ratios.

# Proposal of HARQ operations

The sorts of component fields for HARQ functionality in DL control channel depend on which HARQ operation is applied. This section, therefore, starts with comparison of candidate HARQ operations.

# Candidate HARQ operations

There are two representative HARQ operations when considering both retransmission timing and format - Synchronous Non-adaptive and Asynchronous Adaptive. The meaning of asynchronous/synchronous and adaptive/non-adaptive HARQ is described in Appendix.

Table 1 shows the general advantages and disadvantages of two operations, mainly based on scheduling gain and signaling overhead.

Table 1. Comparison of advantages and disadvantages of two candidate HARQ operations.

|  | Asynchronous Adaptive | Synchronous Non-adaptive |
| --- | --- | --- |
| Advantages | - Larger amount of scheduling gain - Easy to avoid collision with high prioritized services (persistently scheduled VoIP MS, MBMS) | - Less control signaling overhead - More useful under larger amount of DL/UL data Traffic - No burden to map HARQ process ID to the associated subframe in various DL/UL ratios and legacy/16m ratios. |
| Disadvantages | - Control signaling overhead - Strong dependency of scheduling gain on the reliability of feedback information | - Solutions needed to avoid collision with high prioritized services (VoIP, MBMS) - Difficult to exploit scheduling gain |

# Proposal of UL HARQ operation for IEEE 802.16m

When we consider asynchronous adaptive operation as the UL HARQ, additional disadvantages besides what is shown in Table 1 exist as follows:

- The leakage of scheduling gain is inevitable since interference level from MSs in other cells will vary in time [1].

- Asynchronous adaptive HARQ has further control signaling overhead in sub-framed structure which would be probably considered in 802.16m. It also requires more processing burden in MS side.

Based on the Table 1 and the above additional disadvantages of asynchronous adaptive operation, we propose synchronous non-adaptive HARQ as the UL HARQ operation for IEEE802.16m. Table 2 compares signaling overhead of various HARQ operations where it is shown that synchronous non-adaptive HARQ operation does not have to consider additional component fields for retransmission.

In reality, legacy 16e adopts ACK/NACK signaling based on AI_SN field for UL HARQ, which implicitly supports asynchronous adaptive HARQ operation. To support synchronous non-adaptive operation for UL HARQ in 16m, we strongly believe that new ACK/NACK channel design is needed and further discussion about specific design technique should be developed based on the analysis of overhead and reliability.

To avoid potential collision with other high prioritized transmissions such as persistently scheduled VoIP and MBMS, we also support partly adaptive HARQ and detailed operation is FFS.

Table 2: Signaling overhead comparison of various HARQ operations.

| Component fields related to HARQ | Asynchronous |  |  |  | Synchronous |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Adaptive |  | Non-adaptive |  | Adaptive |  | Non-adaptive |  |
|  | Int. Tx | Re Tx | Int. Tx | Re Tx | Int. Tx | Re. Tx | Int Tx | Re Tx |
| Resource allocation | 24 | 24 | 24 | 0 | 24 | 24 | 24 | 0 |
| Duration assignment | 2 | 2 | 2 | 0 | 2 | 2 | 2 | 0 |
| Modulation scheme | 2 | 2 | 2 | 0 | 2 | 2 | 2 | 0 |
| Payload size | 5 | 5 | 5 | 0 | 5 | 5 | 5 | 0 |
| Antenna configuration | 2 | 2 | 2 | 0 | 2 | 2 | 2 | 0 |
| HARQ process number | 3 | 3 | 3 | 3 | 0 | 0 | 0 | 0 |
| Redundancy version | 2 | 2 | 0 | 0 | 2 | 2 | 0 | 0 |
| New Data indication | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| CRC | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 0 |
| Total (bits) | 57 | 57 | 54 | 19 | 54 | 54 | 51 | 0 |

# DL HARQ operation for IEEE 802.16m

In case of DL HARQ operation, asynchronous adaptive HARQ operation would be more desirable in terms of scheduling gain since it is easier to estimate and predict the SINR level in DL due to fixed interferences coming from other BSs.

However, we also need to consider benefits of synchronous non-adaptive operation. In particular, when legacy and 16m are considered together and sub-framed structure is considered, the significance of overhead reduction would be increased. Further consideration on HARQ operations in DL may not be excluded.

# Proposal of HARQ process timing and HARQ process number

In order to design the configuration of HARQ operation for IEEE 802.16m, it is needed to find how many HARQ process can be supported to achieve continuous retransmissions. In addition, HARQ timing diagram is required for explicit or implicit signaling of HARQ functionality. Here we assume that processing delay for both MS and BS is 2 TTI including frame alignment and propagation delay [2]. Detailed HARQ timing diagram is presented in text proposal where we can figure out mapping rule for ACK/NACK signaling and retransmission according to each HARQ process number.

Also shown in the HARQ timing diagram is the maximum number of HARQ process for HARQ. Since the analysis of HARQ process number can be adopted for both UL and DL, we summarize it in Table 3 and propose that the maximum number of HARQ processes in 16m considering various DL/UL ratios is eight.

Table 3. HARQ process number for DL/UL HARQ.

| DL:UL ratio | 3:5 | 4:4 | 5:3 | 6:2 | 2:6 |
| --- | --- | --- | --- | --- | --- |
| HARQ process number | 5 | 4 | 3 | 2 | 8 |

# Conclusion

We would like to replace conclusion with the following text proposal.

Text Proposal for the 802.16m SDDs [3]

================================= Start of Proposed Text ====================================

x.x Uplink Hybrid-ARQ operation

Uplink (UL) Hybrid-ARQ (HARQ) is based on synchronous non-adaptive HARQ. Fig. y.y. shows the UL HARQ timing diagram where we can figure out implicit mapping between HARQ process number and the associated unit of TTI for ACK/NACK signaling and retransmission. Considering the N-channel stop and wait protocol, the maximum number of HARQ process is eight. To efficiently support synchronous non-adaptive UL HARQ, new ACK/NACK channel design is definitely required and detailed design is FFS. Detailed method to avoid potential collision with other services such as persistent scheduled VoIP and MBMS is also FFS.

- DL:UL = 5:3 (3 x 8 TTI RTT)

- DL:UL = 6:2 (2 x 8 TTI RTT)

- DL:UL = 4:4 (4 x 8 TTI RTT)

- DL:UL = 3:5 (5 x 8 TTI RTT)

- DL: UL = 2:6 ( 6 x 10 TTI RTT & 2 x 12 TTI RTT)

Figure y.y: UL HARQ timing diagram

=================================== End of Text Proposal ===================================

# References

[1] 3GPP LTE R1-072355, “Further consideration on UL synchronous HARQ and its modification”

[1] IEEE 802.16m-08/096r10, “Consolidated Membership Inputs on 802.16m Frame Structure|”

[3] IEEE 802.16m-08/003, “The draft IEEE802.16m System Description Document”

# Appendix

- Classification of HARQ operations

  - Synchronous/ Asynchronous HARQ (retransmission timing)

HARQ operation can be classified into sorts of two according to whether the time of HARQ process is known as explicit or implicit signaling. Synchronous HARQ define that retransmission for a certain HARQ process is restricted to occur at known TTI between MS and BS. There is no explicit signaling to perform HARQ process. In case of Asynchronous HARQ, retransmission may take place at any time. Therefore, explicit signaling such as HARQ process number, Redundancy Version and new data indicator is required to perform HARQ process.

  - Adaptive/Non-adaptive HARQ (retransmission format)

HARQ operation can be further classified as being adaptive or non-adaptive in terms of transmission attribute (e.g. modulation, code rate, resource block allocation, duration of transmission, etc.). Adaptive means that the some or all of transmission attribute may be changed during retransmission comparing to initial transmission. In case of non-adaptive HARQ, the format for retransmission is not changed or is known to both MS and BS. Hence, associated the control signaling is not required.