| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on the 802.16m Frame Structure Baseline Content For Relay |  |
| Date Submitted | 2008-03-12 |  |
| Source(s) | Jeff Bonta Steve Emeott | E-mail: jeff.bonta@motorola.com steve.emeott@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | - Raporteur group call for comments on contribution C802.16m-08/118r1 |  |
| Abstract | This contribution proposes the 802.16m frame structure baseline content (C802.16m-08/118r1) be modified to also include Ad Hoc Relay in section 11.4.4. |  |
| Purpose | To be discussed and adopted by TGm for use in the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Comments on the 802.16m Frame Structure Baseline Content for Relay

Jeff Bonta, Steve Emeott

Motorola

# Introduction

Within C802.16m-08/118r1, section 11.4.4 on "Relay Support in Frame Structure" limits the definition of a "Relay" to the context of an infrastructure relay (ala IEEE 802.16j). The “Relay” context should also enable a mobile station to be an ad hoc relay for another mobile station(s). Furthermore, Figure 11.4-5 excludes provision for coexistence of infrastructure relay with mobile station relay.  Contribution C802.16m-08/003 describes the use of an Ad Hoc Zone to enable opportunistic ad hoc relay. We propose substitute text and figures to enable opportunistic ad hoc relay as an independent option or coexistence option with infrastructure relay.

# Proposed Changes to Section 11.4.4 and Figure 11.4-5

Modify text in clause 11.4.4 as follows:

When the relay support is enabled, in order to support relay operations, the definition of IEEE 802.16m DL and UL time zones sub-frames is expanded to support communications with RSs as well as communications with MSs. Similarly, the definition of IEEE 802.16m UL sub-frames is expanded to support communications with RSs as well as communications with MSs or ARSs (Ad hoc Relay Stations). An example Example relay-enabled TDD frame structures supporting both RSs and ARSs are is illustrated in Figures 11.4-xxx and 11.4-yyy. The top half of the figures shows the frame structure from the perspective of the BS and the bottom half shows the frame structure from the perspective of the RSs and ARSs. In order to support relay operations, an additional IEEE 802.16m DL relay zone, and an additional IEEE 802.16m UL relay zone, and an additional IEEE 802.16m Ad Hoc Relay Zone are introduced to the frame structure. The UL sub-frames may include an UL Relay Zone, or an Ad Hoc Relay Zone, or both as illustrated in Figure 11.4-xxx. At times of low traffic demand, the IEEE 802.16m Ad Hoc Relay Zone and UL Relay Zone may be grouped into a multi-frame as illustrated in Figure 11.4-yyy. Receive/transmit transmission gaps for the RSs are required between the IEEE 16e DL zone and the IEEE 16m DL relay zone and between the two consecutive IEEE 802.16m DL relay/access zones and between the IEEE 802.16m UL relay/access zones to allow the RSs to transition from receive to transmit operation and vice versa. These gaps are required only for the RSs and thus appear only in the RS frame structure. A receive/transmit transmission gap for the ARSs is required following the IEEE 802.16m Ad Hoc Zone to allow an ARS to transmit during the Ad Hoc Zone and receive from the BS in the next IEEE 802.16m DL sub-frame. The BS may transmit to MSs or receive from MSs during these gaps.

Replace Figure 11.4-5 with the following two figures:

802.16m

Ad Hoc Relay

Zone

MS->MS

802.16e

DL Zone

802.16m

Access DL Zone

RS -> MS

BS -> MS

802.16m

Relay DL

Zone

BS -> MS

BS -> RS

802.16e

UL Zone

802.16m

Access UL Zone

MS -> RS

MS -> BS

802.16m

Relay UL

Zone

RS -> BS

MS -> BS

Transmission gap to allow RS to

switch between RX and TX

DL/UL Switching Point

Sub-frames

Radio Frame = 5 ms

BS/MS/RS Processing Interval = 2 sub-frames = 1.23 ms

802.16e

DL Zone

802.16m

Access DL Zone

RS -> MS

BS -> MS

802.16m

Relay DL

Zone

BS -> MS

BS -> RS

802.16e

UL Zone

802.16m

Access UL Zone

MS -> RS

MS -> BS

802.16m

Relay UL

Zone

RS -> BS

MS -> BS

BS Frame Structure

RS and ARS Frame Structure

Figure 11.4-xxx: RS and ARS Support in TDD Frame Structure with Independent Sub-Frames

802.16e DL Zone

Transmission gap to allow RS to

switch between RX and TX

DL/UL Switching Point

Sub-frames

Radio Frame = 5 ms

BS/MS/RS Processing Interval = 2 sub-frames = 1.23 ms

BS Frame Structure

RS or ARS Frame Structure

16m Relay DL Zone

BS -> MS, BS -> RS

802.16m Access DL Zone

RS -> MS, BS -> MS

802.16e UL Zone

802.16m Access UL Zone

MS -> RS, MS -> BS

16m Relay UL Zone

RS -> BS, MS -> BS

802.16e DL Zone

DL/UL Switching Point

802.16m Access DL Zone

RS -> MS,

BS -> MS

802.16e UL Zone

802.16m Access UL Zone

MS -> RS,

MS -> BS

802.16e DL Zone

16m Relay DL Zone

BS -> MS, BS -> RS

802.16m Access DL Zone

RS -> MS, BS -> MS

802.16e UL Zone

802.16m Access UL Zone

MS -> RS, MS -> BS

16m Relay UL Zone

RS -> BS, MS -> BS

802.16e DL Zone

802.16m Access DL Zone

RS -> MS,

BS -> MS

802.16e UL Zone

802.16m Access UL Zone

MS -> RS, MS -> BS

16m Ad Hoc Relay

Zone    MS->MS

Radio Frame = 6 ms

Figure 11.4-yyy: RS and ARS Coexistence Support in TDD Frame Structure in Alternating Radio Frames

# References

- IEEE 802.16m-08/118r1 (“IEEE 802.16m Frame Structure Baseline Content”)

- IEEE C802.16m-08/003 (“Ad Hoc Relay Frame Structure”)