| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multicarrier proposal for 16m |  |
| Date Submitted | 2008-05-05 |  |
| Source(s) | Adrian Boariu, Shashi Maheshwari, Yousuf Saifullah, Peter Wang Nokia Siemens Networks Zexian Li Nokia | adrian.boariu@nsn.com zexian.li@nokia.com |
| Re: | Comments on IEEE 802.16m-08/003r1 |  |
| Abstract | A multicarrier proposal for 16m. |  |
| Purpose | To incorporate the proposals into the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Multicarrier Proposal for 16m

Adrian Boariu, et al.

NSN, et al.

# Introduction

Multicarrier (MC) operation has been proposed as solution to increase the system throughput.

This contribution is proposing to harmonize different views regarding operation with wider bandwidth, as well as providing a solution for multicarrier operation with adjacent or non-adjacent frequency bands.

Figure 1. Different modes of utilizing the spectrum, exemplified with 5 MHz and 10 MHz narrowband, and 20 MHz wideband systems.

In the case a) a group of non-overlapped single carrier systems can allocate together a common zone that spans across the group spectrum. The advantage of this mode of operation is that can use the guard bands of the adjacent carriers for transmission to MSs that support wider bandwidths. First disadvantage is that the system is not flexible to incorporate non-contiguous bands. Second disadvantage is that the carrier frequencies for the single carrier bands have to stop in order to allow the transmission of the wider band zone with its carrier. The third disadvantage can be that some inefficiency may result with utilization of the wideband zone due to a coarse allocation of the zone.

In the case b) the spectrum is not overlapped; the two narrowband systems can be adjacent or not. This greatly simplifies the design, and it is a natural extension of the single band system. This option offers flexibility and simplicity of operation. As disadvantage we can mention that in the case of adjacent carrier operation, the guard bands are not utilized, which is wastage of resources.

# Proposed Text

[Insert the following section]

11.4.3.1 Multicarrier operation support

A multicarrier system is capable of operating simultaneously on several different single carrier systems that have non-overlapped spectrum. A multicarrier capable MS may be enabled by the BS to operate in multiple adjacent or non-adjacent frequency bands. The multicarrier capabilities of an MS are provided during the network entry to the BS. The resource allocation for a carrier, which is part of a multicarrier operation, is performed within the carrier for every frame. An example of such system is presented in Figure xxx.

Figure xxx. Example of a multicarrier system that operates in two 10 MHZ single carrier systems.

[Insert the following section]

11.4.3.2 Wider channel zone operation support

The wider channel capabilities of an MS are provided during the network entry to the BS. This mode of operation involves adjacent non-overlapped single carrier systems, which together form a wider nominal channel bandwidth. Within the frame duration, a common zone in time across all single carrier systems is allocated. This common zone is the wider channel zone that encompasses all carriers. The common zone has its own control mechanism, and this zone should be flagged appropriately in each of the single carrier systems. Two examples of such configuration are provided in Figure xxy.

Figure xxy. Examples of operation with a zone that has wider spectrum.