| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Analogue beamforming |  |
| Date Submitted | 2007-03-10 |  |
| Source(s) | Toshiyuki Kuze Mitsubishi Electric Corp Andreas F. Molisch, Zhifeng (Jeff) Tao, Philip V. Orlik, Jinyun Zhang Mitsubishi Electric Research Lab | Voice: +81-467-41-2885 Fax: +81-467-41-2486 Kuze.Toshiyuki@ah.MitsubishiElectric.co.jp Voice: 617-621-{7558,7570, 7557, 7595} Fax: 617-621-7550 {molisch, porlik, tao, jzhang}@merl.com |
| Re: | [Cite the specific document number of the appropriate Call for Contributions, the ballot number, etc. Contributions that are not responsive to this section of the template, may be refused or assigned a low priority for review.] |  |
| Abstract | Response to the Call for Contributions on Project 802.16m System Description Document (SDD) (i.e., IEEE 802.16m-08/005). |  |
| Purpose | [Description of what specific action is requested of the 802.16 Working Group or subgroup.] |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Analogue beamforming

[Toshiyuki Kuze, Andreas F. Molisch, Zhifeng (Jeff) Tao, Philip V. Orlik, Jinyun Zhang]

[Mitsubishi Electric Corp., Mitsubishi Electric Research Labs>]

1. Introduction

The goal of IEEE 802.16m is to improve the spectral efficiency of WiMAX systems while keeping the cost of deployments at a minimum. In this contribution, we suggest that analogue beamforming and beam switching (ABF) can help achieve this goal. We will outline the principle of ABF and show how it can help both for range extension and interference reduction of WiMAX systems. We furthermore describe which changes have to be implemented in the standard to accommodate ABF.

The basic principle of ABF is to form at the BS a number of beams within each sector, which can be achieved through a linear antenna array concatenated with a Butler matrix, see Fig. 1

Fig. 1 Beamforming with Butler matrix

The BS then switches through the available beams (both in uplink and downlink) in an arbitrary sequence, and communicates at each time with the MSs located in the respective beams. Due to the beamforming gain, the range of the cell is extended – which is important especially for deployment in rural areas, and during the initial deployment of a network. Furthermore, by adopting appropriate beam switching patterns, the interference situation can also be improved.

Figure 2 Transmission phases of beam switching

We emphasize that ABF is not a theoretically-optimum way of using multiple antenna elements; heterodyning all the signals to/from baseband and digitally processing the signals would allow for higher capacity. However, it does present an excellent tradeoff between performance and complexity – it requires only a single RF chain.

Another important point is that ABF can be combined with spatial multiplexing and other MIMO techniques. The set of N available antenna elements can be divided into K groups of M antennas each (with M*K=N), so that K beams can be formed, and in each beam M antenna elements are available for spatial multiplexing etc. Especially when dual-polarized antennas are used, it is easily possible to use K=4, N=2.

2. Frame structure for ABF

ABF requires the definition of a new zone, so that MSs in different zones can be handled sequentially. DL frame is divided into several zones. In each zone, several MSs using the  same beam is grouped. There is a separate preamble at the beginning of each AFB zone, and the MSs detect which preamble is the strongest for them; this allows to associate themselves gives the best SINR (signal-to-interference ratio).

AAS diversity MAP zone

AAS diversity MAP zone

Figure 3  Frame structure for analog beamforming for AMC

Fig. 3 shows more details of the frame structure when BF is used with AMC..  When BF is used with PUSC/FUSC, conventional AAS zone can be used.  The conventional DL-MAP is used to convey the information of BF zones and existence of zone preamble. Then each zone has a UL-DL compressed map and this MAP shows each zone’s ranging region. In the first stage, MS tries to detect the zone preamble. And if the MS detects a certain zone preamble, then the MS starts ranging process by using this zone’s ranging region. BS decides the MS’s group by using this ranging.

Interference reduction with ABF

ABF can also be used to improve the interference situation. Remember that MSs in different beams are served at different times. Therefore, if two neighboring cells arrange the downlink transmission in such a way that they do not transmit to the same part of the cell-edge at the same time (and similarly for the uplink), then the interference seen by the MS is greatly reduced. Fig. 4 shows an example for such a situation; the numbers in the beams indicate the BF zone during which they are served.

Fig. 4  Interference reduction through beam switching

If the BSs can coordinate the beams, then interference from neighboring cells can be almost completely eliminated, and only second-tier interference remains. If BSs cannot coordinate, then the sequence in which beams are served can be chosen randomly, and independently at each BS. This still leads to a stochastic reduction of the interference, similar to the reduction of interference in random frequency hopping or time-hopping impulse radio.

The hopping sequence can be derived at each BS based upon the base station identification (BS ID).  For example, the BS ID is used as the initial value (i.e., seed) to the feedback shift register that will generate the hopping sequence.

3. Training for ABF

We already mentioned that each zone has its own preamble that enables the MS to determine which beam it best belongs to. This requires that the MS listens to all ABF preambles; a slightly suboptimum solution is that the MS only listens to the beams that are neighboring to the one it is currently communicating with. Additionally, it is not necessary for the MS to listen to the adjacent preambles during each frame: the time that an MS stays within one beam is usually quite large (on the order of seconds), so that infrequent listening is sufficient.

For the uplink, the MS can from time to time transmit ranging signals in the zones associated with adjacent beams (adjacent to the beam the MS is currently in). The periodic ranging shall be arranged in such a way that collisions of the MS signal with that of other ranging signals are minimized; this is arranged by the BS, which controls the UL ranging  through the UL map

4. Feedback for beam scheduling

It may be helpful to let MS feedback to the BS the index of the beam that it receives the best signal.  The BS can use this feedback information to perform beam scheduling.  In AMC mode, the feedback from MS should include channel quality indication for different beams for all the subchannels.  This can further enable frequency and beam scheduling.

5. Conclusions

In this contribution, we have presented a simple but extremely efficient method for increasing SINR, and thus throughput of WiMAX systems. We stress that the method is not an optimum way of exploiting multiple antenna elements: a 4-beam switching system cannot perform as well as a full 4-antenna MIMO systems. However, the complexity of a 4-beam switching system is much lower than a 4-antenna MIMO system – as a matter of fact, the complexity is identical to that of a single-antenna system, with just a switch and 3 antenna elements as extra requirements.

Beam switching provides a low-cost and efficient solution both for range extension and for interference reduction.