| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multiple CP Support |  |
| Date Submitted | 2008-03-14 |  |
| Source(s) | Chongli Liu, Mingyang Sun, Xin Chang, Sean McBeath Huawei Technologies CO., LTD. Ying Du, Xiaolu Dong CATR Shanpeng Xiao, Wenqi Liao CMCC | Voice: +86 13424154960 E-mail: liuchongli@huawei.com duying@mail.ritt.com.cn; xiaoshanpeng@chinamobile.com |
| Re: | E-mail call for Contributions on “C80216m-08_118r1” of Project 802.16m |  |
| Abstract | This contribution covers the considerations about the CP length for IEEE802.16m system. |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Multiple CP Support

Chongli Liu, Mingyang Sun, Xin Chang, Sean McBeath

Huawei

Ying Du, Xiaolu Dong

CATR

Shanpeng Xiao, Wenqi Liao

CMCC

# Introduction

IEEE 802.16e-2005 supports multiple CP. Four options are provided: 1/4Tu, 1/8Tu, 1/16Tu and 1/32Tu. To simplify the system application, in the WIMAX system profile, only 1/8 CP is supported.

Table 1 - CP Length in IEEE 802.16e-2005

| CP | Number of OFDM symbols per frame |
| --- | --- |
| Tg = ¼ Tu (22.85μs) | 43 |
| Tg = 1/8Tu (11.42μs) | 48 |
| Tg = 1/16 Tu ( 5.71μs) | 51 |
| Tg = 1/32 Tu (2.86μs) | 53 |

In C80216m-08_118r1, only 1/8Tu CP length is included in proposal 1 for OFDMA parameters. In this contribution, we suggest that multiple CP lengths should be supported in 802.16m.

# Single CP vs. Multiple CP

Single CP

- Advantage: The frame structure will be simple.

- Disadvantage: It’s not efficient for various network deployments.

Multiple CP

- Advantage: It maintains efficiency for various network deployments.

- Disadvantage: Extra processing may be needed.

# Impact on 16m frame structure

For a pure 16m system, different CP lengths leads to a different number of OFDM symbols in one frame, as shown in Table 2.

Table 2 – Example: Number of OFDM symbols in one frame

| CP mode | ¼ | 1/8 | 1/16 | 1/32 |
| --- | --- | --- | --- | --- |
| Number of OFDM symbols per frame | 43 | 48 | 51 | 53 |
| Idle time | 85.53us | 62.72us | 45.86us | 2.63us |

We need to consider how to assign the OFDM symbols into the 8 sub-frames. The OFDM symbols in one sub-frame may be different. The assignment information may be transmitted using the broadcast channel.

One possible solution is shown in Table 3.

Table 3 Example Assignments

| CP | subframe #1 | subframe #2 | subframe #3 | subframe #4 | subframe #5 | subframe #6 | subframe #7 | subframe #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/4 Tu | 6 | 5 | 5 | 6 | 6 | 5 | 5 | 5 |
| 1/8 Tu | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| 1/16 Tu | 6 | 7 | 6 | 6 | 6 | 6 | 7 | 7 |
| 1/32 Tu | 6 | 7 | 7 | 6 | 6 | 6 | 7 | 7 |

Since the number of OFDM symbols in one subframe may be different, pilot patterns, resource assignment, HARQ timing should be specifically designed.

If legacy support is enabled, the CP length of the 802.16e and the 802.16m zone should be same.

# Conclusion

To improve the spectral efficiency, multiple CP should be supported.

# References

[1] C80216m-08_118r1, “Proposed 802.16m Frame Structure Baseline Content Suitable for Use in the 802.16m SDD”

Text Proposal for C80216m-08_118r1

============================== Start of Proposed Text =================================

11.4.1 Basic Frame structure (Proposal-1)

The IEEE 802.16m basic frame structure is illustrated in Figure 11.4-1. Each 20 ms super-frame is divided into four equally-sized 5ms radio frames. When using the same OFDMA parameters as the reference system, each 5 ms radio frame further consists of eight sub-frames. Each sub-frame can be assigned for either downlink or uplink transmission/reception depending on the duplexing scheme. There are two types of sub-frames in case of a CP length of 1/8 Tu: 1) the regular sub-frames which consist of six OFDMA symbols and 2) the irregular sub-frames that consist of five or less OFDMA symbols.

There are two types of sub-frames: 1) the regular sub-frames which consist of six OFDMA symbols and 2) the irregular sub-frames which consist of five or seven OFDMA symbols.  Irregular sub-frames are used as a result of the preamble, TTG/RTG gaps, or when the CP length is not 1/8 Tu.

============================== End of Proposed Text =================================