| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Inter-cell Multiple Access Scheme in 802.16m Downlink |  |
| Date Submitted | 2008-03-11 |  |
| Source(s) | Shidong Zhou, Yunzhou Li Tsinghua Univ. | E-mail: zhousd@tsinghua.edu.cn, liyunzhou@tsinghua.edu.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | [Cite the specific document number of the appropriate Call for Contributions, the ballot number, etc. Contributions that are not responsive to this section of the template, may be refused or assigned a low priority for review.] |  |
| Abstract | Proposal for Inter-cell Multiple Access Scheme in 802.16m Downlink. |  |
| Purpose | Adopt concept and proposed text into SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Inter-cell Multiple Access Scheme in 802.16m Downlink

Shidong Zhou, Yunzhou Li

Tsinghua Univ.

# 1 Introduction

In this contribution, we propose an inter-cell multiple access scheme for 16m downlink.

# 2 Proposal

IEEE 802.16m should use OFDMA as the intra-cell/sector multiple access scheme and IDMA [1] as the inter-cell/sector multiple access scheme in the downlink.

Within the same cell/sector, users are multiplexed in different subband of the whole system bandwidth using OFDMA. Meanwhile, different cells/sectors use different interleavers for inter-cell/sector-interference randomization, which aims at randomizing the interfering signal(s) and thus to allow for interference suppression at the receiver in line with the processing gain. All transmitted data on each subband of one cell is interleaved/sector using the cell/sector specified interleaver. The users that are far away from the cell/sector edge may use ordinary detection and decoding algorithm to extract their desired information since they suffer little interference from the adjacent cells/sectors. However, the users at the cell/sector edge may use additional algorithm like iterative multi-user detection algorithm in order for more opportunities to achieve high spectrum efficiency at the cell/sector edge. Using IDMA, the frequency resources are reused at the cell/sector edges, e.g., while one base station is transmitting data to a user at the cell/sector edge on some subband, the same subband can also be used by the adjacent cells/sectors without degrading much the performance of the cell/sector-edge user as long as the user has the ability of implementing multi-user detection and interference cancellation.

A pseudo-random method [1] can be used to generate the cell/sector-specific interleaver patterns for IDMA. The number of the available patterns (seeds) is determined by the length of interleaver. A SS can identify the interleaver pattern of the cell/sector by checking its interleaver pattern ID. The seeds can be reused between “far-spaced” cells/sectors in a manner similar to that of frequency reuse in a cellular system. The cell/sector specified interleaver can be also used as channel interleaver, i.e., it is not necessary to apply additional interleavers for interference randomization.

# ------------------------------------------- Start of Proposed SDD Text -------------------------------

## x.x Downlink and Uplink Multiple Access Schemes

IEEE 802.16m uses OFDMA as the intra-cell/sector multiple access scheme and IDMA as the inter-cell/sector multiple access scheme in the downlink.

# ------------------------------------------- End of Proposed SDD Text --------------------------------

# Reference

[1] IEEE C802.16m-07/218r5, “A New Approach for Inter-cell Interference Randomization based on Cell/Sector-specific Interleaver Pattern”.