| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Inter-cell Multiple Access Scheme in 802.16m Downlink |  |
| Date Submitted | 2008-03-11 |  |
| Source(s) | Shidong Zhou, Yunzhou Li Tsinghua Univ. | E-mail: zhousd@tsinghua.edu.cn, liyunzhou@tsinghua.edu.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-08/005: Call for Contributions on Project 802.16m System Description Document (SDD)Target Topic: "Downlink Physical Resource Allocation Unit" |  |
| Abstract | Proposal for Inter-cell Multiple Access Scheme in 802.16m Downlink, and related requirement on Downlink Physical Resource Allocation |  |
| Purpose | Adopt concept and proposed text into SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Inter-cell Multiple Access Scheme in 802.16m Downlink

Shidong Zhou, Yunzhou Li

Tsinghua Univ.

# 1 Introduction

OFDMA is considered as the downlink multiple access scheme for 802.16m. In order to increase the spectrum efficiency, the whole spectrum is shared by all the cells. Therefore, the inter-cell interference should be taken into account for the downlink physical resource allocation in 802.16m. In this contribution, we propose a downlink physical resource allocation scheme considering inter-cell/sector interference management for 802.16m. The scheme is the combination of the fractional frequency reuse and IDMA proposed in [1].

Besides, we provide some proposal on the downlink physical resource allocation related to the effective use of IDMA.

# 2 Proposal

Within the same cell/sector, users are multiplexed in different subband of the whole system bandwidth using OFDMA. Signals for different users shall be assigned with orthogonal resources (time domain and/or frequency domain). The allocation of the subcarriers/symbols for each user can either be distributed (for diversity) or centralized (for scheduling, AMC, MIMO usage, etc.). From the viewpoint of AMC/scheduling/pre-coding, some kind of basic resource unit with proper size (as many other companies proposed) is preferred.

As for the radio resource allocation among different cells/sectors, orthogonal allocation among adjacent cells/sectors is a good way to suppress mutual interference when the traffic is not very heavy. However, when the traffic, especially at the cell/sector edge, some more advance technique other than orthogonal allocation can be applied in additional to orthogonal ones, e.g. IDMA[ ].

In order to illustrate clearly of the resource allocation problem, we take an example of frequency reuse factor as 3. Users in the “center” of the same cell are multiplexed in different subchannels (each subchannel is composed of several subcarriers distributed or adjacent with each other) of the whole system bandwidth using OFDMA.

In order to deal with the interference at the cell edge, the whole system spectrum is divided into four segments as depicted in Fig. 1. The size of each segment can be different and dynamic with the traffic requirement.

Fig.1 Spectrum segmentation for downlink physical resource allocation at the cell edge.

As in Fig. 1, denoting the three adjacent cells as Cell 1, 2 and 3, then fractional frequency reuse is supported, i.e., users at the cell edge of Cell 1 can only be allocated with the red segment of the spectrum, and users at the cell edge of Cell 2 and 3 can only be allocated with the yellow and blue segment of the spectrum, respectively. The dedicated frequency resource segment for each cell is divided into subchannels or chunks in order to multiplex multiple users at the cell edge.

Meanwhile, the users at the edge of the three cells can still share the fourth (green) segment of the spectrum by applying IDMA [1]. Different cells use different interleavers for inter-cell interference randomization, which aims at randomizing the interfering signal(s) and thus to allow for interference suppression at the receiver in line with the processing gain. All transmitted data on this IDMA-dedicated (green) frequency segment of one cell is interleaved using the cell specified interleaver. The users at the cell edge may use advanced detection algorithms such as iterative multi-user detection algorithm in order for more opportunities to achieve higher spectrum efficiency at the cell/sector edge. Using IDMA, the green frequency resources are reused at the cell edges.

In addition, the green zone of resource can be further divided into different subzones, i.e., Z12, Z13, Z23, Z123, corresponding to IDMA between cells 1 and 2(this zone is shared by users from cell 1 and 2), cells 1 and 3, cells 2 and 3, and all cells, respectively. Also, the size of each zone can be dynamic with the traffic of the cells/sectors.

Benefit of using IDMA

With IDMA(interleaver division multiple access), different cells/sectors use different interleavers for inter-cell/sector-interference randomization, which aims at randomizing the interfering signal(s) and thus to allow for interference suppression at the receiver in line with the processing gain. All transmitted data on each subband of one cell/sector is interleaved using the cell/sector specified interleaver. The users that are far away from the cell/sector edge may use ordinary detection and decoding algorithm to extract their desired information since they suffer little interference from the adjacent cells/sectors. However, the users at the cell/sector edge may use additional algorithm like iterative multi-user detection algorithm in order for more opportunities to achieve high spectrum efficiency at the cell/sector edge. Using IDMA, the frequency resources are reused at the cell/sector edges, e.g., while one base station is transmitting data to a user at the cell/sector edge on some subband, the same subband can also be used by the adjacent cells/sectors without degrading much the performance of the cell/sector-edge user as long as the user has the ability of implementing multi-user detection and interference cancellation.

A pseudo-random method [1] can be used to generate the cell/sector-specific interleaver patterns for IDMA. The number of the available patterns (seeds) is determined by the length of interleaver. A SS can identify the interleaver pattern of the cell/sector by checking its interleaver pattern ID. The seeds can be reused between “far-spaced” cells/sectors in a manner similar to that of frequency reuse in a cellular system. The cell/sector specified interleaver can be also used as channel interleaver, i.e., it is not necessary to apply additional interleavers for interference randomization.

It is important to note that, as an interference whitening technique, IDMA enables frequency reuse among adjacent cells/sectors, no matter which kind of resource allocation scheme is used, and no matter which multiple access technique is used for intra-cell/sector multiple access.

Requirement on resource allocation

For the possibility of applying multi-user detection algorithm, the resource blocks allocated on the IDMA-dedicated segment in different cells should be aligned so that one user at the edge of some cell is able to detect and decode the interference signals from the adjacent cells and subtract them from the received signal. As sown in Fig. 2, 2 users from cell A and 3 users from cell B are to share the same time/frequency resource simultaneously. In Fig.2(b), the users are allocated with resources without alignment, thus when iterative decoding is to be performed, signals of all the 5 users should be jointly decoded, which may lead to large computational complexity. While in Fig. 2(a), user 1 from cell A and user 1 from cell B can be jointly processed, user 2 from cell A and user 2, 3 from cell B will be jointly processed, thus much less complexity can be achieved.

The particular ways and algorithms of resource allocation alignment among adjacent cells are FFS.

(a) allocation with user alignment            (b) allocation without alignment

Fig. 2 Resource allocation alignment with users

from adjacent cells for use of non-orthogonal resource allocation

# ------------------------------------------- Start of Proposed SDD Text -------------------------------

## x.x Downlink and Uplink Multiple Access Schemes

IEEE 802.16m uses OFDMA as the downlink intra-cell/sector multiple access scheme.

As for downlink inter-cell/sector multiple access schemes, besides orthogonal(time/frequency) multiple access among adjacent cells/sectors where cell/sector edge traffic is not very heavy, non-orthogonal multiple access shall be included for heavy traffic as cell/sector edge, when adjacent cells/sectors can use the same frequency/time resource simultaneously.

As an additional non-orthogonal multiple access technique, IDMA is used to support higher rate for users at cell/sector edge.

## x.x Downlink Physical Resource Allocation Unit

When non-orthogonal downlink multiple access among adjacent cells/sectors is to be applied for downlink resource allocation, a segment for non-orthogonal resource allocation shall be defined, which is better to be aligned with adjacent cells/sectors. Tthe allocation of such resource segment among users shall support resource unit alignment among users from adjacent cells/sectors which may share the resource.

# ------------------------------------------- End of Proposed SDD Text --------------------------------

# Reference

[1] IEEE C802.16m-07/218r5, “A New Approach for Inter-cell Interference Randomization based on Cell/Sector-specific Interleaver Pattern”.