| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Frame Structure to Support Multiple CP Sizes with Fixed Sub-frame Size |  |
| Date Submitted | 2008-03-10 |  |
| Source(s) | Yungsoo Kim Jumi Lee Yu-Seok Kim Samsung Electronics Co., LTD | E-mail: y2.kim@samsung.com j.esther.lee@samsung.com yseok.kim@samsung.com |
| Re: | Call for Comments on IEEE C802.16m-08/118r1, Proposed 802.16m Frame Structure Baseline Content Suitable for Use in the 802.16m SDD |  |
| Abstract | This contribution proposes 802.16m frame structure to support multiple CP sizes with fixed size sub-frames. |  |
| Purpose | To be discussed and approved by TGm for use in the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Frame Structure to Support Multiple CP Sizes with Fixed Sub-frame Size

Yungsoo Kim, Jumi Lee, Yu-Seok Kim

Samsung Electronics Co., LTD

# Introduction

This contribution proposes frame structure that supports multiple CP sizes to efficiently adapt to various communication environments from indoor to outdoor cells. The proposed frame structure design is based on a fixed sub-frame size, so that it supports coexistence of different CP sizes not only in adjacent cell/frequency, but also within the same cell/frame.

# Baseline Frame Structure

The baseline frame structure prepared by the Frame Structure Rapporteur Group [1] is mainly based on a fixed CP size of 1/8-Tu, where Tu is the OFDMA useful symbol time. Because the legacy system is based on the OFDM symbol with 1/8 CP, the proposed frame structure represents a basis for the design of 16m system. The 1/8 CP size corresponds to time duration of 11.43 us. However, a fixed size CP of 11.43 us can be a waste of resource for some environments such as indoor, while it may not be sufficiently long enough in a very large cell with sever multi-path propagation. It is, therefore, desirable to have multiple CP sizes in order to efficiently support the various communication environments.

The baseline frame structure content also proposes OFDMA parameters for three different CP sizes: Short, Normal, and Long. However, it does not explain how frame structure is designed using the different CP sizes. Blindly adapting various CP sizes in the frame structure design, however, can potentially lead to several important problems that severely impairs actual deployment with multiple CP sizes. In TDD mode, the DL and UL periods of the frames with different CP sizes are in general not time aligned. This can cause interference problem when different CP sizes are adapted. Another important drawback is that co-existence of different CP sizes within a frame is in general not possible, so that legacy support is not possible when using CP sizes other than the legacy 1/8 CP.

This contribution proposes a frame structure design that provides a solution for these problems. It proposes to use three different CP sizes of Short, Normal, and Long, to efficiently adapt to various communication environments. It also proposes to use sub-frame structure with a fixed size for the multiple CP sizes to address the problem of co-existence and legacy support.

# Proposed Text Change

Insert the following text into Proposed 802.16m Frame Structure Baseline Content.

----------------------------------------------------Text Start-----------------------------------------------------------------

# 11.3 OFDMA Parameters

| Nominal Channel Bandwidth (MHz) |  | 5 |  | 10 |  | 20 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Over-sampling Factor |  | 28/25 |  | 28/25 |  | 28/25 |  |
| Sampling Frequency (MHz) |  | 5.6 |  | 11.2 |  | 22.4 |  |
| OFDM Symbol Types |  | Full | Half | Full | Half | Full | Half |
| FFT Size |  | 512 | 256 | 1,024 | 512 | 2,048 | 1,024 |
| Sub-carrier Spacing (kHz) |  | 10.94 | 21.88 | 10.94 | 21.88 | 10.94 | 21.88 |
| OFDMA Useful Symbol Time Tu (s) |  | 91.43 | 45.71 | 91.43 | 45.71 | 91.43 | 45.71 |
| CP Length Tcp (s) | Short | 3.21 |  |  |  |  |  |
|  | Normal | 11.43 |  |  |  |  |  |
|  | Long | 18.93 |  |  |  |  |  |
| Symbol Duration Ts (s) | Short | 94.64 | 48.92 | 94.64 | 48.92 | 94.64 | 48.92 |
|  | Normal | 102.86 | 57.14 | 102.86 | 57.14 | 102.86 | 57.14 |
|  | Long | 110.36 | 64.64 | 110.36 | 64.64 | 110.36 | 64.64 |
| Sub-frame Duration (s) |  | 617.14 |  |  |  |  |  |
| Number of Sub-frames per Frame |  | 8 |  |  |  |  |  |
| Idle Time per Frame (s) |  | 62.86 |  |  |  |  |  |

Table 11.3-1: OFDMA parameters for IEEE 802.16m

In addition to the full OFDM symbol, the proposed frame structure uses half OFDM symbol that consists of a CP with the same size of the CP of the full OFDM symbol, and the useful symbol part which is half of that of the full OFDM symbol. The proposed frame structure uses three different CP sizes: Short, Normal and Long.

# 11.4 Frame Structure

11.4.1 Basic Frame Structure

Super-Frame

Header

Super-Frame (20ms)

Frame (5ms)

Idle Time (62.86 us)

Sub-frame (617.14 us)

Normal CP

Short CP

Long CP

Figure 11.4-1: Propose basic frame structure supporting multiple CP sizes

The IEEE 802.16m basic frame structure is illustrated in Figure 11.4-1. Each 20ms super-frame is divided into four equally sized 5ms radio frames. Each frame further consists of eight equally sized sub-frames. Each sub-frame can be assigned for either downlink or uplink transmission/reception depending on the duplexing scheme. The sub-frame is defined for three different CP sizes, in which the number of OFDM symbols within the sub-frame is varied according to the CP sizes.

The sub-frame size is 617.14us, which is equal to six full OFDM symbols with Normal CP size. For Short CP size, the sub-frame consists of six full symbols and one half symbol with idle time of 0.71 us. For Long CP size, the sub-frame consists of five full symbols and one half symbol with idle time of 0.36 us. The idle time is needed to exactly match the same sub-frame size of 617.14 us. The CP size of one of the symbols within the sub-frame can be increased by the idle time instead of keeping the idle time.

For TDD mode, guard time is needed at the switching point between DL and UL periods. Irregular sub-frame is defined with additional idle time by reducing OFDM symbols within the sub-frame. The size of the idle time is chosen considering the cell size the CP size is aimed for.

| CP Sizes, s |  | # of OFDM Symbols |  | Idle Time, s |
| --- | --- | --- | --- | --- |
|  |  | Full | Half |  |
| Normal | 11.43 | 6 (5) | 0 (0) | 0 (102.86) |
| Short | 3.21 | 6 (6) | 1 (0) | 0.71 (49.29) |
| Long | 18.93 | 5 (4) | 1 (0) | 0.36 (175.71) |

( ) corresponds to irregular Sub-frame case

Table yy: Numerology for the Sub-frame with multiple CP sizes

11.4.2 Frame Structure Supporting Legacy Frames

16m BS’s view

Legacy MS’s view

16m MS’s view

Figure 11.4-3: Frame Structure supporting Legacy Frames

16m BS’s view

Legacy MS’s view

16m MS’s view

Figure 11.4-3: Frame Structure supporting Legacy Frames

----------------------------------------------------End of Text-----------------------------------------------------------------

# 4. References

[1] C802.16m-08/118r1, “Proposed 802.16m Frame Structure Baseline Content Suitable for Use in the 802.16m SDD”

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.