| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Frame Structure to Support Multiple CP Sizes with Fixed Sub-frame Size |  |
| Date Submitted | 2008-03-12 |  |
| Source(s) | Yungsoo Kim Jumi Lee Yu-Seok Kim Samsung Electronics Co., LTD | E-mail: y2.kim@samsung.com j.esther.lee@samsung.com yseok.kim@samsung.com |
| Re: | Call for Comments on IEEE C802.16m-08/118r1, Proposed 802.16m Frame Structure Baseline Content Suitable for Use in the 802.16m SDD |  |
| Abstract | This contribution proposes 802.16m frame structure to support multiple CP sizes with fixed size sub-frames. |  |
| Purpose | To be discussed and approved by TGm for use in the 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Frame Structure to Support Multiple CP Sizes with Fixed Sub-frame Size

Yungsoo Kim, Jumi Lee, Yu-Seok Kim

Samsung Electronics Co., LTD

# Introduction

This contribution proposes frame structure that supports multiple CP sizes to efficiently adapt to various communication environments from indoor to outdoor cells. The proposed frame structure design is based on a fixed size sub-frame, so that it supports coexistence of different CP sizes not only in adjacent cell/frequency, but also within the same cell/frame.

# Baseline Frame Structure

The baseline frame structure prepared by the Frame Structure Rapporteur Group [1] is based on a fixed CP size of 1/8-Tu with the time duration of 11.43 us, where Tu is the OFDMA useful symbol time. Because the legacy system is based on the OFDM symbol with 1/8 CP, the proposed frame structure represents a basis for the design of 16m system. However, a fixed size CP of 11.43 us can be a waste of resource for some environments such as indoor, while it may not be sufficiently long in a very large cell with severe multi-path propagation. Therefore, it is desirable to have multiple CP sizes to efficiently adapt to various communication environments.

The baseline frame structure content also proposes OFDMA parameters for three different CP sizes: Short, Normal, and Long. However, it does not explain how a frame structure can be designed using the different CP sizes. Moreover, blindly adopting multiple CP sizes in the frame structure design can lead to several important problems that severely impair deployment. First, in TDD mode, the DL and UL periods of the frames with different CP sizes are in general not time aligned. This can cause severe interference problem when different CP sizes are used. Another important drawback is that co-existence of different CP sizes within a frame is in general not allowed, as in the legacy system, and thus legacy support is not possible for the frame with other CP sizes.

This contribution proposes a frame structure design that provides a solution for these problems. It proposes to use three different CP sizes of Short, Normal, and Long, to efficiently adapt to various communication conditions. It also proposes to use fixed size sub-frame structure to address the problem of co-existence and legacy support.

# Proposed Text Change

Insert the following text into Proposed 802.16m Frame Structure Baseline Content.

---------------------------------------------------- Start of Text-----------------------------------------------------------------

# 11.3 OFDMA Parameters

| Nominal Channel Bandwidth (MHz) |  | 5 |  | 10 |  | 20 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Over-sampling Factor |  | 28/25 |  | 28/25 |  | 28/25 |  |
| Sampling Frequency (MHz) |  | 5.6 |  | 11.2 |  | 22.4 |  |
| OFDM Symbol Types |  | Full | Half | Full | Half | Full | Half |
| FFT Size |  | 512 | 256 | 1,024 | 512 | 2,048 | 1,024 |
| Sub-carrier Spacing (kHz) |  | 10.94 | 21.88 | 10.94 | 21.88 | 10.94 | 21.88 |
| OFDMA Useful Symbol Time Tu (s) |  | 91.43 | 45.71 | 91.43 | 45.71 | 91.43 | 45.71 |
| CP Length Tcp (s) | Short | 3.21 |  |  |  |  |  |
|  | Normal | 11.43 |  |  |  |  |  |
|  | Long | 18.93 |  |  |  |  |  |
| Symbol Duration Ts (s) | Short | 94.64 | 48.92 | 94.64 | 48.92 | 94.64 | 48.92 |
|  | Normal | 102.86 | 57.14 | 102.86 | 57.14 | 102.86 | 57.14 |
|  | Long | 110.36 | 64.64 | 110.36 | 64.64 | 110.36 | 64.64 |

Table 11.3-1: OFDMA parameters for IEEE 802.16m

In addition to the full OFDM symbol, the proposed frame structure uses half OFDM symbol that consists of a CP with the same size and the useful symbol part with half the size of those of the full OFDM symbol, respectively. The proposed frame structure supports three different CP sizes: Short, Normal and Long.

# 11.4 Frame Structure

11.4.1 Basic Frame Structure

Super-Frame

Header

Super-Frame (20ms)

Frame (5ms)

Idle Time (62.86 us)

Sub-frame (617.14 us)

Normal CP

Short CP

Long CP

Figure 11.4-xx: Proposed basic frame structure supporting multiple CP sizes

The IEEE 802.16m basic frame structure is illustrated in Figure 11.4-xx. Each 20ms super-frame is divided into four equally sized 5ms radio frames. Each frame further consists of eight equally sized sub-frames. Each sub-frame can be assigned for either downlink or uplink communications depending on the duplexing scheme. The sub-frame is defined for three different CP sizes, in which the number of OFDM symbols within the sub-frame is varied according to the CP size.

The sub-frame size is 617.14us, which is equal to six full-symbols with Normal CP size. For Short CP, the sub-frame consists of six full-symbols and one half-symbol with idle time of 0.36 us. For Long CP, the sub-frame consists of five full-symbols and one half-symbol with idle time of 0.71 us. The Short and Long CP sizes are chosen to best match the fixed sub-frame size of 617.14 us, and the idle time is further needed to exactly match the sub-frame size. As an alternative to having the idle time, the CP size in one of the OFDM symbols can be increased by the same amount.

The proposed basic frame structure can be used for both FDD and TDD modes. For TDD mode, guard time is needed at the switching point between DL and UL periods. Irregular sub-frame is defined to have an increased idle time by reducing the number of OFDM symbols in the sub-frame. The size of the idle time is chosen considering the cell size the CP size is aimed for.

| CP Sizes, s |  | # of OFDM Symbols |  | Idle Time, s |
| --- | --- | --- | --- | --- |
|  |  | Full | Half |  |
| Normal | 11.43 (11.43) | 6 (5) | 0 (0) | 0 (102.86) |
| Short | 3.21 ( 3.21) | 6 (6) | 1 (0) | 0.36 (49.29) |
| Long | 18.93 (18.93) | 5 (4) | 1 (0) | 0.71 (175.71) |

( ) corresponds to irregular sub-frame case.

Table yy: Numerology for the sub-frame with multiple CP sizes

Each super-frame starts with a super-frame header. The super-frame header indicates the CP size and the type of each sub-frame. The super-frame is preferably designed with the same CP size for all deployments to simplify the initial acquisition by the terminal.

11.4.2 Frame Structure Supporting Legacy Frames

16m BS’s view

Legacy MS’s view

16m MS’s view

Legacy Frame Duration

16m Frame Duration

Figure 11.4-yy: Frame Structure supporting Legacy Frames

The proposed 802.16m frame structure supports the legacy frames by distributing sub-frames to legacy and 16m. The co-existence of 16m and legacy is achieved by TDM in both DL and UL, as illustrated in Figure 11.4-yy. The sub-frames assigned to legacy have Normal CP size, while the sub-frames for 16m can have any CP sizes best suited to the deployment condition and/or the service type. For example, in the femto, pico or micro cell, Short CP is used for 16m, while Normal CP is used for legacy. In another example, Normal CP is used for bi-directional communication for both legacy and 16m, while Long CP is used for 16m multicast/broadcast services.

16m BS’s view

Legacy MS’s view

16m MS’s view

Legacy Frame Duration

16m Frame Duration

Figure 11.4-zz: Frame Structure supporting Legacy Frames

Another method of co-existence of 16m and legacy is to share UL resources in FDM, while TDM is used in the DL, as illustrated in Figure 11.4-zz. In the UL, certain frequency resources are reserved for legacy while the remaining frequency resources in each sub-frame are allocated to 16m. This can help increase the UL coverage of legacy in outdoor environments. When 16m uses either Long or Short CP, a guard band is further needed between the legacy and 16m bands in the UL.

----------------------------------------------------End of Text-----------------------------------------------------------------

# 4. References

[1] C802.16m-08/118r1, “Proposed 802.16m Frame Structure Baseline Content Suitable for Use in the 802.16m SDD