| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | 802.16m Frame structure for TDD Multi-Hop Relays in OFDMA domain |  |
| Date Submitted | 2008-04 -08 |  |
| Source(s) | Mariana Goldhamer Alvarion ltd. 21A, Ha Barzel Street Tel Aviv | E-mail: mariana.goldhamer@alvarion.com |
| Re: | [16jm] Relay Frame Structure - call for proposals |  |
| Abstract |  |  |
| Purpose | Actions: 1. Modification of ToC 2. Capture of the text in the SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# 802.16m Frame structure for TDD Multi-Hop Relays in OFDMA domain

Mariana Goldhamer

Alvarion Ltd.

# Introduction

The approach within the draft P802.16j/D3 includes two time intervals dedicated to Relay operation per MAC Frame. However, due to the limitation of the small number of available sub-frames in 802.16m SDD, this approach has serious performance limitations.

This contribution provides an OFDMA-domain solution to overcome this problem. The solution is enhanced with better spectral efficiency based on the definition of dedicated and shared allocations for access mode (BS/RS to SS) operation.

# Scarcity of sub-frames

Limitations occur due to the usage of time separation and the short MAC Frame duration. The limitation, system wise, is generated by having too many features to be supported in the time domain. Many of these features did not exist at the time when 802.16e was drafted. See below a list of features to be supported by an 802.16m system, noting that there are only 8 sub-frames per frame and a delay requirement of 10ms.

1. Legacy DL

2. Legacy Relay DL

3. Legacy UL

4. Legacy Relay UL

5. 16m Multicast/Broadcast Zone

6. 16m Access mode, Reuse 3

7. 16m Access mode, Reuse 1

8. 16m Access mode, MIMO Zone

9. 16jm Relay, Access mode DL

10. 16jm Relays, Access mode, UL

11. 16m Coexistence with Bluetooth

12. 16m Coexistence with 802.11

13. 16m Coexistence with UMTS/LTE.

The solution to this problem is to transfer part of the time-separated activities in the OFDMA domain.

# Legacy support

The Legacy support can be done by TDM in DL or by either one of TDM or FDM in UL. For simplicity, this contribution presents only the TDM approach for both DL and UL.

# Interference map

In the general case a Relay Station (RS) will have a number of RS surrounding it, which for the highest range and data traffic should be separated in frequency domain. For example, we will consider that the Relays have sector antennas in the access mode; in this configuration the interference created to one SS located at the cell edge is shown in the following figure:

RS 1

RS 2

RS 3

RS 4

SS

Fig. 1   The strongest interferers of a sector RS

With omni antennas there are also four interfering cells.

In order to separate the 4 cells which are interfering are needed 4 segments (sub-channel groups) in the OFDMA domain. These segments will allow for the maximum cell size and will be used essentially for increasing the SINR (signal to interference and noise ratio) of specific users, typically at the cell margin. A better spectral efficiency will be obtained if the links not interfering one which each other will be grouped in a “shared” allocation built from sub-channels dedicated to this usage.

For example, a deployment scenario described in SDD (fig. 2) is shown below. This “aggressive” deployment scenario suffers from interference at the intersection of “red” and “blue” passes, which conduct to low data rate or lack of coverage.

Dedicated

Segment 1

Dedicated

Segment 2

BS

RS2

RS1

RS1

RS2

Fig 2: SDD example for aggressive segment reuse

A better spectral efficiency and coverage will be obtained if dedicated and shared segments will be introduced. In fig. 2 there are areas around the RS cell center which can be reused in parallel. We introduce in fig. 3 the idea of the Shared segment. The reused spectrum can be appreciated from fig. 3:

Dedicated

Segment 1

Dedicated

Segment 2

Shared

Segment

BS

RS2

RS1

RS1

RS2

Fig. 3 Usage of dedicated and shared segments in Relay operation

If we make the simplified assumption that used spectrum is reflected by the coverage, in fig. 2 are used 9*2=18 squares while in fig. 3 are used 1+1+5=7 squares, representing a 18/7 = 250% better spectrum efficiency.

So the use of the Shared segment may significantly increase the spectral efficiency. Usage of the dedicated segment will make possible the increased cell size.

# Proposal

Insert text for SDD, clause 11.4.4 Relay Support in Frame Structure

Relay operation

11.4.4.1 TDD operation

Each of the Tx and Rx activities can be supported in both Relay directions (bi-directional Relay), using two time partitions.

Time partition 1 (BS: Tx, RS2n+1: Rx, RS2n: Tx)

The functional description of the BS/Relay operation during time partition 1 is given in Fig. 4. It can be seen that the Relay RS1 is the central point of the Receive (Rx) activity, while Relay RS2 is the central point of the Tx activity.

BS

RS 1

RS 2

RS 3

Figure 5    Operation during time partition 1 - TDD

The possible isolation for the first scenario is the SS-SS separation (90-100dB in NLOS for 100m) and the segment separation (25dB for using adjacent carriers and 40 dB for using alternate carriers). If the interference still persists, the scheduling of the interfering SSs shall be done in such a way that they are separated in time domain, even if the penalty is some delay for those SS. A possibility would be to schedule the interfering SSs in different sub-channels.

Time partition 2 (BS Rx, RS2n+1 Tx)

The functional description of the BS/Relay operation during time partition 2 is given in the figure 5. It can be seen that the Relay is the focal point of the Transmit (Tx) activity.

BS

RS 1

RS 2

RS 3

Fig. 5 Operation during time partition 2 - TDD

There is interference potential between:

  - 1. Transmitting SS to BS and the receiving SS from Relay (SS to SS interference). Scheduling of the interfering SSs using different sub-channels can resolve this situation.

  - 2. Transmitting SS to Relay and the BS (SS to BS interference). This scenario is less problematic, due to the higher separation distance.

Frame structure

The Frame structure is presented in Fig. 6. BS is considered to be in HOP 0. The first Relay is in HOP 1, the 2nd Relay in HOP 2, etc. The Frame partition starts with the legacy sub-frame, followed by the Time partition 1, another Legacy Frame for UL and Time partition 2.

To each functional behavior is allocated a partition in the OFDMA (frequency) domain, named “segment”. Each transmission will use the suitable partitions associated to a specific antenna.

UL and DL activities are mixed inside the Frame.

The first time partition includes the following segments:

- DL Segment which carries the BS and RS2n traffic. This segment will be able to carry at least two different STC (space-time coding)/MIMO modes: one for BS-SS communication and one for BS-RS communication. During the BS and RS2 transmissions the Relays RS1, RS3 are in the Receive Mode.

- UL Relay Segment carries:

  - Up-link traffic from the SSs (RS1, RS3, etc. access mode)

  - Backward link of the next hop Relay (RS2, RS4, etc.).

This segment may be either split in dedicated and shared sub-channel groups or alternatively different segments may be allocated to the dedicated and shared UL RS traffic. BS downlink traffic may be also scheduled during the shared part of the Relay segment, if it will not create interference.

The RS is isolated in the access activity (RS-MS) from the BS due to the different used sub-channel segment and the significant distance between RS and BS. The isolation is increased by the usage of directional antenna for the Relay access operation and feeding link (BS-RS link).

The Frame Control Header (FCH) will be sent in all the DL segments which are intended for different MIMO/STC modes or for different antennae. The FCH may be sent at the start of the multi-frame only.

The second time partition includes the following segments:

- BS / RS UL Segment carries:

  - BS access traffic (SS transmissions).

  - RS1, RS3 backward link

This segment will carry at least two different STC modes: one for BS-SS communication (sub-channel group for BS access mode) and one for BS-RS communication. Different sub-channel groups will be allocated to this activity.

During the BS UL transmissions the RS is in the Transmit Mode.

- DL Relay Segment carries:

  - Downlink RS1, RS3, etc. access traffic to the SSs associated with it. This segment may be split in dedicated and shared segments or different segments may be allocated to the dedicated and shared DL RS traffic. The shared segment can be also used by the up-link BS activity not creating interference to Relays.

  - Forward link to the next hop RS2, RS4, etc.

RS in HOP 2n+1 forward transmission to RS in HOP 2n+2 using STC/MIMO

Preamble

RS-FCH

Preamble

Shared Segment for RS in HOP 2n+1 transmission to SS and SS transmissions to (BS/RS in HOP 2n)

Shared segment for SS transmission to RS in HOP 2n+1 and (BS/RS in HOP 2n) transmissions to SS

Preamble

Dedicated Segment for RS in HOP 2n+1 transmission to SS

SS transmission to (BS/Relay in HOP 2n)

Preamble

BS-FCH

Dedicated segment for SS transmission to RS in HOP 2n +1

Preamble

RS in HOP 2n+2 backward transmission to RS in HOP 2n+1 using STC/MIMO

Preamble

RS in HOP 2n+1 backward transmission to (BS/RS in HOP 2n) using STC/MIMO

(BS/RS in HOP 2n) forward transmission to RS in HOP 2n+1 using STC/MIMO

(BS/RS in HOP 2n) transmission to SS

BS-RS FCH

BS-RS FCH

Preamble

Legacy

UL

Zone

Legacy

DL

Zone

Fig. 6    TDD Frame structure for 16m Relay

This scheme has the advantage of minimizing the number of switching points in the Relay operation.

### Propagation delays in Multi Hop-operation

The following Table illustrates the propagation delays in the multi-hop operation for 6 Hops.

Table 1  Propagation times

| Frame 1, BS DL | Frame 1, BS UL | Frame 2, BS DL | Frame 2, BS UL | Frame 3, BS DL | Frame 3, BS UL | Frame 4, BS DL |
| --- | --- | --- | --- | --- | --- | --- |
| BS->RS1 | RS1->BS | RS2->RS1 | RS3->RS2 | RS4->RS3 | RS5->RS4 | RS6->RS5 |
|  | RS1 -> RS2 | RS2->RS3 | RS3>RS4 | RS4->RS5 | RS5>RS6 |  |

As can be seen from the above table, the propagation time from BS to RS6 is only 3 MAC Frames.

End text for SDD

### Advantages

The advantages of this solution are:

- Lower MAC overheads

- Better granularity for resource allocations (in time domain the resource allocation is more or less fixed by the sub-frame size and number)

- Support for MIMO in BS-RS communication

-    Significant lower data traffic forward delays; only 1 frame is needed for 2 hops. This proposal takes advantage of the new sub-frame structure, which will reduce the 802.16m latency.

- Better spectral efficiency generated by the usage of “shared segments”.