| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | 802.16m Frame structure for FDD Multi-Hop Relay in OFDMA domain |  |
| Date Submitted | 2008-04 -08 |  |
| Source(s) | Mariana Goldhamer Alvarion Ltd. 21A, Ha Barzel Street Tel Aviv | E-mail: mariana.goldhamer@alvarion.com |
| Re: | [16jm] Relay Frame Structure - call for proposals |  |
| Abstract |  |  |
| Purpose | Actions: 1. Modification of ToC 2. Capture of the text in the SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# 802.16m Frame structure for FDD Multi-Hop Relay in OFDMA domain

Mariana Goldhamer

Alvarion Ltd.

# Introduction

This contribution addresses the Relay Frame structure for FDD operation.

# Interference map

The interference situation is similar with the TDD case.

In the general case a Relay Station (RS) will have a number of RS surrounding it, which for the highest range and data traffic should be separated in frequency domain. For example, we will consider that the Relays have omnidirectional antennas in the access mode; in this configuration the interference created to one SS located at the cell edge is shown in the following figure:

RS 1

RS 6

RS 4

RS 3

RS 7

RS 2

Inter-cell interference area

Strongest

Interferers

RS 5

Fig. 1   The strongest interferers of a omnidirectional RS

In case that are used sector antennas there are also four interfering cells.

In order to separate the four cells which are interfering are needed four segments (sub-channel groups) in the OFDMA domain. These segments will allow for the maximum cell size and will be used essentially for increasing the SINR (signal to interference and noise ratio) of specific users, typically at the cell margin. A better spectral efficiency will be obtained if the links not interfering one which each other will be grouped in a “shared” allocation built from sub-channels dedicated to this usage.

For example, a deployment scenario described in SDD (fig. 2) is shown below. This “aggressive” deployment scenario suffers from interference at the intersection of “red” and “blue” passes, which conduct to low data rate or lack of coverage.

Dedicated

Segment 1

Dedicated

Segment 2

BS

RS2

RS1

RS1

RS2

Fig 2: SDD example for aggressive segment reuse

A better spectral efficiency and coverage will be obtained if dedicated and shared segments will be introduced. In fig. 2 there are areas around the RS cell center which can be reused in parallel. We introduce in fig. 4 the idea of the Shared segment. The reused spectrum can be appreciated from fig. 3:

Dedicated

Segment 1

Dedicated

Segment 2

Shared

Segment

BS

RS2

RS1

RS1

RS2

Fig. 3 Usage of dedicated and shared segments in Relay operation

If we make the simplified assumption that used spectrum is reflected by the coverage, in fig. 3 are used 9*2=18 squares while in fig. 4 are used 1+1+5=7 squares, representing a 18/7 = 250% better spectrum efficiency.

So the use of the Shared segment may significantly increase the spectral efficiency. Usage of the dedicated segment will make possible the increased cell size.

# Proposal

Insert text for SDD, clause 11.4.4  Relay Support in Frame Structure

11.4.4.2 FDD operation

The FDD operation uses the frequency f1 for BS Tx and f2 for SS Tx; generally f1>f2. In order to avoid the Rx/Tx operation of the Relay, while using the same frequency, are needed two time partitions.

During time partition 1 the relay transmits on both f1 and f2, while during time partition 2 the relay receives on both f1 and f2. No duplexer is needed. The BS can work in full duplex mode. In case of full-duplex Relay operation, the time partitions are extended for the full available time.

Time partition 1 (BS-Tx, RS Rx)

The functional description of the BS/Relay operation during time partition 1 is given in the figure 4. It can be seen that the Relays in Hops 1,3,etc. are the central points of the Receive (Rx) activity, while the Relays in Hops 2,4,etc. are the central point of the Tx activity.

BS

RS 1

RS 2

RS 3

RS 4

f1

f2

f1

f2

f2

f1

f1

f2

f2f2

Fig. 4 Operation during time partition 1 - FDD

This time partition should include the following segments, corresponding to the fig. 4 activity:

- DL Segment which carries the BS traffic on f1. This segment should be able to carry at least two different STC (space-time coding)/MIMO modes: one for BS-SS communication and one for BS-RS communication. During the BS DL transmissions RS1, RS3, etc. are in the Receive Mode.

- UL Relay Segment carries:

  - Up-link traffic from the SSs (Relay Access mode) on f2, for RS1, RS3, etc.

  - Backward link from RS2, RS4, etc. on f2.

This segment may be either split in dedicated and shared sub-channel groups. Alternatively, different segments may be allocated to the dedicated and shared UL RS1, RS3, etc. traffic. BS downlink traffic may be also scheduled during the shared part of the Relay segment, if it will not create interference.

The RS is isolated in the access activity (RS-MS) from the BS due to the different used sub-channel segment and the significant distance between RS and BS. The isolation is increased by the usage of directional antenna for the Relay access operation and feeding link (BS-RS link).

Time partition 2 (BS Rx, RS Tx)

The functional description of the BS/Relay operation during time partition 2 is given in figure 5. It can be seen that the Relay is the focal point of the Transmit (Tx) activity.

BS

RS 1

RS 2

RS 3

RS 4

f2

f2

f2

f1

f1

f1

f1

f2

f1

Fig. 5 Operation during time partition 2 - FDD

This time partition includes the following segments:

- BS UL Segment carries:

  - BS access traffic (SS transmissions) on f2.

  - RS1, RS3, etc. backward link on f2

This segment will carry at least two different STC modes: one for BS-SS communication (sub-channel group for BS access mode) and one for BS-RS communication. During the BS UL transmissions the RS is in the Transmit Mode.

- DL Relay Segment carries:

  - Downlink RS access traffic to the SSs associated with it on f1. This segment may be split in dedicated and shared segments or different segments may be allocated to the dedicated and shared DL RS traffic. The shared segment can be also used by the up-link BS activity not creating interference to Relays.

  - Forward link to the next hop RS on f1.

Frame structure

The FDD Frame structure is shown in fig. 6.

The BS is considered to be in HOP 0. The first Relay is in HOP 1. The Frame partition starts with the BS DL, which is also relevant for the Relay Stations in the hop 2n.

To each functional behavior is allocated a segment in the OFDMA domain.

A Relay will transmit in two different directions in the same time (Bi-directional Relay). Each transmission will use the suitable segments associated to a specific antenna and the suitable frequencies.

UL and DL activities are mixed inside the Frame. The permutations used for UL and DL shall be compatible, but not necessary identical.

The Frame Control Header (FCH) will be sent in all the DL segments which are intended for different MIMO/STC modes or for different antennae. The FCH may be sent at the start of the multi-frame only.

RS in HOP 2n+1 forward transmission to RS in HOP 2n+2 using STC/MIMO – f1

Preamble

RS-FCH

Preamble

Shared Segment for

RS inHOP 2n+1 DL transmissions to SS – f1

Shared segment for SS up-link transmissions to RS in HOP 2n+1 – f2

Preamble

Dedicated Segment for RS in HOP 2n+1 DL transmissions to SS – f1

SS up-link transmissions to (BS/Relay in HOP 2n) including separation between shared/dedicated sub-channels – f2

Preamble

BS-FCH

Dedicated segment for SS up-link transmissions to RS in HOP 2n +1 – f2

Preamble

RS in HOP 2n+2 backward transmissions to RS in HOP 2n+1 using STC/MIMO – f2

Preamble

Preamble

RS in HOP 2n+1 backward transmissions to (BS/RS in HOP 2n) using STC/MIMO – f2

(BS/RS in HOP 2n) forward transmissions to RS in HOP 2n+1 using STC/MIMO – f1

(BS/RS in HOP 2n) DL transmission to SS, including separation between shared/dedicated sub-channels – f1

BS-RS FCH

BS-RS FCH

Legacy

DL

Zone

Legacy

UL

Zone

Fig. 6   FDD Frame structure

Based on fig. 6, the frame structure per each frequency is detailed in fig. 7 for f1 and fig. 8 for f2.

RS in HOP 2n+1 forward transmission to RS in HOP 2n+2 using STC/MIMO – f1

Preamble

RS-FCH

Preamble

Shared Segment for RS in HOP 2n+1

DL transmissions

to SS – f1

Preamble

Dedicated Segment for RS in HOP 2n+1 DL transmissions to SS - f1

Preamble

BS-FCH

Preamble

(BS/RS in HOP 2n) forward transmissions to RS in HOP 2n+1 using STC/MIMO – f1

(BS/RS in HOP 2n) DL transmission to SS, including separation between shared/dedicated sub-channels – f1

BS-RS FCH

Legacy

DL

Zone

Legacy

UL

Zone

Fig. 7  802.16m Relay Frame structure for operation on f1

Shared segment for SS up-link transmissions to RS in HOP 2n+1 – f2

SS up-link transmissions to (BS/Relay in HOP 2n) including separation between shared/dedicated sub-channels – f2

Dedicated segment for SS up-link transmissions to RS in HOP 2n +1 – f2

Preamble

RS in HOP 2n+2 backward transmissions to RS in HOP 2n+1 using STC/MIMO – f2

Preamble

RS in HOP 2n+1 backward transmissions to (BS/RS in HOP 2n) using STC/MIMO – f2

BS-RS FCH

Legacy

DL

Zone

Legacy

UL

Zone

Fig. 8   802.16m Relay Frame structure for operation on f2

End text for SDD

### Advantages

The advantages of this solution are:

- FDD operation with only two time-separated intervals

- No need for duplexer

- Support for MIMO in BS-RS communication

-    Significant lower data traffic forward delays; only 1 frame is needed for 2 hops. This proposal takes advantage of the new sub-frame structure, which will reduce the 802.16m latency.

- Better spectral efficiency generated by the usage of “shared segments”.