| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed 802.16m Frame Structure For Frequency Overlay Mode |  |
| Date Submitted | 2008-05-01 |  |
| Source(s) | Jaehee Cho Samsung Electronics DongSeung Kwon, Sungcheol Chang ETRI | Voice: +82-31-279-5596 E-mail: jaehee1.cho@samsung.com E-mail: dskwon@etri.re.kr |
| Re: | IEEE 802.16m-08/003r1 |  |
| Abstract | Frame structure that can support frequency overlay is proposed |  |
| Purpose | To be discussed and adopted by TGm for use in 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed 802.16m Frame Structure for multi-carrier support

Jaehee Cho

Samsung electronics

DongSeung Kwon, Sungcheol Chang

ETRI

# 1. Introduction

Including this contribution, C80216m-08_303, C80216m-08_327, C80216m-08_357 and C80216m-08_401 are dealing the operation of multi-carrier and framestructure. Reflecting the contributions, we revised our original proposed text.

# 2. Proposed Text

[Insert the following subclause after subclause 11.4.1.2]

11.4.1.3 Frame Structure to support multi-carrier operation

Figure 11.12 shows the proposed frame structure example for multi-carrier support. A number of narrow BW carriers can be aggregated to support wide BW operation. Thus, narrow BW and wide BW MSs can operate in the same spectrum. Each carrier can have its own superframe header (the location and structure is subject to the results of the DL control RG). Further, some carrier may have only part of superframe header or no superframe header. When the separation of center frequency between two adjacent carriers is multiples of subcarrier spacing, no guard subcarriers are necessary between adjacent carriers.

Figure 11.12: Example of the proposed frame structure to support multi-carrier operation

[Insert the following subclause in subclause 19]

19.1 Multi-carrier operation Principles

For multi-carrier operation, carriers are differentiated based on its functionalities and control channel configuration. The primary and secondary carriers are defined based on MAC/PHY functions.

- Primary carrier: Traffic and set of PHY/MAC controls (Set-P) exchange in the primary carrier.

- Secondary carrier: Traffic and set of PHY/MAC controls (Set-S) exchange in the secondary carrier.

- Set-S is basically the subset of Set-P. So the primary carrier is mostly in charge of maintaining full control of MS mobility, state and context.

- Further, Set-S can include some distinctive functions like returning to the primary carrier indicator.

- The categorization of control Set-P and Set-S is FFS.

Each carrier may have different control channel configuration.

- Fully configured carrier: All control channels (synchronization, broadcast, common and dedicated control signaling channels) are configured. Further, information regarding the other carriers can also be included in the control channels.

- Partially configured carrier: No or part of the control channels are configured.

Only the fully configured carrier can be the primary carrier. On the other hand, any carrier can be secondary carrier.

Following MC operations are identified:

- Carrier aggregation

  - MS shall always maintain its physical layer connection to the primary carrier.

- Fast switching

  - MS can switch its physical layer connection from the primary to the secondary carrier for an agreed time interval. During the interval, MS doesn’t need to maintain its physical layer connection to the primary.

References

[1] IEEE 802.16m-08/003r1, The Draft IEEE 802.16m System Description Document

[2] IEEE C802.16m-08/208r2, Proposed 802.16m DL Control Channel Scheme to Use More Subcarriers in Guard Band.

[3] IEEE C802.16m-08/143r1, Sub-carrier Alignment for IEEE 802.16m Multi-band Frame Structure.