| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed 802.16m Frame Structure for multi-carrier support |  |
| Date Submitted | 2008-05-15 |  |
| Source(s) | Jaehee Cho (Samsung Electronics)) Kamran Etemad (Intel) DongSeung Kwon, ByungJae Kwak, Sungcheol Chang (ETRI) Mo-Han Pong (Nortel) I-Kang Fu (MediaTek) Naftali Chayat (Alvarion) Richard Li, Chie-Ming Chou (ITRI) | E-mail: jaehee1.cho@samsung.com Kamran.etemad@intel.com dskwon@etri.re.kr mhfong@nortel.com IK.fu@mediatek.com naftali.chayat@alvarion.com richard929@itri.org.tw |
| Re: | IEEE 802.16m-08/003r1 |  |
| Abstract | Frame structure that can support frequency overlay is proposed |  |
| Purpose | To be discussed and adopted by TGm for use in 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed 802.16m Frame Structure for multi-carrier support

Jaehee Cho1), Kamran Etemad2), DongSeung Kwon3), ByungJae Kwak3), Sungcheol Chang3), Mo-Han Pong4), I-Kang Fu5), Naftali Chayat6), Richard Li7), Chie-Ming Chou7)

Samsung Electronics1), Intel Corportaion2), ETRI3), Nortel4), MediaTek5), Alvarion6), ITRI7)

# 1. Introduction

The current SDD text for multi-carrier allows operation of multiple carriers with different bandwidths, duplexing of more (or other) radio configurations in contiguous or non-contiguous bands under the same MAC instantiation. The SDD text also mentions that the MAC entity may support simultaneous presence of MSs with different capabilities, such as operation over one channel at a time only or aggregation across channels, operation.

This contribution addresses the requirements and consideration in frame structure and down link control channel structure to support Multicarrier feature as currently defined in SDD.

# 2. Proposed Text

[Proposed text 1]

[Insert the following subclause after subclause 11.4.1.2]

11.4.1.3 Frame Structure to support multi-carrier operation

The support for multiple RF carriers can be accommodated with the same frame structure used for single carrier support, however, some considerations in the design of protocol and channel structure may be needed to efficiently support this feature.

In general each MS operating under IEEE 802.16m standard is controlled by one RF carrier, here is called the primary RF carrier. When multi-carrier feature is supported, the system may define and utilize additional RF carriers to improve the user experience and QoS or provide services through additional RF carriers configured or optimized for specific services.

Figure 11.12 shows that the same frame structure would be applicable to both single carrier and multicarrier mode of operation. A number of narrow BW carriers can be aggregated to support effectively wider BW operation. Each carrier may have its own synchronization channel and superframe header (the location and structure is subject to the results of the DL control RG). Further, some carriers may have only part of superframe header.

The multicarriers involved in multiccarier operation may be in a contiguous or non-contiguous spectrum. When carriers are in the same spectrum and adjacent and when the separation of center frequency between two adjacent carriers is multiples of subcarrier spacing, no guard subcarriers are necessary between adjacent carriers.

Figure 11.12: Example of the proposed frame structure to support multi-carrier operation

[Proposed text 2]

[Insert the following subclause in subclause 11.x.6]

11.x.6 Multi-carrier Control Structure

The carriers involve in a multicarrier system, from one MS point of view, can be divided into two types:

- Primary carrier is the carrier where the BS and the MS exchange traffic and full PHY/MAC control information defined in 16m specification. Further, the primary carrier is in charge of delivering all control information for proper MS operation. Each MS shall have only one primary carrier.

- Secondary carrier is the carrier which the MS may use for traffic, only per BS’s specific allocation commands and rules received from the primary carrier. The secondary carrier may also include control signaling to support multicarrier operation..

Based on the primary and/or secondary usage, the carriers of a multi-carriers system may be configured differently as follows:.

- Fully configured carrier: A carrier for which all control channels including synchronization, broadcast, multicast and unicast control signaling are configured. Further, information and parameters regarding multi-carrier operation and the other carriers can also be included in the control channels.

- Partially configured carrier: A carrier with essential control channel configuration to support traffic exchanges during multi-carrier operation.

A primary carrier shall be fully configured while a secondary carrier may be fully or partially configured depending on usage and deployment model.

[Proposed text 3]

[Insert the following subclause in subclause 19]

19.1 Multi-carrier operation Principles

The following is common in all modes of multicarrier operation:

- The system defines N standalone primary RF carriers as defined in section 11.x.6, each fully configured with all synchronization, broadcast, multicast and unicast control signaling channels. Each MS in the cell is connected to and its state being controlled through only one of the primary carriers.

- In the multicarrier operation a common MAC can utilize radio resources in one or more of the secondary carriers as defined in section 11.x.6, while maintaining full control of MS mobility, state and context through the primary carrier.

- Some information about the secondary carriers including their presence and location shall be made available to the user through the primary carriers. The primary carrier may also provide user the information about the configuration of the secondary carrier.

- The resource allocation can span across multiple RF carriers. Link adaptation feedback mechanisms would need to incorporate measurements relevant to both primary and secondary carriers.

- The multi-carrier may be used in the downlink and/or uplink asymmetrically based on system load (i.e., for static/dynamic load balancing), peak data rate, or QoS demand.

- A primary RF carrier may dynamically utilize resources across multiple secondary RF carriers. Multiple primary RF carriers may also share the same secondary carrier.

- The multiple carriers may be in different parts of the same spectrum block or in non-contiguous spectrum blocks.

- Each user will be connected to only one primary carrier. A secondary carrier for a MS, if fully configured, may serve as primary carrier for other MS’s.

There are two scenarios to multicarrier deployment.

Scenario 1: All carriers in the system are fully configured to operate standalone and may support some users as their primary carrier.

Scenario 2: In this case, in addition to fully configured and standalone RF carriers the system also utilizes additional supplementary radio carriers optimized as data pipes for certain services or traffic types using limited control signaling capability. Such supplementary carriers may be used only in conjunction with a primary carrier and cannot operate standalone to offer 802.16m services for a MS.

In multi-carrier operation, MS can access multiple carriers. The following multi-carrier operations are identified:

- Carrier aggregation

  - MS shall always maintain its physical layer connection and monitor the control information on the primary carrier.

- Carrier switching

  - MS can switch its physical layer connection from the primary to the secondary carrier per BS’ instruction. When the MS is connected to the secondary carrier, the MS doesn’t need to maintain its physical layer connection to the primary carrier.

  - This mode may be used for the cases of single radio MS or non-contiguous spectrum.