| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Hybrid ARQ for synchronous allocation in distributed subcarrier mode |  |
| Date Submitted | 2008-05-05 |  |
| Source(s) | Alexei Davydov, Alexander Maltsev Intel Russia, Nizhny Novgorod, Ul. Turgeneva, 30 | E-mail: alexei.davydov@intel.com, alexander.maltsev@intel.com Voice: +7 831 4162444 |
| Re: | IEEE 802.16m-08/016r1 Call for Contributions on Project 802.16m System Description Document (SDD) Topic: Hybrid ARQ (protocol and timing) |  |
| Abstract | This contribution proposes synchronous non-adaptive HARQ for IEEE 802.16m operating in distributed subcarrier mode |  |
| Purpose | For discussion and adoption by IEEE 802.16m group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Hybrid ARQ for synchronous allocation in distributed subcarrier mode

Alexei Davydov, Alexander Maltsev

Intel Corporation

# Introduction

Hybrid Automatic Repeat Request (HARQ) is an effective mean to improve robustness of the system to link adaptation errors caused by channel estimation errors, channel quality information feedback delay, user mobility, etc. When a received packet is found to be in error through the use of Cyclic Redundancy Check (CRC), a retransmission request (or Negative Acknowledgment - NACK) will be sent to the transmitter. The transmitter will then retransmit the same packet or it’s portion until the initial packet is successfully decoded at the receiver or a maximum retransmission limit is reached. The difference with conventional ARQ protocol is that erroneous packets are not discarded by the receiver but stored in a buffer for the following joint demodulation with the next retransmission(s).

# Background

Different methods for control signaling overhead reduction are considered in scope of TGm project. Some of them will create the synchronous timing of HARQ retransmissions. For example, for applications that generate periodic traffic, a persistent scheduling may be used. Persistent scheduling relies on the constant characteristics of the traffic parameters during transmission session, such as predetermined packet size, transmission mode and packet arriving interval. For the persistent scheduling a fixed synchronous resource allocation is usually applied for the entire transmission session. In this case substantially reduction of signaling overhead may be achieved. It should be noted that the persistent resources are also be reserved for estimated HARQ retransmissions thus making the HARQ retransmissions in persistent allocation essentially synchronous.

Some level of signaling overhead reduction may be guaranteed for dynamic scheduling with synchronous HARQ. In the synchronous HARQ mode, the retransmissions have to occur at a pre-determined time-frequency instances. In general, the fact that retransmissions can occur only at a fixed time-frequency instances reduces some flexibility, such as possibility to postpone the retransmission or possibility to use the channel information for more effective allocation of the retransmissions. At the same time synchronous HARQ provides several benefits such as reduction of control signalling overhead (the resource allocation for the retransmission is assumed to be the same as in the original transmission).

Figure 1 shows the example of synchronous allocation of HARQ retransmissions for considered above examples.

# Problem Statement

All the above mentioned examples use synchronous allocation for each HARQ retransmission. That means that HARQ retransmission does not change from frame to frame and occurs at the same time-frequency allocation as the initial transmission. That means that for applications that require short latencies (comparing to channel coherence time) the channel is almost static across the retransmissions. In this case the expected post-processing Signal-to-Noise Ration (SNR) improvement due to retransmission is about 3dB for Chase Combining and provided by time diversity and coherent combining only.

To keep signaling overhead at low level no channel adaptation is usually assumed for persistent scheduling. In this case the usage of distributed subcarrier mode seems to be more reasonable since it provides the more robust data transmission, thus minimizing the potential amount of HARQ retransmissions. In such distributed subcarrier mode the frequency diversity (additionally to time diversity) may be provided for the each next retransmission thus giving additional efficiency and robustness for the synchronous allocation of HARQ retransmissions.

# Proposed solution

In this section we describe a method for improvement of the efficiency of the synchronous allocation of the HARQ retransmissions for distributed subcarrier mode by means of providing frequency and spatial diversity to the each next retransmission while keeping overhead almost at the same level.

To exploit frequency diversity it is proposed to apply additional predetermined sub channel offset to the original data burst allocation. In this case the fading experienced by specific subcarrier in data burst will change due to change in the burst position (see Figure 2). The new parameter that defines the additional sub channel offset (S-HARQ sub channel offset) may be signalled in control channel during original transmission or specified as a fixed parameter in the system description.

In case of vertical or diagonal MIMO mode the additional robustness for synchronous allocation of HARQ retransmissions may be guaranteed by exploiting the spatial diversity (additionally or separately to the frequency diversity). The spatial streams of the each retransmission may be cyclically rotated. The cyclic rotation of spatial signals s1,s2,…,sNss over Nssspatial streams is illustrated below with rotation offset equal to one for each new retransmission:

s1s2⋮sNsstransmission ⇒ sNsss1⋮sNss−11st retranmission ⇒ sNss−1sNss⋮sNss−22nd retranmission ⇒ ⋯.

# Comparative study

To demonstrate the efficiency of the proposed solution a link layer performance analysis for the aforementioned techniques has been performed. The system parameters used in the simulations are summarized in Table 1 [1].

Table 1. Link layer simulation parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| FFT Size | 1024 |  |
| Bandwidth | 10 | MHz |
| Permutation | DL PUSC | Distributed |
| Antenna setting | 1x1/2x2 | 2x2 SM, MMSE |
| TX/RX Ant. Corr. | 0.5/0.5 |  |
| Mobile speed | 3 | km/h |
| Retransmission delay | 10 | Ms |
| HARQ | Chase Combining |  |
| Packet Length | 44 bytes | VoIP active state |

In Figure 3 the generic Packet Error Rate (PER) versus Signal-to-Noise Ratio (SNR) reference curves are shown for the initial transmission for all modulation coding schemes (MCS) in the distributed subcarrier mode. The similar graphs with residual PER performance versus SNR are shown in Figure 4 for the conventional synchronous allocation method after the first HARQ retransmission. It can be seen from the figures that the first packet retransmission and Chase Combining provides about 4dB SNR improvement (3dB coherent combining, 1dB time diversity) comparing to the performance of the initial transmission. Figure 5 demonstrates the efficiency of the proposed solution with predetermined sub channel rotation in the same scenario. It can be seen that simple predetermined sub channel rotation scheme that require low overhead provides 1-2dB improvement over conventional synchronous allocation of HARQ retransmissions.

A similar comparison has been carried out for 2x2 vertical STC-B MIMO mode. Figure 6 shows the reference PER performance versus SNR for the first 2x2 MIMO transmission. The performance of the conventional synchronous allocation of HARQ retransmissions and proposed solution for MIMO systems (with predetermined spatial stream rotation, but without sub channel rotation) are presented in figure 7 and 8 respectively. It can be seen that spatial stream rotation provides 1-2 dB improvement over simple packet retransmission in MIMO mode.

# Text proposal

Insert the following text into SDD Section 10 – Media Access Control Sub-Layer [2]

-------------------------------  Text Start  ---------------------------------------------------

# 10.x Data/Control Plane

# 10.x.x Downlink scheduling

The BS may use data bursts rotation for synchronous allocation of HARQ retransmission to improve it’s efficiency. The rotation may be applied for the whole DL persistent allocation region or within it. In addition in case of vertical spatial multiplexing transmission spatial stream rotation may be used for further improvement. A predetermined data burst rotation offset and spatial stream rotation indicator will be signaled via L1/L2 control channel(s).

# 10.x.y Uplink scheduling

The BS may use data bursts rotation within UL persistent allocation region to improve efficiency of HARQ retransmission. In addition in case of vertical spatial multiplexing transmission spatial stream rotation may be used for further improvement. A predetermined data burst rotation offset and spatial stream rotation indicator will be signaled via L1/L2 control channel(s).

-------------------------------  Text End  ---------------------------------------------------

# References

[1] IEEE 802.16m-08/004, “802.16m Evaluation Methodology”

[2] IEEE 802.16m-08/003, “The Draft IEEE 802.16m System Description Document”

Figure 1. Conventional synchronous allocation of HARQ retransmissions in time-frequency domain

Figure 2.  Proposed synchronous allocation of HARQ retransmissions in time-frequency domain

Figure 3. PER versus SNR for the initial transmission (reference curves)

Figure 4. Residual PER versus SNR after the 1st retransmission for the conventional synchronous allocation of HARQ retransmissions

Figure 5. Residual PER versus SNR after the 1st retransmission for the proposed synchronous allocation of HARQ retransmissions

Figure 6. PER versus SNR for the initial vertical MIMO transmission (reference curves)

Figure 7. Residual PER versus SNR after the 1st retransmission for the conventional MIMO synchronous allocation of HARQ retransmissions

Figure 8. Residual PER versus SNR after the 1st retransmission for the proposed MIMO synchronous allocation of HARQ retransmissions