| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Equal-Gain Precoder for Closed Loop MIMO Transmission |  |
| Date Submitted | 2008-05-05 |  |
| Source(s) | Yu-Tao Hsieh, Chen-Ming Chen, Chien-Yu Kao, Chao-Wang Hwang, Pang-An Ting, Richard Li ITRI Shang-Ho Tsai NCTU | Voice: +886-3-5914854 E-mail: ythsieh@itri.org.tw |
| Re: | Call for Contributions of IEEE 802.16m_08/016r1 on the topic of DL MIMO schemes |  |
| Abstract | This contribution proposes a MIMO precoding scheme to improve the conventional precoding approach with fewer feedback overhead. |  |
| Purpose | Discussion and approval by the task group. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Equal-Gain Precoder for Closed Loop MIMO Transmission

Yu-Tao Hsieh, Chen-Ming Chen, Chien-Yu Kao, Chao-Wang Hwang, 
Pang-An Ting, Richard Li,

ITRI

Shang-Ho Tsai

NCTU

Introduction

In the IEEE 802.16m System Requirements [1], The MIMO technique with different antenna configurations is regarded as one of the basic components for improving the system capacity without expanding the channel bandwidth. It is well known that the precoding/beamforming approach is widely used in WirelessMAN- OFDMA Reference System with designated precoding vector/matrix [2]. It is also demonstrated that the existing precoding matrix in the Reference System features fixed amount of feedback overhead and searching complexity due to constant codebook size. In this contribution we propose an equal-gain precoding matrix for MIMO scenarios. We show in the following sections that the proposed precoding matrix will attain the similar system performance with less feedback overhead.

Equal-Gain Precoding Matrix

In the existing closed loop precoding MIMO techniques a set of precoding vectors/matrices are adopted based on the time-varying MIMO channel. The codebooks are designed independent of the number of transmit or receive antennas. On the other hand, the proposed equal-gain precoding matrix quantizes the precoding matrix using the equi-spaced constellations on the unit circle. The number of constellation points is formed by M=2q where q denotes the bitwidth of the codebook as shown in Figure 1 as an example. Note that although the consumed bit number for the precoding matrix may grow in the number of M by N where M is the number of transmit antennas and N is the number of receive antennas, the proposed precoding matrix can attain the similar system performance using a total of less feedback overhead than the existing precoding matrix.

Figure 1 Example of an equal-gain precoder with bitwidth q=3

Simulations

The simulation evaluates the system performance of the existing precoding matrix and the proposed equal-gain approach. We let M=4 and N=1. The feedback overhead of the existing precoding matrix is fixed at 6 bits per precoding MIMO transmission.

Proposed Texts

We suggest the following proposed texts

============== Start of proposed texts =================================================

11.x Physical layer

11.x.y DL MIMO schemes

11.x.y.z Precoding MIMO

In the precoding MIMO schemes, the equal-gain precoder with equi-spaced constellations can be taken into consideration for improving the system performance while decreasing the feedback overhead. Figure x shows an example.

Figure x Example of equal-gain precoder with encoding bitwidth of 3.

============== End of proposed texts ================================================

References

[1] M. Cudak, “802.16m System Requirements,” IEEE 802.16m-07/002r4

[2] IEEE P802.16-2005, Section 8.4.5.4.10.15.