| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Equal-Gain Precoders for Closed Loop MIMO Transmission |  |
| Date Submitted | 2008-05-13 |  |
| Source(s) | Yu-Tao Hsieh, Cheng-Ming Chen, Chien-Yu Kao, Chao-Wang Huang, Pang-An Ting, Richard Li ITRI Shang-Ho Tsai NCTU | Voice: +886-3-5914854 E-mail: ythsieh@itri.org.tw |
| Re: | Call for Contributions of IEEE 802.16m_08/016 on the topic of DL MIMO schemes |  |
| Abstract | This contribution proposes an equal-gain MIMO precoding scheme to simplify the conventional precoding approach while preserving the system performance. |  |
| Purpose | Discussion and approval by the task group. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Equal-Gain Precoders for Closed Loop MIMO Transmission

Yu-Tao Hsieh, Cheng-Ming Chen, Chien-Yu Kao, Chao-Wang Huang, 
Pang-An Ting, Richard Li

ITRI

Shang-Ho Tsai

NCTU

Introduction

In the IEEE 802.16m System Requirements [1], The MIMO technique with different antenna configurations is regarded as one of the basic components for improving the system capacity without expanding the channel bandwidth. It is well known that the precoding/beamforming approach is widely used in WirelessMAN- OFDMA Reference System with a set of designated precoding vectors/matrices. It is also demonstrated that the existing precoding matrix in the Reference System features fixed amount of feedback overhead and searching complexity due to constant codebook size. In this contribution we propose an equal-gain precoding matrix for MIMO scenarios. We show in the following sections that the performance of the proposed precoder is similar to the codebook-based approach in the Reference System. The benefit of the equal-gain precoding lies in its simple encoding operations since only phase shifts are needed in the precoding process.

Equal-Gain Precoding Matrix

In the existing closed loop precoding MIMO techniques one among the pre-defined precoding matrices are chosen based on the MIMO channel. The codebook sizes for different number of transmit antennas can be the same, e.g. 3 bits or 6 bits. On the other hand, the proposed equal-gain precoder givens the quantized precoding matrix using the equi-spaced constellations on the unit circle, where the number of constellation points is formed by M =2q where q denotes the bitwidth of the codebook as shown in Figure 1. Note that the feedback bit number for the precoding matrix may grow in the product of Nt by Ns, where Nt is the number of transmit antennas and Ns is the number of streams.

Figure 1 Example of an equal-gain precoder with bitwidth q=2

Fig. 2 shows the block diagram of a MIMO precoding system, where the vectors f and g are called the precoder

and the postcoder, respectively. At the first stage, assume one transmit symbol x (can be complex such as QPSK and M-QAM) is multiplied by the Nt x 1 precoding vector f . For equal gain precoding, all elements in f have unit magnitude. The MIMO channel H is represented by an Nr x Nt matrix. Then the received signal is given by

(1)

where n is the noise vector. The receive vector r is multiplied by the 1xNr postcoder g to form z, i.e

(2)

In general, we will let the post-coder be

(3)

and the output data can be expressed to be

(4)

where  is the gain factor as

(5)

and

.          (6)

Figure 2. Block diagram of a MIMO precoder/beamformer

Simulations

This section evaluates the system performance of the existing precoding matrix and the proposed equal-gain approach. The number of data streams is fixed at Ns=1. The elements in the MIMO channel are assumed as iid complex Gaussian. Each element in the Nt -1 outputs uses 2-bit quantization. The first element in the precoding vector is normalized to be unity. We compare the performance of the following precoders: STBC with antenna selection, equal gain (EG) precoder, Grassmannian precoder [2], and the optimal precoder. Figure 3 presents the performance result with 2T1R (2 transmit and 1 receive) or 2T2R configurations. The performance of the EG precoder (feedback with 2 bits) is close to the Grassmannian precoder (feedback with 2 bits) and is worse than the optimal SVD solution within 1 dB. Figure 4 shows the performance comparison in 4T1R (4-transmit and 1-receive) and 4T2R configurations. We observe that the EG precoder (feedback with 6 bits) and the Grassmannian precoder (feedback with 6 bits) have nearly the same performance, and outperform the antenna selection by around 3 dB in 4T1R case and around 2.2 dB in 4T2R case. The optimal precoder without quantization outperforms the EG and Grassmannian precoders by 1.1 dB in 4T1R case and by 1 dB in 4T2R case.

Although the performance is similar between the Grassmannian and EG precoders, there is one advantage in using the equal-gain precoder. That is the elements of the equal-gain precoder are composed of equi-spaced constellations on the unit circle. Thus the phase shift operations can be used instead of complex-valued multiplication operations. Moreover, when the encoded bits are less than 3, the elements in the precoder will contain only {1,-1, j, -j} as shown in Figure 1. The multiplication operations can be replaced by IQ data interchange or sign change. In this case the encoding process will become much easier than the conventional precoding procedures.

Figure 3. Bit error probability comparison for different precoders with two transmit antennas.

Figure 4. Bit error probability comparison for different precoders with four transmit antennas.

Conclusions

We propose to provide the equal-gain precoder as a candidate for the DL MIMO precoding/beamforming scheme. From the simulation results it can be observed that the equal-gain precoder performs close to the Grassmannian precoder with two and four transmit antennas. Moreover, the equal-gain precoder has additional advantages such as low storage requirement for the precoding codebooks, and encoding simplicity on the transmit side. Therefore it can be a promising approach to be considered for the advanced DL MIMO techniques.

Proposed Texts

We suggest the following proposed texts

============== Start of proposed texts =================================================

11 Physical layer

11.x DL MIMO schemes

11.x.y DL MIMO Precoding

In the precoding MIMO schemes, the equal-gain precoder with elements encoded by the equi-spaced constellations on the unit circle can be taken into consideration. Figure x shows an example.

Figure x Example of equal-gain precoder with quantization of 2 bits.

============== End of proposed texts ================================================

References

[1] M. Cudak, “802.16m System Requirements,” IEEE 802.16m-07/002r4

[2] D. J. Love, R. W. Heath and T. Strohmer, “Grassmannian beam-forming for multiple-input multiple-output wireless systems," IEEE Trans. Inform. Theory, vol. 49, pp.2735-2747, Oct. 2003.