| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Pilot Allocation in High-Mobility Resource Block |  |
| Date Submitted | 2008-05-09 |  |
| Source(s) | Lai-Huei Wang, Yu-Tao Hsieh, Jen-Yuan Hsu, Rong-Terng Juang, Pang-An Ting, Richard Li ITRI Pei-Kai Liao, Chih-Yuan Lin, Ciou-Ping Wu, Paul Cheng MediaTek Inc. | Voice: + 886 3 5914854 E-mail: ythsieh@itri.org.tw |
| Re: | IEEE 802.16m-08/003r1,”Call for Contributions on Project 802.16m System Description Document (SDD)”. |  |
| Abstract | A pilot allocation method to reduce the intercarrier interference effect for high-mobility conditions |  |
| Purpose | Discussion and approval by the task group. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Pilot Allocation in High-Mobility Resource Block

Lai-Huei Wang, Yu-Tao Hsieh, Jen-Yuan Hsu, Rong-Terng Juang, Pang-An Ting, Richard Li

ITRI

Pei-Kai Liao, Chih-Yuan Lin, Ciou-Ping Wu, Paul Cheng

MediaTek

# Introduction

The 802.16m system requirements [1] cover the performance demand for subscriber stations at high mobility up to 350km/hr. At such high mobility, the inter-carrier interference (ICI) caused by Doppler spread is severe and greatly reduce throughput for link with high signal to noise power ratio (SNR). For 802.16e systems, the performance, such as packet error rate (PER) or data throughputs, is bounded by the ICI even in a high SNR environment. In [2], we have presented pilot design in high-mobility zone for 802.16m systems. It has been shown by simulations in [2] the system throughput can be improved by up to 50 percents with the proposed pilot design with a velocity up to 350km/hr.

In [3], control and data multiplexing scheme within a data mini-frame has been proposed as Fig.1. In the scheme, the first two symbols are used for control zones and the following 4 symbols are used for data zones. In the data zone, the dedicated pilots are provided based on needs in each specific resource block. For example, mobile station (MS) 1 who is far away from the base station (BS) may need denser pilots on frequency and the MS 2 who is on a train with a velocity of 350km/hr may need denser pilots on time.

Figure 1. Control and data multiplexing scheme in [3]

Following the previous work, we address the issue of high-mobility surroundings. In the data zone in Fig.1, we propose using the anti-polar pilot pattern in [2] for high-mobility environments. Fig.2 depicts an instance of the procedure for determination of pilot pattern. First, MS measures CINR by the common pilots broadcasted by BS periodically. Then, the MS may estimate the velocity roughly via the variation of CINR. Once high velocity is detected, the MS feedbacks the request of the dedicated pilot pattern for high-mobility to BS. With the high-mobility pilot pattern, the MS can estimate the more accurate ICI and CCI information and feedback them to BS.

Figure 2. Procedure for determination of pilot pattern

In addition to the scheme above, we also propose an alternate way of pilot allocation. By decreasing the number of antennas, the pilot patterns with two-stream pilots on adjacent subcarriers can be modified to the anti-polar pilots in [2] where pilots are adjacent two by two. Take the pilot pattern of MediaTek [4] as an example. The original 4-stream pilot pattern in Fig.3 (a) can be transferred to the anti-polar pilot pattern in Fig.3 (b) by decreasing the stream number from four to two. As well, the 2-stream pilot pattern can be transferred to the anti-polar pilot pattern for single stream.

(a) 4-stream pilot pattern of MediaTek         (b) Proposed 2-stream pattern

Figure 3. Pilot allocation from 4-stream to 2-stream pattern with the proposed pilot pattern

# Examples of pilot allocation for SISO and 2x2 MIMO in high-mobility resource block

For the data zone in Fig.1, examples of the proposed pilot patterns of SISO and 2x2 MIMO for high-mobility MSs are showed in Fig.4 (a) and 4 (b), respectively. Therein, the resource block is of size 18 sub-carriers by 4 symbols. Focusing on the problem of ICI in high-mobility surroundings, we arrange the pilots to be adjacent two by two and modulate the two adjacent pilots as anti-polar form. It can be shown that with such pilot pattern, ICI can be self-canceled without complicated computation. As a result, we have the near ICI-free performance of channel estimation without canceling the ICI on each pilot. The simulation results in [2] show that the accurate channel estimation gives rise to significant improvement of the performance of ICI cancellation on data sub-carriers.

(a) SISO           (b) 2x2 MIMO

Figure 4. Pilot patterns for high-mobility resource block

In another scheme, by decreasing the stream number, we also propose the anti-polar pilot pattern with one or two streams transferred shown in Fig.3 (b) from the original pilot pattern in Fig.3 (a) with two or four streams, respectively. The resource block is of size 18 sub-carriers by 6 symbols.

# Proposed SDD Text

11.5.3.2 Dedicated pilot structure

Resource blocks with dedicated pilots can be allocated on user’s demand for specific scenarios (e.g., high-mobility situations). Users can be assigned resource blocks with the same pilot allocation as that for low-mobility users, while the adjacent pilot subcarriers in frequency are grouped for high-mobility support. and all pilots in each group are used for the same transmit antenna. There are at most two antennas used for high-mobility support in the pilot structure for low-mobility usage with 4-antennas.

# Reference

[1] IEEE 802.16m-07/002r4, “TGm System Requirements Document”

[2] IEEE C80216m-08/135r3, “Pilot Design for High-mobility Zone in 802.16m”

[3] IEEE C80216m-08/214r1, “16m Downlink Common Pilot Structure for TDM Control Structure”

[4] IEEE C802.16m-08/139r2, “Design Considerations of Pilot Structures for Downlink MIMO Transmission”