| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Uplink Pilot Design for 802.16m |  |
| Date Submitted | 2008-05-05 |  |
| Source(s) | Jia-Hao Wu, Lai-Huei Wang, Chen-Ming Chen, Yu-Tao Hsieh, Pang-An Ting, Richard Li ITRI | Voice: + 886 3 5914854 E-mail: ythsieh@itri.org.tw |
| Re: | Call for Contributions of IEEE 802.16m_08/016r1 on the topic of “Uplink Pilot Structures” |  |
| Abstract | Proposal of the uplink pilot allocation method in 802.16m |  |
| Purpose | Discussion and approval by the task group. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Uplink Pilot Design for 802.16m

Jia-Hao Wu, Lai-Huei Wang, Chen-Ming Chen, Yu-Tao Hsieh, Pang-An Ting, Richard Li

ITRI

# Introduction

According to IEEE 802.16m system requirement document (SRD)[1], the system performance targets at least two times of WirelessMAN-OFDMA reference systems. In order to achieve the target system performance in terms of user throughput and VoIP capacity, we may need to adopt MIMO techniques to enhance spectrum efficiency (SE). Furthermore, we may need to make transmission overhead as low as possible without abusing transceiver performance.

In this contribution, we propose several pilot allocation schemes for different kinds of resource blocks. In contrast to 802.16e uplink tile structure, the pilot allocation schemes here are based on 12 (sub-carriers) x 6 (symbols) and 18x6 resource blocks. As we show later, the pilot overhead is reduced compared with 802.16e uplink transmission. And we will show the resultant channel estimation performance. Furthermore, the orthogonal code base SDMA pilot allocation is then introduced to save the pilot overhead when multi-stream transmission (SDMA or MIMO) is enabled.

# Proposed pilot allocation for 12X6 resource block scheme

Fig. 1 and Fig. 2 illustrate the pilot allocation schemes for 12x6 resource block size. We consider the 4-stream cases. Although in 802.16m SRD, it defines only 2x4 scheme as the target MIMO configuration for uplink transmission, consideration of 4-stream uplink transmission may render the pilot allocation flexible for Multi-user MIMO (MU-MIMO) environments. In addition, 4-stream pilot allocation can be easily degenerated to fit the 2-stream or single stream transmission schemes.

In mobile cellular systems, we may suffer from different kinds of channel delay spread which results in different frequency selectivity. In order to reduce pilot overhead and simultaneously cope with the variation of frequency selectivity, we propose moderate-overhead pilot allocation scheme with 1/12 = 8.3% overhead for 1-stream and low-overhead pilot allocation scheme with 1/18 = 5.6% overhead for 1-stream. (The uplink training overhead for LTE is 1/7 = 14.3%)

From Fig. 1, we group the four adjacent pilots as a pilot cluster. The pilot clusters are allocated to the resource block boundary and close to the corners. By this way we can minimize the boundary issues in channel estimation which may degrade channel estimation accuracy due to extrapolation. Furthermore, Fig. 2 illustrates a similar pilot allocation scheme with Fig. 1. We note that these two pilot cluster allocation schemes are the same. The only difference is the pilot tone for the specific stream may change its relative location in a pilot cluster. And we can evaluate channel estimation performance of these allocation schemes by computer simulation.

# Proposed pilot modulation scheme for Multi-stream and SDMA transmission

# In order to enhance spectral efficiency, multi-user MIMO techniques such as SDMA may be adopted in 802.16m. As a result, we need to support multiple-stream pilot-aided channel estimation in uplink transmission.

(Blind channel estimation is time-consuming and complicated so that it’s not feasible for mobile wireless environment ) To support multi-stream pilot-aided channel estimation algorithm and save the pilot overhead, we propose a pilot modulation scheme which makes modulation sequence mutually orthogonal for different streams (different users or antennas). Fig. 3 illustrates a simple example about this orthogonal pilot modulation scheme.

For slow time-varying channel, we may assume the channel doesn’t change in a resource block time slot. As an example, for the specific pilot subcarrier, we can collect received pilot tones in corresponding symbols and the received signal can be approximately written as:

If we can make pilot vectors  and  mutually orthogonal, then we can separately estimate channel of stream 0 and channel of stream 1 by projection  on  and . For example, .

Fig. 4 and Fig. 5 give an example of orthogonal modulated pilots for 12x6 resource block pilot allocation. We can see that the pilot overhead is saved 50% when 4-stream transmission.

Fig. 3

# Proposed pilot allocation for 18X6 resource block scheme

For 18x6 resource block scheme, the similar pilot allocation can be applied. The number of pilots in a resource block may increase to keep the pilot spacing. Fig. 6 illustrates the one and two-stream pilot allocation scheme. For multi-user SDMA, the orthogonal modulation scheme may be adopted to save the pilot overhead. In Table. 1, we summarize the pilot overhead.

Table 1

|  | 12x6 1-stream | 12x6 2-stream | 12x6 4-stream | 12x6 4-stream Orthogonal |
| --- | --- | --- | --- | --- |
| Moderate Overhead | 8.3% | 16.7% | 33.3% | 16.7% |
| Low Overhead | 5.6% | 11.1% | 22.2% | 11.1% |
|  | 18x6 1-stream | 18x6 2-stream | 18x6 4-stream | 18x6 4-stream Orthogonal |
| Overhead | 7.4% | 14.8% | 29.6% | 14.8% |

# Channel estimation performance of proposed pilot allocation schemes

# Proposed SDD Text

[Chapter] Physical Layer

………

[Section] 11.X Uplink Multiple Access 
………

[Subsection] 11.X.Y Uplink Pilot Pattern Design Consideration

Uplink pilot pattern design should take spectral efficiency and channel estimation performance into consideration. In order to apply multi-user MIMO techniques to enhance spectral efficiency, we shall consider multiple stream pilot allocation patterns. Furthermore, different pilot patterns should be designed to cope with diverse channel conditions to maintain channel estimation accuracy and keep the pilot overhead as low as possible.

Fig. X and Fig. Y give an example of uplink pilot patterns based on multiple-stream 12 (subcarriers) x 6 (symbols) resource blocks.

[Other additions]

………

# Reference

- IEEE 802.16m-07/002r4, “IEEE 802.16m system requirements.”

- IEEE 802.16m-07/037r2, “Draft IEEE 802.16m evaluation methodology.”

- IEEE C802.16m-08/139r2, “Design Considerations of Pilot Structures for Downlink MIMO Transmission.”