| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Uplink Pilot Design for 802.16m |  |
| Date Submitted | 2008-05-09 |  |
| Source(s) | Jia-Hao Wu, Lai-Huei Wang, Cheng-Ming Chen, Yu-Tao Hsieh, Pang-An Ting, Richard Li ITRI | Voice: + 886 3 5914854 E-mail: ythsieh@itri.org.tw |
| Re: | Call for Contributions of IEEE 802.16m_08/016r1 on the topic of “Uplink Pilot Structures” |  |
| Abstract | Proposal of the uplink pilot allocation method in 802.16m |  |
| Purpose | Discussion and approval by the task group. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Uplink Pilot Design for 802.16m

Jia-Hao Wu, Lai-Huei Wang, Cheng-Ming Chen, Yu-Tao Hsieh, Pang-An Ting, Richard Li

ITRI

# Introduction

According to IEEE 802.16m system requirement document (SRD)[1], the system performance targets at least two times of WirelessMAN-OFDMA reference systems. In order to achieve the target system performance in terms of user throughput and VoIP capacity, we may need to adopt MIMO techniques to enhance spectrum efficiency (SE). Furthermore, we may need to make transmission overhead as low as possible without abusing transceiver performance.

In this contribution, we propose several pilot allocation schemes for different kinds of resource blocks. In contrast to 802.16e uplink tile structure, the pilot allocation schemes here are based on 12 (sub-carriers) x 6 (symbols) and 18x6 resource blocks. As we show later, the pilot overhead is reduced compared with 802.16e uplink transmission. And we will show the resultant channel estimation performance. Furthermore, the orthogonal code base SDMA pilot allocation is then introduced to save the pilot overhead when multi-stream transmission (SDMA or MIMO) is enabled.

# Proposed pilot allocation for 12X6 resource block scheme

Fig. 1 illustrates the pilot allocation schemes for 12x6 resource block size. We consider the 4-stream cases. Although in 802.16m SRD, it defines only 2x4 scheme as the target MIMO configuration for uplink transmission, consideration of 4-stream uplink transmission may render the pilot allocation flexible for multi-user MIMO (MU-MIMO) environments. In addition, 4-stream pilot allocation can be easily degenerated to fit the 2-stream or single stream transmission schemes.

In mobile cellular systems, the mobile station may suffer from different kinds of channel delay spread which results in different frequency selectivity. In order to reduce pilot overhead and simultaneously cope with the variation of frequency selectivity, we propose moderate-overhead pilot allocation scheme with 1/12 = 8.3% overhead for 1-stream (see Fig.1 (a), (b)) and low-overhead pilot allocation scheme with 1/18 = 5.6% overhead for 1-stream (see Fig.1 (c), (d)). Note that the uplink training overhead for LTE is 1/7 = 14.3%.

From Fig. 1, we group the four adjacent pilots as a pilot cluster. The pilot clusters are allocated to the resource block boundary and close to the corners. By this way we can minimize the boundary issues in channel estimation which may degrade channel estimation accuracy due to extrapolation. Fig. 1 (a) and (c) (or (b) and (d)) have the same pilot cluster allocation but the pilot positions of specific stream in a pilot cluster are different. And we can evaluate channel estimation performance of these allocation schemes by computer simulation.

# Fig. 1

# Proposed pilot allocation for 18X6 resource block scheme

For 18x6 resource block scheme, the similar pilot allocation can be applied. The number of pilots in a resource block may increase to keep the pilot spacing. Fig. 2 illustrates an example of 4-stream pilot allocation schemes.

Fig. 2

# Channel estimation performance of proposed pilot allocation schemes

Table 1 shows the simulation parameters used in resource block channel estimation performance evaluation. The total power of pilots in a resource block is normalized for the purposes of fairness. From the fig. 3, the suggested low-overhead scheme (see. Fig. 1) discussed above has rarely channel estimation performance loss compared with moderate-overhead scheme at low SNR. Furthermore, when operating at high mobility environment, the suggested low-overhead and moderate-overhead channel estimation performance are very close. The selection of pilot allocation schemes depends on how accuracy of channel state information (CSI) we need. For multi-stream transmission case, accurate CSI is desirable to enhance MIMO detection performance.

Fig. 4 shows the channel estimation performance of moderate-overhead scheme and IEEE 802.16e tile structure. Note that the total power of pilots in a resource block is normalized.

Table 1

| Channel Mode | ITU-VA |
| --- | --- |
| Resource Block | 12(subcarriers)x6(symbols) |
| Pilot Boosting | 2.5 dB for 16m Moderate overhead pilots |
| Stream # | 2 |
| FFT size | 1024 |
| Channel Estimation Scheme | 2D-MMSE |
| Channel Estimation Parameters | Match to channel condition |

Fig. 3

Fig. 4

# Proposed SDD Text

[Chapter] Physical Layer

………

[Section] 11.X Uplink Multiple Access 
………

[Subsection] 11.X.Y Uplink Pilot Pattern Design Consideration

Uplink pilot pattern design should take spectral efficiency and channel estimation performance into consideration. In order to apply multi-user MIMO techniques to enhance spectral efficiency, we shall consider multiple stream pilot allocation patterns. Furthermore, different pilot patterns should be designed to cope with diverse channel conditions to maintain channel estimation accuracy and keep the pilot overhead as low as possible.

Fig. X and Fig. Y give examples of uplink pilot patterns based on multiple-stream 12 (subcarriers) x 6 (symbols) resource blocks and 18 (subcarriers) x 6 (symbols) resource blocks, respectively.

Fig. Y

# Reference

- IEEE 802.16m-07/002r4, “IEEE 802.16m system requirements.”

- IEEE 802.16m-07/037r2, “Draft IEEE 802.16m evaluation methodology.”

- IEEE C802.16m-08/139r2, “Design Considerations of Pilot Structures for Downlink MIMO Transmission.”