| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | New Preamble Sequence Design Using Quasi Complete Complementary Codes for IEEE 802.16m |  |
| Date Submitted | 2007-05-05 |  |
| Source(s) | Pei-Kai Liao and Paul Cheng MediaTek Inc. No.1, Dusing Road 1, Science-Based Industrial Park, Hsinchu, Taiwan 300, R.O.C. Chao-Yu Chen, Kuei-Ying Lu, and Chi-chao Chao Institute of Communications Engineering, National Tsing Hua University, Hsinchu 30013, Taiwan, R.O.C. Yih-Guang Jan and Yang-Han Lee Tamkang University, Taiwan, R.O.C. Shiann-Tsong Sheu National Central University, Taiwan, R.O.C. | pk.liao@mediatek.com paul.cheng@mediatek.com d919609@oz.nthu.edu.tw g9564517@oz.nthu.edu.tw ccc@ee.nthu.edu.tw yihjan@ee.tku.edu.tw yhlee@ee.tku.edu.tw stsheu@ce.ncu.edu.tw |
| Re: | IEEE 802.16m-08/016, “Call for Contributions on Project 802.16m System Description Document (SDD)” |  |
| Abstract | This contribution proposes a new preamble sequence based on quasi complete complementary codes, which have low peak-to-average power ratios and good auto-correlation/cross-correlation properties, as well as admit efficient encoding and decoding algorithms. |  |
| Purpose | Propose to be discussed and adopted by TGm for the use in Project 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

New Preamble Sequence Design Using Quasi Complete Complementary Codes for IEEE 802.16m

Pei-Kai Liao and Paul Cheng

MediaTek Inc.

Chao-Yu Chen, Kuei-Ying Lu, Chi-chao Chao

National Tsing Hua University

Yih-Guang Jan and Yang-Han Lee

Tamkang University

Shiann-Tsong Sheu

National Central University

- Introduction

Cell search is an important issue in cell-based communication systems.  Before data signals can be transmitted, the preamble sequences assist cell search and establishment of the initial radio link. Each base station has its own cell-specific preamble sequence, and the mobile station decides which base station to connect to based on the received preamble. Therefore, the preamble sequences employed greatly affect the performance of cell search.

Two basic requirements for the preambles in orthogonal frequency division multiplexing (OFDM) systems are low peak-to-average power ratios (PAPRs) and low cross-correlations between any two distinct preamble sets. The pseudo-noise (PN) preamble sequences in 802.16e are determined by using massive computer search [1]. No structures of the employed preamble sequences are known.

In this document, we propose a preamble sequence based on quasi complete complementary codes (QCCCs), which comprise several sets of quasi auto-complementary codes (QACCs). The constructed QCCCs have low PAPRs and good auto-correlation/cross-correlation properties. Simulation results show that the proposed QCCC-based preambles perform as well as the computer-searched preambles employed in 802.16e, and the rich algebraic structures of QCCCs admit efficient encoding and decoding algorithms.

- Preamble Structure

The proposed preamble structure in the time domain is the same as that in 802.16e as shown in Figure 1. There is one OFDM symbol in the preamble which is transmitted before other data symbols. And the proposed preamble structure in the frequence domain is shown in Figure 2, where there are two sequences in the preamble. The employed sequences for one cell-specific preamble are the two (different) composing sequences in one QACC of the constructed QCCC. All the QACCs of the constructed QCCC of length 128 are listed in Appendix A. We allocate the preamble binary sequence pair  and  of lengths 128 to 1024 subcarriers by the following assignment:

where  is the frequency-domain BPSK modulated signal at the th subcarrier for segment . For example, the Figure 3 shows the preamble in the frequency domain for segment 0.

- Construction of Quasi Complete Complementary Codes

Before we provide the construction of the QCCCs, we introduce some defitions and notations. Let  be a binary sequence of length , where . The aperiodic auto-correlation function  of  at displacement  is defined as

Figure 1: Preamble structure in the time domain.

Figure 2: Preamble structure in the frequency domain using QCCC.

Figure 3: Frequency domain allocation using QCCC for segment 0.

And the aperiodic cross-correlation function  of  and , where , at displacement  is defined as

We also denote the -tuple vectors by

and 	, which is the all-one vector. For vectors  and , let  where “” represents the product. Then the definitions for the QACC and QCCC are given below.

Definition 1  We call a pair of length- sequences  a quasi auto-complementary code of order 2 if

If the left equality holds, this set is in fact an auto-complementary code (ACC) [2] of order 2 or called a Golay pair [3].

Definition 2  The  sets of  length- pair , , …,  are called a quasi complete complementary code of order  if every pair is a QACC and every two distinct pairs satisfy

for any ;  .

If the left equality holds and every pair is an ACC, then the  sets constitute a complete complementary code (CCC) [2]. Hence QCCCs are modifications of CCCs with slight imperfections in the correlation properties. The construction of high-order QCCCs composed of QACCs of order 2 and length 128 is given below.

[QCCC Construction]  Let , be a vector of length , where the first  elements of  are the cyclic-shift ( positions to the right) of  and , and then for integer , let  , be a vector of length , where the first  elements are the cyclic-shift ( positions to the right) of  and the rest elements are identical to those of . If we denote

where  and  denotes binary addition, then   is a QACC of length . For all possible  and , we can obtain that there are  composing QACCs in the constructed QCCC as listed in Appendix A.

- Properties of QCCC

The maximum PAPRs of the preambles using QCCC and the 802.16e preambles are calculated and compared in Table 1. We can see that the PAPR values of QCCC-based preambles are similar to those of the 802.16e preambles. In Table 1, we also consider the average absolute values of the cross-correlations of QCCC and PN sequences used in 802.16e. Since the cross-correlations of the sequences in the same ACC of order 2 are summed as in Definition 2, the largest possible cross-correlation is twice the sequence length while that of PN sequences is equal to the sequence length. The ratio of the average absolute value of the cross-correlation to the largest possible value is used as the comparison criterion for the correlation property. It can be found that the ratio of PN sequences of length 284 in 802.16e is approximately the same as that of the QCCC of length 128. Thus when the correlation property is concerned, QCCC also performs as well as the PN sequences employed in 802.16e.

Table 1: Comparisons of PAPR and cross-correlation between QCCC-based and PN-based 802.16e preambles

| Preamble Type | QCCC-based | 802.16e PN-based |
| --- | --- | --- |
| PAPR (dB) | 4.41 | 4.35 |
| Length () | 128 | 284 |
| Number of sequences () | 2 | 1 |
| Average cross-correlation () | 8.01 | 8.99 |
|  | 0.031 | 0.032 |

- Simulation Results

We consider OFDM systems of 1024 subcarriers, 10MHz bandwidth, and 11.2MHz sampling frequency as specificed in [4]. The symbol duration is 91.43 s and the guard time is 11.43 s. The carrier frequency is 2.5 GHz, and the standard in [5] specifies a maximum oscillator error of 22 parts per million (ppm), which corresponds to the maximum possible frequency error of  subcarriers.

We consider the modified ITU Pedestrian B and Vehicular A channel models, which has the delay profile and the power profile given in Table 23 of [4] with each tap an independent, zero-mean complex Gaussian random variable.

For computer simulations, the joint frequency-offset estimation and cell search detection algorithm modified from [6] is employed. The frequency offset is set within  subcarriers. The order of the QCCCs of length 128 is 120, and we randomly choose 114 sets from those as cell-specific preamble sequences to be comparable with those in 802.16e. In Figures 4 to 6, we can see that the QCCC-based preamble performs as well as the PN-based preamble under diferrent channel models and different velocities.

Figure 4: Performance comparison between PN-based and QCCC-based preambles on modified ITU Pedestrian B channel with velocity 3km/hr.

Figure 5: Performance comparison between PN-based and QCCC-based preambles on modified ITU Vehicular A channel with velocity 30km/hr.

Figure 6: Performance comparison between PN-based and QCCC-based preambles on modified ITU Vehicular A channel with velocity 120km/hr.

- Conclusion

In this contribution, we propose a new preamble sequence based on QCCCs. The constructed quasi complete complementary code of length 128 has order 120. They possess the properties of low PAPRs and small cross-correlation values between every two composing QACCs. Simulation results show that the QCCC-based preambles perform as well as the original computer-searched PN-based preambles in 802.16e.

Since the PN sequences employed in 802.16e have no known structures, all the preamble sequences need to be stored for transmission and detection. Also during cell search, all the preamble sequences should be compared to determine the transmitted sequence. While the proposed QCCCs possess rich algebraic structures, thereby admitting low-complexity encoding and decoding algorithms, which may significantly accelerate the process of cell search and reduce the excessive amount of hardware memory in transceiver design.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Add the following into the TGm System Description Document]

11.x Downlink Preamble

11.x.1 Preamble Structure

11.x.2 Preamble Code Sequences

Code sequences of IEEE 802.16m preamble is based on Quasi Complete Complementary Codes, which is constructed by the following procedure.

The construction of high-order QCCCs composed of QACCs of order 2 and length  is given below.

[QCCC Construction]  For any integer , let , be a vector of length , where the first  elements of  are the cyclic-shift ( positions to the right) of  and , and then for integer , let  , be a vector of length , where the first  elements are the cyclic-shift ( positions to the right) of  and the rest elements are identical to those of . If we denote

where  and  denotes binary addition, then   is a QACC of length  and the combination of  forms a QCCC.

With the construced QCCC sequences, we allocate the preamble binary sequence pair  and  of length  to subcarriers by BPSK modulation.

-----------------------------------------------------------End of the Text---------------------------------------------------------

Appendix A – QCCC of length 128

Here is the list of the constructed QCCC of length 128. The vector  given in [QCCC Construction] can be viewed as a permutation of . Each pair {} is a QACC of order 2, and we have

Note that  are -tuple binary vectors with all-zero components except a single nonzero (one) component, i.e., their Hamming weights are one.

Table 2: List of 120 QACCs of the constructed QCCC of length 128

| Permutationof {1,2,3,4,5,6,7} | (p,q) | c | c' |
| --- | --- | --- | --- |
| 4,1,2,3,5,6,7 | 0,0 | v4v1+v1v2+v2v3+v3v5+v5v6+v6v7+v1+v3+v6+s4 | c+v4 |
| 4,1,2,3,5,6,7 | 0,1 | v4v1+v1v2+v2v3+v3v5+v5v6+v6v7+v2+v5+v6+s2 | c+v4 |
| 4,1,2,3,5,6,7 | 1,0 | v4v1+v1v2+v2v3+v3v5+v5v6+v6v7+v1+v2+v3+v5+s3 | c+v4 |
| 4,1,2,3,5,6,7 | 1,1 | v4v1+v1v2+v2v3+v3v5+v5v6+v6v7+s1 | c+v4 |
| 3,4,1,2,5,6,7 | 0,0 | v3v4+v4v1+v1v2+v2v5+v5v6+v6v7+v4+v2+v6+s4 | c+v3 |
| 3,4,1,2,5,6,7 | 0,1 | v3v4+v4v1+v1v2+v2v5+v5v6+v6v7+v1+v5+v6+s2 | c+v3 |
| 3,4,1,2,5,6,7 | 1,0 | v3v4+v4v1+v1v2+v2v5+v5v6+v6v7+v4+v1+v2+v5+s3 | c+v3 |
| 3,4,1,2,5,6,7 | 1,1 | v3v4+v4v1+v1v2+v2v5+v5v6+v6v7+s1 | c+v3 |
| 2,3,4,1,5,6,7 | 0,0 | v2v3+v3v4+v4v1+v1v5+v5v6+v6v7+v3+v1+v6+s4 | c+v2 |
| 2,3,4,1,5,6,7 | 0,1 | v2v3+v3v4+v4v1+v1v5+v5v6+v6v7+v4+v5+v6+s2 | c+v2 |
| 2,3,4,1,5,6,7 | 1,0 | v2v3+v3v4+v4v1+v1v5+v5v6+v6v7+v3+v4+v1+v5+s3 | c+v2 |
| 2,3,4,1,5,6,7 | 1,1 | v2v3+v3v4+v4v1+v1v5+v5v6+v6v7+s1 | c+v2 |
| 3,6,1,2,4,5,7 | 0,0 | v3v6+v6v1+v1v2+v2v4+v4v5+v5v7+v6+v2+v5+s4 | c+v3 |
| 3,6,1,2,4,5,7 | 0,1 | v3v6+v6v1+v1v2+v2v4+v4v5+v5v7+v1+v4+v5+s2 | c+v3 |
| 3,6,1,2,4,5,7 | 1,0 | v3v6+v6v1+v1v2+v2v4+v4v5+v5v7+v6+v1+v2+v4+s3 | c+v3 |
| 3,6,1,2,4,5,7 | 1,1 | v3v6+v6v1+v1v2+v2v4+v4v5+v5v7+s1 | c+v3 |
| 2,3,6,1,4,5,7 | 0,0 | v2v3+v3v6+v6v1+v1v4+v4v5+v5v7+v3+v1+v5+s4 | c+v2 |
| 2,3,6,1,4,5,7 | 0,1 | v2v3+v3v6+v6v1+v1v4+v4v5+v5v7+v6+v4+v5+s2 | c+v2 |
| 2,3,6,1,4,5,7 | 1,0 | v2v3+v3v6+v6v1+v1v4+v4v5+v5v7+v3+v6+v1+v4+s3 | c+v2 |
| 2,3,6,1,4,5,7 | 1,1 | v2v3+v3v6+v6v1+v1v4+v4v5+v5v7+s1 | c+v2 |
| 1,2,3,6,4,5,7 | 0,0 | v1v2+v2v3+v3v6+v6v4+v4v5+v5v7+v2+v6+v5+s4 | c+v1 |
| 1,2,3,6,4,5,7 | 0,1 | v1v2+v2v3+v3v6+v6v4+v4v5+v5v7+v3+v4+v5+s2 | c+v1 |
| 1,2,3,6,4,5,7 | 1,0 | v1v2+v2v3+v3v6+v6v4+v4v5+v5v7+v2+v3+v6+v4+s3 | c+v1 |
| 1,2,3,6,4,5,7 | 1,1 | v1v2+v2v3+v3v6+v6v4+v4v5+v5v7+s1 | c+v1 |
| 2,5,6,1,3,4,7 | 0,0 | v2v5+v5v6+v6v1+v1v3+v3v4+v4v7+v5+v1+v4+s4 | c+v2 |
| 2,5,6,1,3,4,7 | 0,1 | v2v5+v5v6+v6v1+v1v3+v3v4+v4v7+v6+v3+v4+s2 | c+v2 |
| 2,5,6,1,3,4,7 | 1,0 | v2v5+v5v6+v6v1+v1v3+v3v4+v4v7+v5+v6+v1+v3+s3 | c+v2 |
| 2,5,6,1,3,4,7 | 1,1 | v2v5+v5v6+v6v1+v1v3+v3v4+v4v7+s1 | c+v2 |
| 1,2,5,6,3,4,7 | 0,0 | v1v2+v2v5+v5v6+v6v3+v3v4+v4v7+v2+v6+v4+s4 | c+v1 |
| 1,2,5,6,3,4,7 | 0,1 | v1v2+v2v5+v5v6+v6v3+v3v4+v4v7+v5+v3+v4+s2 | c+v1 |
| 1,2,5,6,3,4,7 | 1,0 | v1v2+v2v5+v5v6+v6v3+v3v4+v4v7+v2+v5+v6+v3+s3 | c+v1 |
| 1,2,5,6,3,4,7 | 1,1 | v1v2+v2v5+v5v6+v6v3+v3v4+v4v7+s1 | c+v1 |
| 6,1,2,5,3,4,7 | 0,0 | v6v1+v1v2+v2v5+v5v3+v3v4+v4v7+v1+v5+v4+s4 | c+v6 |
| 6,1,2,5,3,4,7 | 0,1 | v6v1+v1v2+v2v5+v5v3+v3v4+v4v7+v2+v3+v4+s2 | c+v6 |
| 6,1,2,5,3,4,7 | 1,0 | v6v1+v1v2+v2v5+v5v3+v3v4+v4v7+v1+v2+v5+v3+s3 | c+v6 |
| 6,1,2,5,3,4,7 | 1,1 | v6v1+v1v2+v2v5+v5v3+v3v4+v4v7+s1 | c+v6 |
| 1,4,5,6,2,3,7 | 0,0 | v1v4+v4v5+v5v6+v6v2+v2v3+v3v7+v4+v6+v3+s4 | c+v1 |
| 1,4,5,6,2,3,7 | 0,1 | v1v4+v4v5+v5v6+v6v2+v2v3+v3v7+v5+v2+v3+s2 | c+v1 |
| 1,4,5,6,2,3,7 | 1,0 | v1v4+v4v5+v5v6+v6v2+v2v3+v3v7+v4+v5+v6+v2+s3 | c+v1 |
| 1,4,5,6,2,3,7 | 1,1 | v1v4+v4v5+v5v6+v6v2+v2v3+v3v7+s1 | c+v1 |
| 6,1,4,5,2,3,7 | 0,0 | v6v1+v1v4+v4v5+v5v2+v2v3+v3v7+v1+v5+v3+s4 | c+v6 |
| 6,1,4,5,2,3,7 | 0,1 | v6v1+v1v4+v4v5+v5v2+v2v3+v3v7+v4+v2+v3+s2 | c+v6 |
| 6,1,4,5,2,3,7 | 1,0 | v6v1+v1v4+v4v5+v5v2+v2v3+v3v7+v1+v4+v5+v2+s3 | c+v6 |
| 6,1,4,5,2,3,7 | 1,1 | v6v1+v1v4+v4v5+v5v2+v2v3+v3v7+s1 | c+v6 |
| 5,6,1,4,2,3,7 | 0,0 | v5v6+v6v1+v1v4+v4v2+v2v3+v3v7+v6+v4+v3+s4 | c+v5 |
| 5,6,1,4,2,3,7 | 0,1 | v5v6+v6v1+v1v4+v4v2+v2v3+v3v7+v1+v2+v3+s2 | c+v5 |
| 5,6,1,4,2,3,7 | 1,0 | v5v6+v6v1+v1v4+v4v2+v2v3+v3v7+v6+v1+v4+v2+s3 | c+v5 |
| 5,6,1,4,2,3,7 | 1,1 | v5v6+v6v1+v1v4+v4v2+v2v3+v3v7+s1 | c+v5 |
| 6,3,4,5,1,2,7 | 0,0 | v6v3+v3v4+v4v5+v5v1+v1v2+v2v7+v3+v5+v2+s4 | c+v6 |
| 6,3,4,5,1,2,7 | 0,1 | v6v3+v3v4+v4v5+v5v1+v1v2+v2v7+v4+v1+v2+s2 | c+v6 |
| 6,3,4,5,1,2,7 | 1,0 | v6v3+v3v4+v4v5+v5v1+v1v2+v2v7+v3+v4+v5+v1+s3 | c+v6 |
| 6,3,4,5,1,2,7 | 1,1 | v6v3+v3v4+v4v5+v5v1+v1v2+v2v7+s1 | c+v6 |
| 5,6,3,4,1,2,7 | 0,0 | v5v6+v6v3+v3v4+v4v1+v1v2+v2v7+v6+v4+v2+s4 | c+v5 |
| 5,6,3,4,1,2,7 | 0,1 | v5v6+v6v3+v3v4+v4v1+v1v2+v2v7+v3+v1+v2+s2 | c+v5 |
| 5,6,3,4,1,2,7 | 1,0 | v5v6+v6v3+v3v4+v4v1+v1v2+v2v7+v6+v3+v4+v1+s3 | c+v5 |
| 5,6,3,4,1,2,7 | 1,1 | v5v6+v6v3+v3v4+v4v1+v1v2+v2v7+s1 | c+v5 |
| 4,5,6,3,1,2,7 | 0,0 | v4v5+v5v6+v6v3+v3v1+v1v2+v2v7+v5+v3+v2+s4 | c+v4 |
| 4,5,6,3,1,2,7 | 0,1 | v4v5+v5v6+v6v3+v3v1+v1v2+v2v7+v6+v1+v2+s2 | c+v4 |
| 4,5,6,3,1,2,7 | 1,0 | v4v5+v5v6+v6v3+v3v1+v1v2+v2v7+v5+v6+v3+v1+s3 | c+v4 |
| 4,5,6,3,1,2,7 | 1,1 | v4v5+v5v6+v6v3+v3v1+v1v2+v2v7+s1 | c+v4 |
| 5,2,3,4,6,1,7 | 0,0 | v5v2+v2v3+v3v4+v4v6+v6v1+v1v7+v2+v4+v1+s4 | c+v5 |
| 5,2,3,4,6,1,7 | 0,1 | v5v2+v2v3+v3v4+v4v6+v6v1+v1v7+v3+v6+v1+s2 | c+v5 |
| 5,2,3,4,6,1,7 | 1,0 | v5v2+v2v3+v3v4+v4v6+v6v1+v1v7+v2+v3+v4+v6+s3 | c+v5 |
| 5,2,3,4,6,1,7 | 1,1 | v5v2+v2v3+v3v4+v4v6+v6v1+v1v7+s1 | c+v5 |
| 4,5,2,3,6,1,7 | 0,0 | v4v5+v5v2+v2v3+v3v6+v6v1+v1v7+v5+v3+v1+s4 | c+v4 |
| 4,5,2,3,6,1,7 | 0,1 | v4v5+v5v2+v2v3+v3v6+v6v1+v1v7+v2+v6+v1+s2 | c+v4 |
| 4,5,2,3,6,1,7 | 1,0 | v4v5+v5v2+v2v3+v3v6+v6v1+v1v7+v5+v2+v3+v6+s3 | c+v4 |
| 4,5,2,3,6,1,7 | 1,1 | v4v5+v5v2+v2v3+v3v6+v6v1+v1v7+s1 | c+v4 |
| 3,4,5,2,6,1,7 | 0,0 | v3v4+v4v5+v5v2+v2v6+v6v1+v1v7+v4+v2+v1+s4 | c+v3 |
| 3,4,5,2,6,1,7 | 0,1 | v3v4+v4v5+v5v2+v2v6+v6v1+v1v7+v5+v6+v1+s2 | c+v3 |
| 3,4,5,2,6,1,7 | 1,0 | v3v4+v4v5+v5v2+v2v6+v6v1+v1v7+v4+v5+v2+v6+s3 | c+v3 |
| 3,4,5,2,6,1,7 | 1,1 | v3v4+v4v5+v5v2+v2v6+v6v1+v1v7+s1 | c+v3 |
| 3,1,2,4,5,6,7 | 0,0 | v3v1+v1v2+v2v4+v4v5+v5v6+v6v7+v1+v4+v6+s4 | c+v3 |
| 3,1,2,4,5,6,7 | 0,1 | v3v1+v1v2+v2v4+v4v5+v5v6+v6v7+v2+v5+v6+s2 | c+v3 |
| 3,1,2,4,5,6,7 | 1,0 | v3v1+v1v2+v2v4+v4v5+v5v6+v6v7+v1+v2+v4+v5+s3 | c+v3 |
| 3,1,2,4,5,6,7 | 1,1 | v3v1+v1v2+v2v4+v4v5+v5v6+v6v7+s1 | c+v3 |
| 2,3,1,4,5,6,7 | 0,0 | v2v3+v3v1+v1v4+v4v5+v5v6+v6v7+v3+v4+v6+s4 | c+v2 |
| 2,3,1,4,5,6,7 | 0,1 | v2v3+v3v1+v1v4+v4v5+v5v6+v6v7+v1+v5+v6+s2 | c+v2 |
| 2,3,1,4,5,6,7 | 1,0 | v2v3+v3v1+v1v4+v4v5+v5v6+v6v7+v3+v1+v4+v5+s3 | c+v2 |
| 2,3,1,4,5,6,7 | 1,1 | v2v3+v3v1+v1v4+v4v5+v5v6+v6v7+s1 | c+v2 |
| 2,6,1,3,4,5,7 | 0,0 | v2v6+v6v1+v1v3+v3v4+v4v5+v5v7+v6+v3+v5+s4 | c+v2 |
| 2,6,1,3,4,5,7 | 0,1 | v2v6+v6v1+v1v3+v3v4+v4v5+v5v7+v1+v4+v5+s2 | c+v2 |
| 2,6,1,3,4,5,7 | 1,0 | v2v6+v6v1+v1v3+v3v4+v4v5+v5v7+v6+v1+v3+v4+s3 | c+v2 |
| 2,6,1,3,4,5,7 | 1,1 | v2v6+v6v1+v1v3+v3v4+v4v5+v5v7+s1 | c+v2 |
| 1,2,6,3,4,5,7 | 0,0 | v1v2+v2v6+v6v3+v3v4+v4v5+v5v7+v2+v3+v5+s4 | c+v1 |
| 1,2,6,3,4,5,7 | 0,1 | v1v2+v2v6+v6v3+v3v4+v4v5+v5v7+v6+v4+v5+s2 | c+v1 |
| 1,2,6,3,4,5,7 | 1,0 | v1v2+v2v6+v6v3+v3v4+v4v5+v5v7+v2+v6+v3+v4+s3 | c+v1 |
| 1,2,6,3,4,5,7 | 1,1 | v1v2+v2v6+v6v3+v3v4+v4v5+v5v7+s1 | c+v1 |
| 1,5,6,2,3,4,7 | 0,0 | v1v5+v5v6+v6v2+v2v3+v3v4+v4v7+v5+v2+v4+s4 | c+v1 |
| 1,5,6,2,3,4,7 | 0,1 | v1v5+v5v6+v6v2+v2v3+v3v4+v4v7+v6+v3+v4+s2 | c+v1 |
| 1,5,6,2,3,4,7 | 1,0 | v1v5+v5v6+v6v2+v2v3+v3v4+v4v7+v5+v6+v2+v3+s3 | c+v1 |
| 1,5,6,2,3,4,7 | 1,1 | v1v5+v5v6+v6v2+v2v3+v3v4+v4v7+s1 | c+v1 |
| 6,1,5,2,3,4,7 | 0,0 | v6v1+v1v5+v5v2+v2v3+v3v4+v4v7+v1+v2+v4+s4 | c+v6 |
| 6,1,5,2,3,4,7 | 0,1 | v6v1+v1v5+v5v2+v2v3+v3v4+v4v7+v5+v3+v4+s2 | c+v6 |
| 6,1,5,2,3,4,7 | 1,0 | v6v1+v1v5+v5v2+v2v3+v3v4+v4v7+v1+v5+v2+v3+s3 | c+v6 |
| 6,1,5,2,3,4,7 | 1,1 | v6v1+v1v5+v5v2+v2v3+v3v4+v4v7+s1 | c+v6 |
| 6,4,5,1,2,3,7 | 0,0 | v6v4+v4v5+v5v1+v1v2+v2v3+v3v7+v4+v1+v3+s4 | c+v6 |
| 6,4,5,1,2,3,7 | 0,1 | v6v4+v4v5+v5v1+v1v2+v2v3+v3v7+v5+v2+v3+s2 | c+v6 |
| 6,4,5,1,2,3,7 | 1,0 | v6v4+v4v5+v5v1+v1v2+v2v3+v3v7+v4+v5+v1+v2+s3 | c+v6 |
| 6,4,5,1,2,3,7 | 1,1 | v6v4+v4v5+v5v1+v1v2+v2v3+v3v7+s1 | c+v6 |
| 5,6,4,1,2,3,7 | 0,0 | v5v6+v6v4+v4v1+v1v2+v2v3+v3v7+v6+v1+v3+s4 | c+v5 |
| 5,6,4,1,2,3,7 | 0,1 | v5v6+v6v4+v4v1+v1v2+v2v3+v3v7+v4+v2+v3+s2 | c+v5 |
| 5,6,4,1,2,3,7 | 1,0 | v5v6+v6v4+v4v1+v1v2+v2v3+v3v7+v6+v4+v1+v2+s3 | c+v5 |
| 5,6,4,1,2,3,7 | 1,1 | v5v6+v6v4+v4v1+v1v2+v2v3+v3v7+s1 | c+v5 |
| 5,3,4,6,1,2,7 | 0,0 | v5v3+v3v4+v4v6+v6v1+v1v2+v2v7+v3+v6+v2+s4 | c+v5 |
| 5,3,4,6,1,2,7 | 0,1 | v5v3+v3v4+v4v6+v6v1+v1v2+v2v7+v4+v1+v2+s2 | c+v5 |
| 5,3,4,6,1,2,7 | 1,0 | v5v3+v3v4+v4v6+v6v1+v1v2+v2v7+v3+v4+v6+v1+s3 | c+v5 |
| 5,3,4,6,1,2,7 | 1,1 | v5v3+v3v4+v4v6+v6v1+v1v2+v2v7+s1 | c+v5 |
| 4,5,3,6,1,2,7 | 0,0 | v4v5+v5v3+v3v6+v6v1+v1v2+v2v7+v5+v6+v2+s4 | c+v4 |
| 4,5,3,6,1,2,7 | 0,1 | v4v5+v5v3+v3v6+v6v1+v1v2+v2v7+v3+v1+v2+s2 | c+v4 |
| 4,5,3,6,1,2,7 | 1,0 | v4v5+v5v3+v3v6+v6v1+v1v2+v2v7+v5+v3+v6+v1+s3 | c+v4 |
| 4,5,3,6,1,2,7 | 1,1 | v4v5+v5v3+v3v6+v6v1+v1v2+v2v7+s1 | c+v4 |
| 4,2,3,5,6,1,7 | 0,0 | v4v2+v2v3+v3v5+v5v6+v6v1+v1v7+v2+v5+v1+s4 | c+v4 |
| 4,2,3,5,6,1,7 | 0,1 | v4v2+v2v3+v3v5+v5v6+v6v1+v1v7+v3+v6+v1+s2 | c+v4 |
| 4,2,3,5,6,1,7 | 1,0 | v4v2+v2v3+v3v5+v5v6+v6v1+v1v7+v2+v3+v5+v6+s3 | c+v4 |
| 4,2,3,5,6,1,7 | 1,1 | v4v2+v2v3+v3v5+v5v6+v6v1+v1v7+s1 | c+v4 |
| 3,4,2,5,6,1,7 | 0,0 | v3v4+v4v2+v2v5+v5v6+v6v1+v1v7+v4+v5+v1+s4 | c+v3 |
| 3,4,2,5,6,1,7 | 0,1 | v3v4+v4v2+v2v5+v5v6+v6v1+v1v7+v2+v6+v1+s2 | c+v3 |
| 3,4,2,5,6,1,7 | 1,0 | v3v4+v4v2+v2v5+v5v6+v6v1+v1v7+v4+v2+v5+v6+s3 | c+v3 |
| 3,4,2,5,6,1,7 | 1,1 | v3v4+v4v2+v2v5+v5v6+v6v1+v1v7+s1 | c+v3 |

References

- Y. Segal, I. Kitroser, Y. Leiba, E. Shasha, and Z. Hadad, “Preambles design for OFDMA PHY layer, FFT sizes of 1024, 512, and 128,” IEEE C802.16e-04/125r1, Aug. 2004.

- N. Suehiro and M. Hatori, “Even-shift orthogonal sequences,” IEEE Trans. Inform. Theory, vol. 34, pp. 143--146, Jan. 1988.

- J. A. Davis and J. Jedwab, “Peak-to-mean power control in OFDM, Golay complementary sequences, and Reed-Muller codes,” IEEE Trans. Inform. Theory, vol. 45, pp. 2397--2417, Nov. 1999.

- R. Srinivasan et al., “Draft IEEE 802.16m Evaluation Methodology,” IEEE 802.16m-07/037r2, Dec. 2007.

- IEEE Standard for Local and Metropolitan Area Networks -- Part 16: Air Interface for Fixed Broadband Wireless Access Systems, IEEE Std. 802.16, 2004.

- T. Cui and C. Tellambura, “Joint data detection and channel estimation for OFDM systems,” IEEE Trans. Commun., vol. 54, pp. 670--679, Apr. 2006.