| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A New Preamble Structure for Robust Timing/Frequency Synchronization in IEEE 802.16m System |  |
| Date Submitted | 2008-05-05 |  |
| Source(s) | Pei-Kai Liao, Chih-Yuan Lin, Ciou-Ping Wu, and Paul Cheng MediaTek Inc. No.1, Dusing Road 1, Science-Based Industrial Park, Hsinchu, Taiwan 300, R.O.C. | pk.liao@mediatek.com paul.cheng@mediatek.com |
| Re: | IEEE 802.16m-08/016, “Call for Contributions on Project 802.16m System Description Document (SDD)” – Preambles |  |
| Abstract | This contribution proposes a new preamble structure for P802.16m SDD to improve timing/frequency synchronization performance with low overhead. |  |
| Purpose | Propose to be discussed and adopted by TGm for the use in Project 802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

A New Preamble Structure for Robust Timing/Frequency Synchronization IEEE 802.16m System

Pei-Kai Liao, Chih-Yuan Lin, Ciou-Ping Wu, and Paul Cheng

MediaTek Inc.

- Introduction

This contribution is to propose a new preamble structure to provide robust timing/frequency synchronization for P802.16m system. In current IEEE 802.16e system, PRBS is used as preamble code sequence and it is interleaved by 3 over sub-carriers to separate the code sequences by FDM for each segment to avoid preamble collision. However, there are some drawbacks for this preamble design. First, non-structural code sequences increase the hardware complexity of transceiver design due to additional memory for the storage of all code sequences and computational burden of code sequence searching. Second, because each segment uses a different code sequence for its preamble, it not only lacks of additional multi-cell diversity gain but may suffer from interferences from other cells at the cell edge. Thus, it might result in poor synchronization performance for cell-edge users. Finally, small sub-carrier spacing may cause synchronization difficulties in high mobility environment due to large Doppler spread. To resolve these drawbacks, one may like to design a better preamble structure together with a structural code sequences to enhance the synchronization performance.

In this contribution, we focus on the design of preamble structure instead of code sequences. The proposed preamble structure occupies the time length of one regular OFDM symbol only. No additional overhead is required so as to maximize the spectral efficiency. Robustness to Doppler spread due to high mobility, compared to IEEE 802.16e system, is another advantage it holds. This also helps IEEE 802.16m system to meet strict performance requirement in high mobility environment (up to 350 km/hr). Furthermore, any kind of code sequences, including both structural and non-structural code sequences, can be applied to enjoy the good properties of the proposed preamble structure.

- Considerations of New Preamble Design

- Low overhead:

In order to keep high spectral efficiency of 802.16m system, at most one OFDM symbol for preamble is desired, especially for the case where legacy support is enabled. More than one OFDM symbol overhead may waste precious spectral resources.

- Improved synchronization performance for cell-edge users:

For cell-edge users, it is desired to have similar synchronization performance as those at the location close to base station. Good preamble structure can enhance the synchronization performance of cell-edge users without degrading others.

- Robustness to Doppler spread induced by high mobility:

According to SRD, it is required to maintain the network connection under the speed up to 350 km/hr. Preamble structure which is robust to large Doppler spread is desired so as to make MS remain synchronized with BS to keep network connection.

- Avoidance of preamble collision between segments:

Preamble collision degrades the detection rate of correct preamble sequence and thus results in misjudgment of segment identification. This may also delay the network entry or fail MS’s connection with BS. To avoid performance degradation, a scheme for avoidance of preamble collision between segments is desirable.

- Fast temporal domain preamble detection:

Fast temporal domain preamble detection without decoding preamble code sequence can largely decrease the number of failure trials and thus speed up the synchronization process. Special characteristics of time-domain preamble sequence may be helpful for this process.

- Proposed Preamble Structure

Figure 1 shows the time domain structure of the proposed preamble. In order to minimize the overhead, the proposed preamble is deigned to occupy one OFDM symbol length only. Therefore, the total occupied length in time domain is (N+CP) samples. Inside the preamble, it is partitioned into two parts – first preamble and second preamble. To meet the constraint for the length of the preamble (N+CP), CP is applied to second preamble only so the time length of first preamble is N/2 samples while the time length of second preamble is (N/2+CP).

Figure 1–Temporal domain structure of the proposed preamble

In the frequency domain, N/2 sub-carriers are deployed over the whole bandwidth for both first and second preamble. In other words, they are two separate short OFDM symbols in time domain. This arrangement makes sub-carrier spacing twice larger than that of a regular OFDM symbol. Figure 2 illustrates an example of frequency domain structure of the proposed preamble. The same as 802.16e system, DC tones are nulled and guard tones are applied to meet the spectral mask requirement in both first and second preamble. Since there is no CP applied to first preamble, code sequence is interleaved by K1 over sub-carriers so that first preamble has K1-period structure in time domain and the first period can act as a CP to resolve the decoding problem due to the lack of CP. Thus, the period length has to be larger than the CP length of a regular OFDM symbol. The number of K1 is an integer, which is limited by the following equation.

,

where  is the number of sub-carriers in a regular OFDM symbol and  is the length of CP. For second preamble, code sequence can be non-interleaved or interleaved by K2 over sub-carriers. The number of K2 is an integer as well and depends on K1 and the number of neighboring segments. Normally, it is larger than the number of neighboring segments so as to avoid preamble collision by FDM.

Figure 2–Frequency domain structure of the proposed preamble (Example)

Any kind of code sequences can be applied to the proposed preamble structure. Depending on the arrangement of code sequences, the proposed preamble structure can be categorized into two types – separate structure and joint structure.

- For separate structure, code sequences applied to first and second preamble are two independent codewords. The codeword applied to first preamble is identical over all cells for amplifier gain training, coarse synchronization and synchronization parameters estimation while the codeword applied to second preamble is different among segments for fine synchronization, channel estimation and segment identification.

- For joint structure, code sequences applied to first and second preamble are two partitions of one codeword and interleaved by the same interleaving factor (K1=K2). The combined codeword from two partitions is used for amplifier gain training, synchronization, channel estimation and segment identification.

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Editor’s Notes: add the following into the TGm System Description Document]

11.x Downlink Preamble

11.x.1 Preamble Structure

IEEE 802.16m downlink preamble is partitioned into two parts – first preamble and second preamble. In this design, CP is applied to second preamble only so that the time length of first preamble is N/2 samples and the time length of second preamble is (N/2+CP) samples. Its time domain structure is illustrated in Figure 11.x.1.

Figure 11.x.1–Temporal domain structure of the proposed preamble

In the frequency domain, N/2 sub-carriers are deployed over the whole bandwidth in both first and second preamble. In other words, they are two separate short OFDM symbols in time domain. This arrangement makes sub-carrier spacing twice larger than that of a regular OFDM symbol. Figure 11.x.2 illustrates an example of frequency domain structure of the proposed preamble. DC tones are zeroed and X [the number of guard tones is FFS] guard band sub-carriers are applied to meet the spectral mask requirement in both first and second preamble. Code sequence in first preamble is interleaved by K1 over sub-carriers so that first preamble has K1-period structure in time domain and the first period functions as a CP. The period length shall be larger than the CP length of a regular OFDM symbol. The number of K1 is an integer, which is limited by the following inequality.

,

where  is the number of sub-carriers in a regular OFDM symbol and  is the length of CP. For second preamble, code sequence is interleaved by K2 over sub-carriers. The number of K2 is an integer and depends on K1 and the number of neighboring segments. K2=1 means non-interleaving.

Figure 11.x.2–Frequency domain structure of the proposed preamble (Example)

11.x.2 Preamble Code Sequences

-----------------------------------------------------------End of the Text---------------------------------------------------------