| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Support for Location Based Services |  |
| Date Submitted | 2008-05-05 |  |
| Source(s) | Mei-Dai Chen, I-Kang Fu, Yih-Shen Chen, Kelvin Chou, and Paul Cheng MediaTek Inc. No.1, Dusing Rd. 1, Hsinchu Science-Based Industrial Park, Hsinchu, Taiwan 300, R.O.C. Chung-Hsien Hsu, and Kai-Ten Feng NCTU/MediaTek Inc. 1001 Ta Hsueh Road, Hsinchu, Taiwan 300, R.O.C | dora.chen@mediatek.com IK.Fu@mediatek.com stanley.hsu@mediatek.com |
| Re: | IEEE 802.16m-08/016, “Call for Contributions on Project 802.16m System Description Document (SDD)”. – Comments on P802.16m SDD |  |
| Abstract | This contribution discusses the requirement of the parameters to describe and transmit the MS’s location. |  |
| Purpose | To be discussed and adopted by TGm into the P802.16m SDD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Support for Location Based Services

Mei-Dai Chen, I-Kang Fu, Yih-Shen Chen, Kelvin Chou, and Paul Cheng

MediaTek Inc.

Chung-Hsien Hsu and Kai-Ten Feng

NCTU/MediaTek Inc.

# 1. Introduction

With the increasing population of the mobile devices, people require not only the basic high quality voice services, but also other advanced services, such as location based services (LBSs). In addition to assist MS to acquire its GPS location [1], the BS may also need to know MS’s location so as to perform the LBSs like emergency services, information services, navigation services, location based billing services, tracking and management services and etc. In the contribution, we discuss the applications of LBS and the location tracking methods.

- Lists of LBS applications

Here we briefly describe some popular LBS applications.

- Emergency 911 safety services

One of the most important applications of LBSs is emergency safety service which geometrically locates a person who is unaware of his/her location or is not able to reveal it due to the emergency situation such as injury, criminal attack, and so forth. With the exact location automatically transferred to the emergency services, the assistance can be provided quickly and efficiently.

- Information services

Location-based information services provide information that is related to MS’ location. For example, services such as guided tours, notification about nearby places of interest (restaurants, theaters, etc.), transportation services, and other services that can be provided to tourists moving around in the unfamiliar environment or city.

- Navigation services

Navigation services are applicable for mobile users. MS report their location to base station (BS), and then serving operator can provide geometrical information in a map and route to the destination.

- Location based billing services

Location-based billing services enable a mobile location service provider to dynamically charge service fee depending on MS’s location.

- Tracking and management services

Tracking and management services are applicable both corporate markets and consumers. For example, by tracking postal packages, a company can know easily where their goods are at any time. Vehicle tracking can also be applied to locate and dispatch an ambulance that is nearest to a given call. Another similar application allows companies to locate their field person (salespeople, or repair engineers) so that they are able to dispatch the nearest engineer and provide their customers with accurate personal arrival time.

# Estimation of MS location

To enable the above-mentioned LBS applications, some location-related parameters should interact between MS and BS. BS then can estimate MS location with those parameters. Therefore, three main issues should be considered:

- Location parameters to be updated

- Parameter updating method

- Location estimation method

For example, periodical reporting of GPS location is the simplest solution, but the GPS location may not always be available and the reporting requires huge MAC overhead. In the following, we discuss how BS estimates MS location with/without MS’s assistance.

3.1. Discussion on Estimation Methods

The wireless location technologies can be classified into network-based and satellite-based location schemes. The typical algorithms for network-based location techniques are the angle of arrival (AOA) and the time difference of arrival (TDOA). For the satellite-based location method, the global positioning system (GPS) is the well-adapted technique. It measures the time of arrival (TOA) of signals coming from different satellites. However, several researches have been studied [3] [4] that the performance of the location techniques listed above varies under different environments. Due to weak incoming signals or shortage of signal sources (e.g. at rural area), the network-based (i.e. TDOA, AOA) methods result in degraded performance for the location determination of mobile devices [5] [6]. On the other hand, the major problem for the satellite-based (i.e. GPS) method is that the performance considerably degrades while the satellite signals are severely blocked (e.g. at urban valley area)

It can be considered, for example, the hybrid location estimation and tracking system which combine both network-based and satellite-based location schemes. Here, it is assumed that each MS is equipped with a GPS receiver in order to obtain the TOA measurements from the satellites. As shown in Fig. 1, the TDOA measurements are conducted at the MS by obtaining the signals via the downlink from the BSs. The AOA signals are transmitted from the MS to the BSs using the uplink. The AOA measurements are performed at the BS using its antenna arrays for two-dimensional adaptive beam steering. In order to avoid signal degradation due to the near-far effect, it is assumed that only the home BS provides the capability of the AOA measurements.

Fig. 1 The schematic diagram of the hybrid mobile location scheme

This hybrid location estimation scheme can be applied to either MS-based or MS-assisted system. The choice between these two types of systems depends on the requirement of the communication bandwidth and the computation power of the MS. With this example, the MS’s location can be determined more accurately. Not only the longitude (x) and the latitude (y) of the MS can be obtained, but also the altitude (z) of the MS can be estimated.

From the above, it is apparent to know that obtaining the location of the MS is easy. In order to make LBSs more efficient, the ability of computing and transmitting the MS’s location is needed. Therefore, it is suggested that there should be some parameters, such as the longitude (x), the latitude (y), and the altitude (z), to describe the MS’s location.

    - Simulation Results and Comparison

Here are some simulations performed in Fig. 2 to discuss the performances of MS location estimation techniques based on MS. The MS is assumed to travel at a constant speeds of (3, 4, 0) m/s along the x and y directions, i.e. (x, y, z) = (27 + 3t, 36 + 4t, 300) in meters. Fig. 2 shows the performance of this hybrid location system (measured in RMS error) comparing with the GPS and the cellular system under urban, suburban, and rural environments. It can be seen that the GPS system has the worst performance comparing with the other two systems in the urban area; while the cellular system causes degraded results in the rural area. The hybrid system is capable of adjust itself to accommodate different situations, which provides consistent performance comparing with the GPS and the cellular systems.

Fig. 2 Performance comparisons between the GPS, the cellular, and the hybrid system in urban (top), suburban (middle), and rural (bottom) areas.

# 4. Conclusion

The applications of LBSs are widely applied in our daily lives. With quickly obtaining the MS’s location, the LBSs can be provided to the mobile users efficiently. Therefore, since it can be easy to estimate the MS’s location by some algorithms like the above mentioned example, it is suggested that there should be some parameters to describe the MS’s location. Directly transmitting the MS’s location can bring more efficiency to the applications of LBSs.

5. Proposed Text

----------------------------------------------------Start of the Text----------------------------------------------------------------

[Insert the following text after line#3, page#23 in 802.16m-08/003r1 (P802.16m SDD)]

14 Support for Location Based Services

14.1 Mobile Station Location Estimation and Tracking

14.1.1 Terminal based MS Location Estimation and Tracking

14.1.2 Network based MS Location Estimation and Tracking

14.1.3 Hybrid MS Location Estimation and Tracking

----------------------------------------------------End of the Text---------------------------------------------------------------

References

[1] P802.16Rev2/D4.

[2] Chung-Hsien Hsu et al, “Location Estimation and Tracking System for Mobile Devices in Project 802.16m SDD,” IEEE C802.16m-07/202r1, November 2007.

[3] Y. T. Chan, C. H. Yau, and P. C. Yau, “Linear and Approximate Maximum Likelihood Localization from TOA Measurements,” IEEE Signal Processing and Its Applications, vol. 2, pp. 295-298, July 2003.

[4] X. Wang, Z. Wang, and B. O’Dea, “A TOA-Based Location Algorithm Reducing the Errors due to Non-Line-of-Sight (NLOS) Propagation,” IEEE Trans, vol. 52, Jan. 2003.

[5] D. Porcino, “Performance of a OTDOA-IPDL Positioning Receiver for 3GPP-FDD Mode,” IEE 3G Mobile Communication Technologies, pp. 221-225. Mar. 2001.

[6] SnapTrack, “Location Technologies for GSM, GPRS, and UMTS Network,” a QUALCOMM Company white paper, Jan. 2003.