| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | System State Diagram for IEEE 802.16m system |  |
| Date Submitted | May 5, 2008 |  |
| Source(s) | Changhoi Koo, Baowei Ji Sungjin Lee, Hyunjeong Kang, Jaehyuk Jang and Jungje Son | chkoo@samsung.com Samsung Telecommunication America Samsung electronics |
| Re: | Call for Contributions on Project 802.16m System Description Document (SDD) Topic: The entire content of IEEE 802.16m-08/003r1 |  |
| Abstract | System State Transition Diagram IEEE 802.16m Air-Interface Protocol |  |
| Purpose | For discussion and approval by IEEE 802.16 Working Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

System State Diagram for IEEE 802.16m system

Changhoi Koo, Baowei Ji

Samsung Telecommunicatoin America

Sungjin Lee, Hyunjeong Kang, Jaehyuk Jang, and Jungje Son

Samsung Electronics

- Introduction

This contribution proposes the System State Transition Diagram for section 6 of SDD documents (IEEE802.16m-08/003r1) for IEEE802.16m system.

- Proposed Text

--------------------------------- Begin of Text --------------------------------------------------------

6. IEEE802.16m Top Level System State Diagram

The following figure AAA illustrates the system state transition diagram for IEEE802.16m BS and MS. System state diagram for IEEE802.16m systems consists of 4 states and 3 modes. The states are Initialization state, Access state, Connected state and Idle state. Connected state consists of 3 separated modes which are sleep mode, active mode and scanning mode based on its operation and MAC message generation. And also, Idle state consists of 2 separated modes which are awake mode and idle mode based on its operation and MAC message generation.

Figure AAA. System State Transition Diagram of IEEE802.16m

6.1 Initialization State

[Contents is TBD]

Figure BBB. Initialization State Transition Diagram

6.2 Access State

[Contents is TBD)

Figure CCC. Access State Transition Diagram

6.3 Connected State

[Contents is TBD]

Figure DDD. Connected State Transition Diagram

6.3.1 Sleep mode

[Contents is TBD]

6.3.2 Active mode

[Contents is TBD]

6.3.3 Scanning mode

[Contents is TBD]

6.4 Idle State

[Contents is TBD]

Figure EEE. Idle State Transition Diagram

6.4.1 Awake mode

[Contents is TBD]

6.4.2 Idle mode

[Contents is TBD]