| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | System State Diagram for IEEE 802.16m system |  |
| Date Submitted | May 5, 2008 |  |
| Source(s) | Changhoi Koo, Baowei Ji Sungjin Lee, Hyunjeong Kang, Jaehyuk Jang and Jungje Son | chkoo@samsung.com. Baowei.ji@samsung.com Samsung Telecommunication America Samsung electronics |
| Re: | Call for Contributions on Project 802.16m System Description Document (SDD) Topic: The entire content of IEEE 802.16m-08/003r1 |  |
| Abstract | System State Transition Diagram IEEE 802.16m Air-Interface Protocol |  |
| Purpose | For discussion and approval by IEEE 802.16 Working Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

System State Diagram for IEEE 802.16m system

Changhoi Koo, Baowei Ji

Samsung Telecommunicatoins America

Sungjin Lee, Hyunjeong Kang, Jaehyuk Jang, and Jungje Son

Samsung Electronics

- Introduction

This contribution proposes the System State Transition Diagram for section 6 of SDD documents (IEEE802.16m-08/003r1) for IEEE802.16m system.

- Proposed Text

--------------------------------- Begin of Text --------------------------------------------------------

6. IEEE802.16m Top Level System State Diagram

The following figure AAA illustrates the system state transition diagram for IEEE802.16m BS and MS. System state diagram for IEEE802.16m systems consists of 4 states,  Initialization state, Access state, Connected state and Idle state. The Connected state consists of 3 separated modes which are sleep mode, active mode and scanning mode based on its operation and MAC message generation. And Idle state consists of 2 separated modes, idle mode and awake mode based on its operation and MAC message generation.

Figure AAA. System State Transition Diagram of IEEE802.16m

6.1 Initialization State

Initial State is where an MS performs cell selection by scanning and synchronizing to a BS preamble, and acquiring the system configuration information through BCH before it is ready to perform a ranging process to enter Access State. The MS can return back to scanning step in case that it fails to perform action required to each step.

Figure BBB. Initialization State Transition Diagram

During this state, if the MS could not properly perform the BCH information decoding and cell selection, it should be back to perform the scanning and DL synchronization. The red dashed line stands the abnormal case. If the MS successfully decodes BCH information, it goes to Access state. If the abnormal case occurs in Access state, the stats transition should be achieved from the Initialization state again.

6.2 Access State

Access State is where the MS performs network entry to the selected BS by going through several processes. A MS performs the ranging process using RNG-REQ/RSP MAC message in order to get UL synchronization. A MS performs the basic capability negotiation with BS using SBC-REQ/RSP MAC message. A MS then performs the authentication and authorization process through key exchanging. And finally a MS performs the registration process using REG-RES/RSP MAC message followed by a MS gets the MAC CID and IP address.

Figure CCC. Access State Transition Diagram

Upon successfully performing the access state operation, a MS goes to connected state in order to exchange the user data between BS and MS. Otherwise a MS goes back to Initialization state in case of abnormal operation.

6.3 Connected State

The state consists of 3 modes; sleep mode, active mode and scanning mode. During Connected State, MS maintains at least one connection as established during Access State, while MS and BS may establish additional transport connections. In addition, to save power consumption of the MS during exchanging the user date, a MS or BS can request a transition to sleep mode. And also, MS can scan neighbor cell’s signal to reselect a cell which provides robust and reliable services.

Figure DDD. Connected State Transition Diagram

6.3.1 Active mode

During Active Mode, the MS and the BS perform normal operations to exchange the DL/UL traffic transaction between MS and BS. And MS can perform the Fast re-entry procedures after handover: while in handover, MS CID and IP address are remained. Without going through access state, MS can keep in connected state with target BS.

6.3.2 Sleep mode

During Sleep mode, MS can do power saving during traffic interval. MS in Active mode transits to sleep mode through sleep mode MAC management messages such as MOB_SLP-REQ/RSP. MS does not transmit and receive any traffic to/from its BS in sleep interval. A MS can receive a MOB_TRF-IND message during listening interval and then whether a MS can transit to active mode or be stayed in sleep interval according to a indication bit in MOB_TRF-IND message.

6.3.3 Scanning mode

During scanning mode, the MS may be temporarily unavailable to the BS, and performs scanning operation. While in active mode, MS transits to scanning mode through explicit scanning transaction through MOB_SCN-REQ/RSP. In this mode, MS is unavailable to BS. In addition, a MS can perform the implicit scanning procedures that MS performs a scanning other BSs without scanning management messages generation.

6.4 Idle State

During Idle state, the MS performs power saving by switching between Paging listening interval and Paging Unavailable interval in Idle mode, and the MS performs the location update in Awake mode.

Figure EEE. Idle State Transition Diagram

6.4.1 Awake mode

During this mode, location update procedures are achieved. While in awake mode, MS can transit to access state if required.

6.4.2 Idle mode

During the paging listening interval, MOB_PAG-ADV is received. If a MS is paged, MS transits to access state for its network re-entry. Otherwise, in paging unavailable interval, MS does not need to monitor down link channel in order to save its power consumptions. While in this mode, MS can also transit to access state if required.