| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Downlink distributed resource mapping and hopping for 802.16m |  |
| Date Submitted | 2008-05-05 |  |
| Source(s) | Jia Lin, Juejun Liu, Jianmin Lu Huawei Technologies CO., LTD. | Voice: +86-755-28978859 E-mail: linjia@huawei.com |
| Re: | Call for Comments on IEEE C802.16mDL_PHY-08/046. |  |
| Abstract | Proposal for downlink distributed resource mapping and hopping in 802.16m |  |
| Purpose | For discussion and approval by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Downlink distributed resource mapping and hopping for 802.16m

Jia Lin, Juejun Liu, Jianmin Lu

Huawei

# 1 Introduction

802.16e implements TDM as channelization mode to configure distributed resource (PUSC/FUSC) & localized resource (AMC). It uses different sub-zones to configure using different part of the bandwidth.

For requirement of 802.16m, Localized resource block (LRB) can be used for frequency-selective scheduling.  Localized RB is formed from contiguous physical tiles. Distributed Resource block (RB) can be used for frequency diversity scheduling. Distributed resource block consists of multiple distributed physical tiles that are scattered across the entire bandwidth or multiple reserved frequency bands. 802.16m should support multiplexing of Localized resource and Distributed resource in each sub-frame.

In order to make distributed resource achieve better frequency diversity and interference average between different cells, downlink distributed resource mapping and hopping is introduced in this contribution.

# 2 Design Consideration and Feature

In order to achieve better frequency diversity and interference average, Distributed resource mapping in sub-frame should satisfy randomization, and Distributed resource should hop based on per sub-frame, frame or super frame.

Following factors should be considered:

1) Bandwidth of sub-frame

2) Partition of DRBs and LRBs per sub-frame

3) Mapping Offset sequence for resource mapping

4) Hopping Offset sequence for resource hopping

4) Rule of resource mapping in sub-frame/frame/super-frame

5) Rule of resource hopping in different sub-frame/frame/super-frame

6) FFR grouping

# 3 Proposal

Bandwidth of sub-frame is divided into several clusters which contains of distributed resource and localized resource. Further, within a cluster, bandwidth is divided into multiple frequency bands, each of which is called as sub-zone. When multiplexing of Localized resource and distributed resource, Localized resource is located in the LRB sub-zones; Distributed resource is located in the DRB sub-zones. Localized resource and Distributed resource occupy different frequency bands (sub-zones) within a sub-frame. Tiles of distributed RB are located in DRB sub-zones of different clusters in sub-frame. Localized RB is formed from contiguous physical tiles in LRB sub-zone.

Each cluster is composed of multiple tiles which contains of multiple tones. A tone is one subcarrier by one symbol. Distributed resource block consists of multiple distributed physical tiles that are scattered across the entire bandwidth or multiple reserved frequency bands (sub-zones). A cluster corresponds to 788 kHz which can provide adequate frequency selective scheduling performance

## 3.1 Downlink distributed resource mapping

In order to achieve better frequency diversity and interference average, Mapping offset sequence is used for downlink distributed resource mapping, which can be generated by PN sequence. Distributed resource in each cluster is composed of multiple tiles which belong to different DRBs.

- The Mapping offset sequence values are corresponding to tile’s location of DRB in different DRB sub-zones of sub-frame.

- Mapping offset sequence Mn = {O0, O1, O2 … Oi}, value of Oi indicates tile’s location of corresponding DRB in DRB sub-zone i. Mapping offset sequence Mn indicates resource mapping of corresponding DRB in sub-frame.

- Range of value in Mapping offset sequence can vary in terms of different bandwidth and partition of DRB&LRB.

- Mapping offset sequences are orthogonal each other.

- Distributed resource mapping can be corresponding to different Mapping offset sequence in different sub-frame/frame/super-frame.

- Distributed resource mapping is decided by combination of Mapping offset sequences for each sub-frame/frame/super-frame.

For example:

Mapping offset sequence M0 is corresponding to resource mapping of DRB0 in sub-frame 0.

Mapping offset sequence M1 is corresponding to resource mapping of DRB1 in sub-frame 0.

Tile’s location of DRB 0 in DRB sub-zone 0 of sub-frame 0 is corresponding to first value O0 of Mapping offset sequence M0.

Tile’s location of DRB 0 in DRB sub-zone 1 of sub-frame 0 is corresponding to second value O1 of Mapping offset sequence M0.

E.g. For 5MHz bandwidth, as DRB:LRB=1:3, RB size is 18 subcarriers by 6 symbols, which means number of DRB is 6 in sub-frame, and number of LRB is 18. Each cluster has 24 tiles, whose 6 tiles are for 6 different DRBs respectively. A Sub-frame is divided into 6 clusters, each cluster has one PRB for distributed resource. In this case, rang of value in Mapping offset sequence is from 0 to 5.

It is assumed that Mapping offset sequence 0 is M0 = {0, 1, 5, 4, 3, 2}, which is for mapping of DRB0. Mapping offset sequence 1~5 should be generated for keeping them orthogonal each other, Mapping offset sequence 1~5 is for mapping of DRB1~5. Such as:

M1 = {1, 2, 0, 5, 4, 3}

M2 = {2, 3, 1, 0, 5, 4}

M3 = {3, 4, 2, 1, 0, 5}

M4 = {4, 5, 3, 2, 1, 0}

M5 = {5, 0, 4, 3, 2, 1}

Distributed resource mapping in sub-frame is decided by combination of Mapping offset sequences. Distributed resource mapping to sub-frame is show as Figure 1.

Figure 1: Example of downlink resource mapping

Different combination of Mapping offset sequences is used for distributed resource mapping in different sub-frame/frame/super-frame.

- Combination of Mapping offset sequences is Cn = {MDRB0, MDRB1 … MDRBi}, MDRBi is Mapping offset sequence for logical DRBi. Cn indicates distributed resource mapping for each sub-frame/frame/super-frame.

- Combinations of Mapping offset sequences for different sub-frame/frame/super-frame in the same sector are orthogonal each other.

- Combinations of Mapping offset sequences for different sectors in the same sub-frame/frame/super-frame are orthogonal each other.

For example:

Combination of Mapping offset sequences in sub-frame 0 is C0= {M0, M1, M2, M3, M4, M5}. M0 is for DRB0 (MDRB0= M0), M1 is for DRB1 (MDRB1= M1), and so on.

Combination of Mapping offset sequences in sub-frame 1 is changed, which is C1= {M6, M7, M8, M9, M10, M11}.

M0~ M5 are Mapping offset sequence 0~5. M6 is for DRB0 (MDRB0= M6), M7 is for DRB1 (MDRB1= M7), and so on.

Figure 2 shows resource mapping in different sub-frames which are corresponding different combinations of Mapping offset sequences.

Figure 2: Example of resource mapping for different sub-frames in the same sector

Figure 3 shows resource mapping for different sectors which are corresponding different combinations of Mapping offset sequences in the same sub-frame.

Figure 3: Example of resource mapping for different sectors in the same sub-frame number

## 3.2 Downlink distributed resource hopping

Hopping offset sequence is used for distributed resource hopping in this contribution. Hopping offset sequence can be generated by PN sequence. Meanwhile, in order to achieve less interference between different sectors, it is considered different offset in downlink resource mapping of different sector.

- There are different Hopping offset sequences for different sectors. Each sector is corresponding to one Hopping offset sequence.

- Hopping offset sequence n is Hn= {h0, h1, h2 … hi}, value of hi indicates Hopping offset in sub-frame i×n, n≥0.

- For certain bandwidth and partition of DRB & LRB, order of value in Hopping offset sequence is corresponding to sub-frame/frame/super-frame number.

- Value and length of Hopping offset sequence can be changed for different bandwidth, either for partition of DRB & LRB and unit of hopping offset.

- Granularity unit of hopping offset can be based on tile/RB/cluster in sub-frame.

- Hopping offset sequences for different sectors are orthogonal each other.

E.g. for 5MHz bandwidth, as DRB:LRB=1:3, RB size is 18 subcarriers by 6 symbols, which means number of DRB is 6 in sub-frame, and number of LRB is 18. Each cluster has 24 tiles, whose 6 tiles are for 6 different DRBs respectively. Unit of hopping offset is RB. In this case, rang of Hopping offset value is from 0 to 3. In other words, length of Hopping offset sequence is 4.

For sector 1, it is assumed that Hopping offset sequence values are H1 = {0, 2, 1, 3}, whose values are corresponding to different sub-frame numbers.

For sub-frame number = 4n, n≥0, its corresponding Hopping offset value is 0.

For sub-frame number = 4n+1, n≥0, its corresponding Hopping offset value is 2.

In the case of sector 2 and 3, their Hopping offset sequence values can be generated like Hopping offset sequence values of sector 1, it is assumed that H2={1, 3, 2, 0} and H3={2, 0, 3, 1}. Offset between different sectors can be seen in Figure 4. Hopping offset sequences for different sectors are orthogonal each other.

Meanwhile, combination of Mapping offset sequences for mapping distributed resource can be changed in different sub-frame/frame/super-frame, which impacts mapping of distributed resource in each cluster of different sub-frame. For example as Figure 5 shown, combination of Mapping offset sequences in sub-frame 0 is C0= {M0, M1, M2, M3, M4, M5}, and combination of Mapping offset sequences in sub-frame 1 is changed, which is C1= {M6, M7, M8, M9, M10, M11}. Hopping Offset between different sub-frames in same sector is shown in Figure 5 and 6.

Figure 4: Offset between different sectors in same sub-frame

Figure 5: Hopping Offset between different sub-frames in same sector (Mapping of distributed resource is changed per frame)

Figure 6: Hopping Offset between different sub-frames in same sector (Mapping of distributed resource is changed per sub-frame)

## 3.3 Downlink FFR grouping

As Figure 7 shown, for FFR grouping, logical resource of sub-frame can be divided into multiple FFR groups. Each FFR group is composed by multiple logical RBs which are LRBs and /or DRBs. The granularity unit of FFR grouping is logical RB. Resource of FFR groups are orthogonal each other.

- Grouping: Logical resource are allocated to multiple FFR groups, each FFR group can have adjacent logical RBs or no-adjacent logical RBs.

- Permutation: Each FFR group is composed by multiple logical RBs that are DRBs and/or LRBs.

- Mapping: Logical DRBs and LRBs are renumbered and mapped into physical resource by resource mapping rule.

Figure 7: Example of FFR group mapping

# 4 Conclusions

In this paper some of the basic considerations of downlink distributed resource mapping and hopping for 802.16m have been discussed. We propose the following as design factors:

- Localized RB is formed from contiguous physical tiles.

- Distributed RB is formed from physical tiles that are spread over sub-frame.

- Locations of tiles for a DRB are corresponding to values of Mapping offset sequence which is for this DRB.

- Distributed resource mapping in a sub-frame is decided by combination of Mapping offset sequences for this sub-frame.

- Value range of Mapping offset sequence varies in terms of different bandwidth and partition of DRB&LRB.

- Combination of mapping offset sequence can be changed in different sub-frame/frame/super-frame, which impacts mapping resource of each DRB in different sub-frame.

- Hopping offset sequence value is hopping offset which is corresponding to different sub-frame/frame/super-frame for a certain bandwidth and partition of DRB & LRB.

- Length of Hopping offset sequence varies in terms of different hopping unit and bandwidth.

- Logical resource of sub-frame can be divided into multiple FFR groups. Each FFR group is composed by multiple RBs which are LRBs and /or DRBs.

Insert the following text into Chapter 11.5.2.3 in IEEE C802.16mDL_PHY-08/046:

------------------------------------------- Start of Proposed SDD Text --------------------------------------------------------

## 11.5.2.3.1 Downlink distributed resource mapping

Bandwidth of sub-frame is divided into several clusters which contains of distributed resource and localized resource. Further, within a cluster, bandwidth is divided into multiple frequency bands, each of which is called as sub-zone. When multiplexing of Localized resource and distributed resource, Localized resource is located in the LRB sub-zones; Distributed resource is located in the DRB sub-zones. Localized resource and Distributed resource occupy different frequency bands (sub-zones) within a sub-frame. Tiles of distributed RB are located in DRB sub-zones of different clusters in sub-frame. Localized RB is formed from contiguous physical tiles in LRB sub-zone.

In order to achieve better interference average, Mapping offset sequence is used for downlink distributed resource mapping, which can be generated by PN sequence. Distributed resource in each cluster is composed of multiple tiles which belong to different DRBs.

- The Mapping offset sequence values are corresponding to tile’s location of DRB in different DRB sub-zones of sub-frame.

- Mapping offset sequence Mn = {O0, O1, O2 … Oi}, value of Oi indicates tile’s location of corresponding DRB in DRB sub-zone i. Mapping offset sequence Mn indicates resource mapping of corresponding DRB in sub-frame.

- Range of value in Mapping offset sequence can vary in terms of different bandwidth and partition of DRB&LRB.

- Mapping offset sequences are orthogonal each other.

- Distributed resource mapping can be corresponding to different Mapping offset sequence in different sub-frame/frame/super-frame.

- Distributed resource mapping is decided by combination of Mapping offset sequences for each sub-frame/frame/super-frame.

Different combination of Mapping offset sequences is used for distributed resource mapping in different sub-frame/frame/super-frame.

- Combination of Mapping offset sequences is Cn = {MDRB0, MDRB1 … MDRBi}, MDRBi is Mapping offset sequence for logical DRBi. Cn indicates distributed resource mapping for each sub-frame/frame/super-frame.

- Combinations of Mapping offset sequences for different sub-frame/frame/super-frame in the same sector are orthogonal each other.

- Combinations of Mapping offset sequences for different sectors in the same sub-frame/frame/super-frame are orthogonal each other.

## 11.5.2.3.2 Downlink distributed resource hopping

Hopping offset sequence is used for distributed resource hopping in this contribution. Hopping offset sequence can be generated by PN sequence. Meanwhile, in order to achieve less interference between different sectors, it is considered different offset in downlink resource mapping of different sector.

- There are different Hopping offset sequences for different sectors. Each sector is corresponding to one Hopping offset sequence.

- Hopping offset sequence n is Hn= {h0, h1, h2 … hi}, value of hi indicates Hopping offset in sub-frame i×n, n≥0.

- For certain bandwidth and partition of DRB & LRB, order of value in Hopping offset sequence is corresponding to sub-frame/frame/super-frame number.

- Value and length of Hopping offset sequence can be changed for different bandwidth, either for partition of DRB & LRB and unit of hopping offset.

- Granularity unit of hopping offset can be based on tile/RB/cluster in sub-frame.

- Hopping offset sequences for different sectors are orthogonal each other.

------------------------------------------- End of Proposed SDD Text --------------------------------------------------------

# Reference

[1] “Proposed SDD text for Downlink Physical Structure”, C80216mDL_PHY-08_021.

[2] “16m Downlink Channelization”, C80216m-08_217r1.