| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Misaligned Super-frame Transmissions between 802.16m Base Stations |  |
| Date Submitted | 2008-05-15 |  |
| Source(s) | Richard Li, Chun-Yen Wang, Chie Ming Chou, ITRI Wern-Ho Sheen, NCTU/ITRI Youn-Tai Lee, III Sungho Moon, Jin Sam Kwak LG Electronics Shailender Timiri Sassan Ahmadi Intel Corporation | Voice: +886-3-591-7596 E-mail: richard929@itri.org.tw lyt@nmi.iii.org.tw {msungho, samji}@lge.com shailender.b.timiri@intel.com sassan.ahmadi@intel.com |
| Re: | IEEE 802.16m-08/016r1 - Call for Contributions on Project 802.16m System Description Document (SDD) Topic: The entire content of IEEE 802.16m-08/003r1 |  |
| Abstract | This contribution proposes to have interleaved super-frame transmissions between neighbor base stations to facilitate more efficient access and/or measurements. |  |
| Purpose | For discussion and approval by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Misaligned Super-frame Transmissions between 802.16m Base Stations

Richard Li, Chun-Yen Wang, Chie-Ming Chou, Wern-Ho Sheen

ITRI, NCTU

Youn-Tai Lee

III

Sungho Moon, Jin Sam Kwak

LG Electronics

Shailender Timiri, Sassan Ahmadi

Intel Corporation

- Introduction

In current draft SDD document, the concept of super-frame has been adopted as part of 802.16m frame structur [1]. Fig. 1 shows the frame structure in [1] for reading convenience. A super-frame consists of four 5ms radio frames and as a result the length of super-frame is 20ms. In the front of each super-frame, a Super-Frame Header is present and will carry system information, control information, synchronization signals, etc. These contents are dedicated for mobile stations to enter system, measure or synchronize with base stations.

Fig. 1. 802.16m frame structure – super-frame, frame, sub-frame, and symbol.

The presence of Super-Frame Header is one time per super-frame. That means the waiting time for these information/signals in Super-Frame Header varies between 0 and 20ms. For some operations, 20ms waiting time may result in too long latency. For example, when a MS intends to perform handover, channel measurement, downlink synchronization, and/or obtaining system control information with or from more than one neighbor cells are necessary. The required information and signals for these operations are put in Super-Frame Header and they come at a rate of 20ms.

In conventional frame structure, all base stations’ super-frames are supposed to be all aligned. This will result in long latency due to performing above-mentioned operations with more than one neighbor cells since the MS needs to wait 20ms for Super-Frame Header per neighbor cell. This contribution proposes an interleaved super-frame transmission method to resolve or mitigate this long latency issue for 802.16m system.

- Misaligned super-frame transmissions

The proposed method is to have misaligned super-frame transmission between neighbor BSs. Fig. 2 illustrates the proposed misaligned super-frame transmission between neighbor BSs. Neighbor 4 BSs form up a set of misaligned super-frame transmissions. As shown in the figure, BS #1, #2, #3, #4 transmit 20ms super-frames periodically and the super-frame timing between four BSs are offset one frame in order from #1 to #4. The set of misaligned super-frame transmission is extended as the system cell planning as shown in Fig. 3 to make sure neighbor cells don’t have aligned super-frames throughout the whole system.

Fig. 2. Proposed misaligned super-frame transmission between neighbor BSs

Fig. 3. Cell planning for misaligned super-frame transmissions.

When a MS camps on one BS and need to perform channel measurement, downlink synchronization, and/or obtaining system control information with or from neighbor cells, the MS will not need to perform these operations with a latency of 20ms per neighbor BS. The MS can quickly finish related operations with neighbor cells with a latency of 5ms per neighbor BS. Consequently, MSs in the system will be able to proceed related procedures with 5ms latency, rather than 20ms, while inducing 20ms super-frame concept into 802.16m frame structure.

- Applied to system information and/or synchronization signal

In current understanding of contents in Super-Frame Header, system information and synchronization signal are two main parts. The concept of misaligned super-frame could be applied to both or either one of system information and synchronization signals.

Fig. 4 shows an application of misaligned super-frame transmission to “system information”. MSs in the system will be able to access system information of neighbor cells with a latency of 5ms while applying 20ms super-frame.

Fig. 4. Misaligned super-frame structure for system information.

Fig. 5 shows an application of misaligned super-frame transmission to “synchronization signal”. In this case, one synchronization signal is transmitted per super-frame in order to save more resources for data. The most challenge task for using only one synchronization signal in a 20ms super-frame is the delay to detect synchronization signal while performing neighbor cell measurement/synchronization for handover. It will need 20ms latency for measurement/synchronization with each neighbor BS. With using the proposed misaligned super-frame transmission, MSs in the system will be able to perform measure and synchronize with neighbor cells with a latency of 5ms while applying 20ms super-frame. Consequently, the proposed misaligned method could enable “transmitting only one synchronization signal within 20ms super-frame” without increment of neighbor cell measurement/synchronization latency, as compared to “transmitting one synchronization signal per 5ms frame”.

Fig. 5. Misaligned super-frame structure for synchronization signal.

Fig. 6 illustrates combination of Fig. 4 and 5. This could be the proposal for 802.16m misaligned super-frame transmission while only one synchronization signal is transmitted per super-frame.

Fig. 6. Misaligned super-frame structure for synchronization signal and system information – in case of one synchronization signal per super-frame.

Fig. 7 illustrates the proposal for 802.16m misaligned super-frame transmission while one synchronization signal is transmitted per two frames.

Fig.7. Misaligned super-frame structure for synchronization signal and system information – in case of one synchronization signal per two frames.

Fig. 8 illustrates the proposal for 802.16m misaligned super-frame transmission while one synchronization signal is transmitted per frame.

Fig.8. Misaligned super-frame structure for synchronization signal and system information – in case of one synchronization signal per frame.

- Conclusion

This contribution proposes to use a misaligned super-frame transmission scheme to avoid long latency of neighbor cell measurement, synchronization, etc. for handover. The proposed scheme could be applied to super-frame transmission no matter one synchronization signal transmitted per frame or per super-frame. Several possible application cases have been discussed in this contribution and the proposed text for the 802.16m SDD is in the end of the document.

Reference

[1] IEEE 802.16m-08_003r1, “The Draft IEEE 802.16m System Description Document”

Proposed Text

The proposed texts are presented for IEEE 802.16_08/003r1.

The first part of text is used to induce misaligned super-frame concept into SDD document. The second part of text is used to further explain of misaligned super-frame transmission in 802.16m in case of one synchronization signal per super-frame or per frame.

-------- Start of the text proposal ---------------------------------------------------------------------------------------

[Insert the following text into a suitable place of Section 11.4 of IEEE 802.16_08/003r1.]

11.4.x Staggered super-frame transmissions

The start of super-frames between neighbor 802.16m cells may be staggered by one-frame increments. Fig. XXX illustrates the concept of staggered super-frames in a TDD system, where staggering of super-frames is applied to a cluster of 4 neighboring BSs, BS1-BS4. In the figure, BS #1, #2, #3, and #4 transmit 20ms super-frames periodically and the super-frame timing between four BSs are offset one frame in order from #1 to #4.

The cluster size for staggered super-frames is a network-configurable parameter and can range from 1 to 4. This allows for the option of a deployment with non-staggered super-frames, such as might be the case for MBSFN transmissions or other scenarios, depending on the requirements.

The concept of staggered super-frame transmissions across neighboring cells is equally applicable to FDD systems.

Fig. XXX. Staggered super-frame transmissions between neighboring BSs.

-------- End of proposed text ---------------------------------------------------------------------------------------------