| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Zero Forcing MU-MIMO with limited signaling overhead for 16m system |  |
| Date Submitted | 2008-05-5 |  |
| Source(s) | Yuantao Zhang , Hua Zhou, Jie Zhang, Jinyin Xue, Jun Tian Fujitsu R&D Center Ltd. Kevin Power, Sunil Vadgama Fujitsu Europe Labs Keiichi Nakatsugawa Fujitsu Laboratories LTD | {zyuantao; zhouhua, jie.zhang; xuejy; tianjun}@cn.fujitsu.com Kevin.Power@uk.fujitsu.com Sunil.Vadgama@uk.fujitsu.com nakatsugawa@jp.fujitsu.com |
| Re: | IEEE 802.16m-08/0016r1 Call for Contributions on Project 802.16m SDD Topic: DL MIMO |  |
| Abstract | Zero-Forcing MU-MIMO with limited signaling overhead for 16m system. |  |
| Purpose | Proposal for consideration in the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Zero-Forcing MU-MIMO with limited signaling overhead for 16m system

FUJITSU

# Introduction

It is required in 16m SRD [1] that MU-MIMO scheme should be supported in the system, where channel knowledge of the different users is exploited to schedule multiple users on the same time and frequency resource. Multiplexed users on the same resource unit can be separated at the UE or at the Node B. The former method is denoted as Unitary pre-coding, where the codebook comprises of several matrixes and the vectors in these matrixes are orthogonal with each other. The latter method is called Zero-Forcing (ZF) pre-coding, where the matrix in the pre-coding matrix cannot be orthogonal to each other, then BS can select arbitrary number of users to be scheduled according to the scheduling scheme. It is beneficial for ZF MU-MIMO schemes if the receiver is aware of the multiuser interference, as will be discussed in the next section. Therefore the transmission of the pre-coding matrix information in BS side should be well designed. Here a scheme is proposed for ZF MU-MIMO scheme to dynamically select the number of users being scheduled using limited downlink control signaling overhead.

# DL signaling issues

Figure.1 shows a general multi-user MIMO structure, where each UE calculates SINR corresponding to each vector in the codebook using the estimated channel and feedback the largest SINR as CQI and the corresponding pre-coding vector index (PVI) to the BS side. Using these feedback values, BS can select appropriate UEs according to a certain scheduling rule and transmit data streams to these UEs simultaneously.

Due to some impairment such as the quantization of the effective channel and the feedback delay, the ZF pre-coding can’t achieve perfect user data stream separation. Thus it would be beneficial to be able to suppress interference caused by the other scheduled users in the UE side using MMSE algorithm. This needs the pre-coding matrix to be known in advance in the UE side. Then the received signal can be utilized to estimate the covariance of the received signal for the MMSE receiver.

Figure 1--- Multi-user MIMO structure

There are at least two options to transmit the pre-coding information. The first is explicitly signaling the index of the allocated pre-coding vectors. The second is to use the pre-coded dedicated pilots. Taking the first scheme into consideration, using a 4 bits (16 vectors) codebook as an example, the possible number of simultaneously scheduled UEs will be two, three or four. Then the information to be transmitted in a straightforward way include the number of users been scheduled (2 bits) and the individual pre-coding vector index been employed (totally 16 bits for 4 users). The pre-coding information can also be transmitted using a brute-force way, where UEs can be allocated into user group with the number of users in each group be 2, 3 or 4. The total number of user groups can be denoted as N=i=24K, where K denotes the number of vectors in the codebook. Therefore the number of bit for 4 bits codebook to signal the pre-coding matrix is log2N=log22500=12bits. These two signaling schemes are either too complex or need too many resources.

# Limited signaling scheme

Taking 4 bits Grassmannian codebook as an example, the steps of the designed limited signaling overhead scheme are listed as follows.

Firstly, due to the fact that the inter-stream interference will be decreased if the selected vectors to form the pre-coding matrix have low correlation values for any pair of them, each vector sn(1≤n≤16)in the codebook is designed to maintain a set denoted as Sn=sn,k1,sn,k2,⋯,sn,kN. The correlation values between each vector in the set and vector snis below a pre-defined threshold T. i.e.

snHsn,kl≤T, k1klkN

Then a trellis based structure is proposed, as shown in Figure.2, where each vector in the codebook stands for a state. The number of the columns in the trellis is equal to the maximum number of users that can be scheduled. Figure.2 is only for illustration and describes several state transitions for simplicity. An example is used here to illustrate the state transitions. For state 1 in the first column, its next states are the vectors in its maintained vector set, which can be denoted asS1=s1,k1,s1,k2,⋯,s1,kN. It should be noted that each vector in S1 also has its own maintained set as in step 1. Supposing the maintained vector set for s1,k1is denoted as S1,k1, then in the second transition, the next states of s1,k1can be denoted as

S1,2=S1∩S1,k1

Further, to reduce the redundant state transitions, it is designed that the index of the vectors transmitted from current vector is always larger than the current vector index, i.e., the index of the vectors in the set S1,2is always larger than the index ofs1,k1. This process is the same for the subsequent transitions. As a result, P2, P3, P4are assumed to be the set consisting of matrices corresponding to the whole 1 states transition, 2 states transition, 3 states transition, respectively.

After the trellis is constructed, all the state transitions in the trellis are uniformly numbered and allocated with a unique index. With 4 bits Grassmannian codebook and T=0.4, it can be calculated that 126 possible user groups can be obtained and log2126=7bits are needed to convey the user group index through control channel.

The total number of the user groups is directly related to the pre-defined threshold and the maximum number of the scheduled users. The threshold can be designed with different values for different number of scheduled users in order to balance the total number of state transitions.

Figure 2--- trellis structure

The resultant user groups and corresponding useful information deducted from the trellis can be stored in each BS and MS for user group numbering and scheduling. An example stored table is proposed as shown in Table.1.

Table 1 Information stored in BS and MS

| Number of vectors | Contained vectors | Upper vector combination index | Vector combination index |
| --- | --- | --- | --- |
| 2 | s1 , s2 | NA | 1 |
| 2 | s1 , s4 | NA | 2 |
| … | … | … | … |
| 3 | s1 , s2, s9 | 1 | k1 |
| 3 | s1 , s4, s9 | 2 | k2 |
| … | … | … | … |
| 4 | s1 , s2, s9, s14 | k1, 1 | kn |
| 4 | s1 , s4, s9, s14 | k2, 2 | kn+1 |
| … | … | … | … |

The first column of Table 1 denotes the number of users in each user groups. The corresponding vectors contained in each user group in denoted in column 2. Column 3 denotes the upper (father) vector combination index for each user group, which is useful for BS scheduling in section 4. The index of each user group in illustrated in last column.

# BS scheduling

Max-sum-rate scheduling method is employed as an example. With the proposed trellis structure and user group select scheme, the scheduling method can be presented as follows.

  - Find if there are 4 vectors that can form a matrix contained in P4. Supposing k1 matrices can be formed and denoted asW4=W4,1,W4,2,⋯,W4,k1. The index of W4,ncorresponding to the highest SINR can be denoted as:

n=argmax1≤n1≤k1SINRn1

where SINRn1denotes the summate SINRs feedback with the vectors in W4,n.

  - Find if there are 3 vectors that can form a matrix contained in P3, meanwhile the matrix is not a sub-matrix ofW4. Supposing k2 matrices can be formed and denoted asW3=W3,1,W3,2,⋯,W3,k2. The highest SINR and the corresponding matrix can be found in a similar way as described in step a.

  - Find if there are 2 vectors that can form a matrix contained in P2, meanwhile the matrix is not a sub-matrix of W4andW3. Supposing k3matrices can be formed and denoted as W2=W2,1,W2,2,⋯,W2,k3.

  - Find the highest SINR of these three steps and the corresponding matrix. This matrix is used as pre-coding matrix after zero-forcing operation. Then BS conveys the matrix index to the scheduled terminals.

# Text proposal

Insert the following text into SDD Sections indicated below

---------------------------text start-----------------------------

11. x Advanced multiple antenna technique

11.x.1 SU-MIMO technique

11.x.1.1 Open loop

11.x.1.1 Closed loop

11.x.2 MU-MIMO technique

11.x.2.1 Non-codebook based pre-coding

11.x.2.2 codebook based pre-coding

11.x.2.2.1 UL feedback design

11.x.2.2.2 DL pre-coding schemes

11.x.2.2.3 DL signaling scheme

11.x.3 Cooperative MU-MIMO technique

11.x.4 Unified SU-MIMO and MU-MIMO technique

11.x.4.1 Unified codebook for SU-MIMO and MU-MIMO.

11.x.4.2 Feedback design

### 11.x.2.3 DL signaling scheme

Due to some impairment such as the quantization of the effective channel and the feedback delay, the ZF pre-coding can’t achieve perfect user data stream separation. Thus it would be beneficial to be able to suppress interference caused by the other scheduled users in the UE side. This needs the pre-coding information to be known in advance in the UE side. Therefore DL signaling carrying the pre-coding matrix information should be well designed with limited signaling overhead. To do that, each vector in the codebook maintains a set and the correlation between this vector and each vector in the set is below a pre-defined threshold. A trellis can be constructed using the maintained sets and the state transitions in the trellis can be obtained and uniformly numbered. When the pre-coding matrix is selected, its corresponding state transition index in the trellis is signalled in DL.

---------------------------------text end---------------------------

# References

[1] IEEE 802.16m-07/002r4, “802.16m System Requirements”