| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for user grouping scheme for MU-MIMO |  |
| Date Submitted | 2008-05-5 |  |
| Source(s) | Hua Zhou, Jie Zhang, Jinyin Xue, Yuantao Zhang, Jun Tian Fujitsu R&D Center Ltd. Kevin Power, Sunil Vadgama Fujitsu Europe Labs Keiichi Nakatsugawa Fujitsu Laboratories LTD | {zhouhua; jie.zhang; xuejy; zyuantao; tianjun}@cn.fujitsu.com Kevin.Power@uk.fujitsu.com Sunil.Vadgama@uk.fujitsu.com nakatsugawa@jp.fujitsu.com |
| Re: | IEEE 802.16m-08/0016r1 Call for Contributions on Project 802.16m SDD Topic: DL MIMO |  |
| Abstract | Proposal for designing MU-MIMO application in IEEE 802.16m systems. |  |
| Purpose | Proposal for consideration in the 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Pairing scheme for MU-MIMO for 16m system

FUJITSU

# Introduction

16m SRD [1] required that MU-MIMO should be supported in the system. The key point for codebook based MU-MIMO is to compute the best CQI and corresponding precoding vector index. Since each terminal can’t obtain the channel matrix for other terminals when deciding the best codewords and CQI for itself, the inter-user interference can’t be controlled efficiently. In this proposal, new feedback and precoding scheme is designed to improve the inter-user interference effect in MU-MIMO mode.

# Issue in precoding design for MU-MIMO

When we consider codebook-based MU-MIMO technique, the key point is to calculate the CQI and the best codeword for each user. The selected codeword and corresponding CQI can be calculated as in the case of the unitary matrix-based codebook:

Fk=argmaxi,j∈1,...PHkFi2σ2+HkFj2CQIk=HkFk2σ2+HkFj2

Note that the CQI computation includes all interference from other precoding vector except of its own. In this case, the CQI is heavily underestimated, so that the whole multiple user throughputs can not be utilized sufficiently.

On the other hand, for non-unitary precoding, the CQI is calculated as:

Fk=argmaxi,j∈1,...P,FiFj2<ρthrdHkFi2σ2+HkFj2CQIk=HkFk2σ2+HkFk2

Although, CQI computation has already considered the interference from other user in advance, the base station may use different codeword, other than the one in CQI calculation. Therefore, the CQI computation may possibly mismatch with the realistic capacity.

From the above description on existent MU-MIMO precoding schemes, the common feature is to compute the best CQI and feedback it and corresponding precoding vector index. All these schemes either overestimate the interference for unitary precoding, or cause possible mismatch when blindly selecting users for non-unitary precoding. In this case, new precoding scheme need to be designed for accounting for these issues.

# Limited feedback transmission

Suppose the number of TX antennas at the BS is N, each user has single Rx antenna (which can be generalized to multiple receiver antenna), and the codebook consists of L unitary or non-unitary codewords,

W=w1,...,wL

To begin with, we suppose that the number of stream for each user is 1, which also can be generalized to more than rank 1.  Here, we propose that each user feedback two groups of parameters, the maximum SNR and corresponding codeword index, and the minimal SNR and corresponding codeword index, which can be generated according to the following:

w1k=argmaxi∈1,...,LHkwi2σ2,CQI1k=Hkw1k2σ2w2k=argmini∈1,...,LHkwi2σ2,CQI2k=Hkw2k2σ2

In other words, w1kis the codeword user k most expects, w2kis the codeword user k hopes the other user to use. The two kinds of CQI reflect the real channel gain and real interference power.

The feedback can be controlled by two methods, one of which is to simply feedback two code indices in the same codebook as in the conventional scheme, in order to get better performance; the other method is to select these two codes from small-size codebook, for example, if conventional scheme uses 6 bit codebook as in [3], we can use 3 bit codebook, in this case, we can keep the total feedback the same as in SU-MIMO and other schemes.

# BS scheduling

At the BS, there exist three possible cases in terms of codewords pairing:

- Perfect matching

In this case, there exist two users whose codewords are matching to each other:

w1k=w2j And w2k=w1j

The real SNR for each user can be easily shown as:

SINRk=0.5∗CQI1k1+0.5∗CQI2k,SINRj=0.5∗CQI1j1+0.5∗CQI2j

Here, we suppose that each user can get half of the total transmission power. Then we can decide the appropriate MCS level for each user based on SNR value according to some mapping methods, such as EESM, MMIB or RBIR.

- Half matching

w1k=w2j or w2k=w1j

Suppose that we first consider w1k=w2j which can be generalized tow2k=w1j. Butw2k, the codeword user k expects user j to use, is not equal tow1j, the codeword that user j expect, i.e,

w1k=w2j And w2k≠w1j

BS can decide user k use codewordw1k, but the codeword for user j is selected upon the following rules:

  - Rule 1: Select user j when codeword correlation is greater than some predefined threshold ρth, i.e.,w2k∗w1jT>=ρth

  - Rule 2: if Rule 1 is satisfied, BS simply decides the codeword for user j as wj=w2k

In this case, the effective SINR for user k can be precisely estimated by:

SINRk=0.5∗CQI1k1+0.5∗CQI2k

On the other hand, the SINR for user j can be described as:

SINRj=0.5∗Hjw2k21+0.5∗CQI2j

Here, the estimation of Hjw2k2 depends on the estimatedHj, suppose we can approximate Hjby linear combination of w1j andw2j, i.e.

Hj≈αj⋅w1jT+βj⋅w2jTs.t. Hjw1j2=CQI1j;Hjw2j2=CQI2j

The scalar αj and βjcan be shown as:

αj=CQI1j−ρj⋅CQI2j1−ρj2, βj=CQI2j−ρj⋅CQI1j1−ρj2

Where,

ρj=w1jTw2j

In the same way, the appropriate MCS level can be selected based on the calculated SNR value.

- non-matching

BS simply select one user with highest CQI to transmit, in which case, SU-MIMO is used.

UserIndex = k, if CQI1k>=CQI1jj, if CQI1k<CQI1j

With these cases, the BS can easily decide to use SU-MIMO or MU-MIMO, and the corresponding data rate or MCS level for the user in SU-MIMO, or users in MU-MIMO.

# Text proposal

Insert the following text into SDD Sections indicated below

---------------------------text start-----------------------------

11. x Advanced multiple antenna technique

11.x.1 SU-MIMO technique

11.x.1.1 Open loop

11.x.1.1 Closed loop

11.x.2 MU-MIMO technique

11.x.2.1 Non-codebook based Precoding

11.x.2.2 Codebook based precoding

11.x.2.2.1 UL feedback design

11.x.2.2.2 DL precoding schemes

11.x.2.2.3 DL signaling design

11.x.3 Cooperative MU-MIMO technique

11.x.4 Unified SU/MU-MIMO technique

11.x.4.1 Unified codebook for SU-MIMO and MU-MIMO

11.x.4.2 Feedback design

11.x.2.2.1 UL feedback design

Each terminal should feedback the codewords indices and the corresponding CQI, where, the codeword indices are selected, including the one leading to maximizing SNR, and the other(s) leading to minimizing SNR, for the codebook.

### 11.x.2.2.2 DL precoding schemes

BS should select the users based on the codewords, i.e., grouping users with codewords matching, in MU-MIMO case, or select one user if there is no matching between codewords.

# References

[1] IEEE 802.16m-07/002r4, “802.16m System Requirements”

[2] IEEE 802.16m-008/004, “802.16m Evaluation Methodology”

[3] P802.16Rev2/D4, April 2008.