| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |
| --- | --- |
| Title | Coexistence Support in IEEE 802.16m Frame Structure |
| Date Submitted | May 5, 2008 |
| Source(s) | Sassan Ahmadi (sassan.ahmadi@intel.com) Intel Corporation |
| Re: | Call for Contributions on Project 802.16m System Description Document (SDD) Frame structure |
| Abstract | Coexistence Support in IEEE 802.16m Frame Structure |
| Purpose | Discussion and Approval |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |

# Coexistence Support in IEEE 802.16m Frame Structure

Sassan Ahmadi

Intel Corporation

Proposed Text for SDD

Insert the following text into Section 11.4.6 in [1]:

-------------------------------  Text Start  ---------------------------------------------------

11.4.6 Coexistence Support in Frame Structure

11.4.6.1 Coexistence with E-UTRA (LTE)

[Coexistence between IEEE 802.16m and E-UTRA in TDD duplex scheme may be facilitated by inserting either idle symbols within the IEEE 802.16m frame or idle sub-frames. Applying a delay or offset between the beginnings of the IEEE 802.16m frame and the E-UTRA TDD frame may allow the time allocated to idle symbols or idle sub-frames to be minimized. Figures 11.4-x and 11.4-y demonstrate how coexistence between IEEE 802.16m and E-UTRA can be achieved in both FDD and TDD modes, respectively to minimize the inter-system interference.

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

DL/UL

10 ms LTE Radio Frame

0.617 ms 802.16m Sub-Frame

0.5 ms LTE Slot

5 ms IEEE 802.16e (IEEE 802.16m) Radio Frame

Notes: Each 10 ms LTE radio frame is equivalent to  two 5 ms 802.16m radio frames.

5 LTE Sub-Frames are equivalent to 8 802.16m Sub-Frames (5:8 Sub-Frame Ratio).

Idle Time=64.64 us

Figure 11.4-x: Alignment of IEEE 802.16m frame with E-UTRA frame in FDD mode

DL

DL

UL

UL

UL

DL

Offset

Figure 11.4-y: Alignment of IEEE 802.16m frame with E-UTRA frame in TDD mode

11.4.5.2 Coexistence with UTRA LCR-TDD (TD-SCDMA)

Coexistence between IEEE 802.16m TDD and UTRA LCR-TDD (TD-SCDMA) may be facilitated by inserting either idle symbols within the IEEE 802.16m frame or idle sub-frames. Figure 11.4-z shows how coexistence between IEEE 802.16m with a 4:4 DL:UL ratio and TD-SCDMA can be facilitated.

0

675

950

1625

2300

2975

3650

4325

5000

750

825

DwPTS=75

UpPTS=125

5ms

UL

UL

UL

DL

DL

DL

DL

UTRA LCR-TDD

DL

DL

DL

UL

UL

UL

UL

DL

IEEE 802.16m

Idle Time: 333 us

DUSP/UDSP

Idle Time: 2 OFDM Symbols= 205.64 us

275 us

TD-SCDMA Slot=675 us

Figure 11.4-z: Alignment of IEEE 802.16m frame with UTRA LCR-TDD frame in TDD mode

References

[1] IEEE C802.16m-08/003r1, “The Draft IEEE 802.16m System Description Document.”